IVI-4.10: IviRFSigGen Class Specification October 14, 2016 Edition Revision 2.0

Important Information

The IviRFSigGen Class Specification (IVI-4.10) is authored by the IVI Foundation member companies. For a vendor membership roster list, please visit the IVI Foundation web site at www.ivifoundation.org, or contact the IVI Foundation.

The IVI Foundation wants to receive your comments on this specification.  See the IVI Foundation web site for contact information.

- Warranty

The IVI Foundation and its member companies make no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. The IVI Foundation and its member companies shall not be liable for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material.

- Trademarks

Product and company names listed are trademarks or trade names of their respective companies.

No investigation has been made of common-law trademark rights in any work.

1.	Overview of the IviRFSigGen Specification	13

1.1	Introduction	13

1.2	IviRFSigGen Class Overview	13

1.3	References	13

1.4	Definitions of Terms and Acronyms	14

2.	IviRFSigGen Class Capabilities	15

2.1	Introduction	15

2.2	IviRFSigGen Group Names	15

2.3	Repeated Capability Names	16

2.3.1	LFGenerator	16

2.3.2	Analog Modulation Source	17

2.4	Boolean Attribute and Parameter Values	17

2.5	.NET Namespace	17

2.6	.NET IviRFSigGen Session Factory	17

3.	General Requirements	19

3.1	Minimum Class Compliance	19

3.1.1	Disable	19

3.2	Capability Group Compliance	19

4.	IviRFSigGenBase Capabilities Group	20

4.1	Overview	20

4.2	IviRFSigGenBase Attributes	20

4.2.1	Frequency	21

4.2.2	Power Level	22

4.2.3	ALC Enabled	23

4.2.4	Output Enabled	24

4.3	IviRFSigGenBase Functions	25

4.3.1	Configure RF	26

4.3.2	Configure ALC Enabled (IVI-C only)	27

4.3.3	Configure Output Enabled (IVI-C only)	28

4.3.4	Disable All Modulation	29

4.3.5	Is Settled	30

4.3.6	Wait Until Settled	31

4.4	IviRFSigGenBase Behavior Model	33

5.	IviRFSigGenModulateAM Extension Group	34

5.1	IviRFSigGenModulateAM Extension Group Overview	34

5.2	IviRFSigGenModulateAM Attributes	34

5.2.1	AM Enabled	35

5.2.2	AM Source	36

5.2.3	AM Scaling	37

5.2.4	AM External Coupling	39

5.2.5	AM Nominal Voltage	41

5.2.6	AM Depth	42

5.3	IviRFSigGenModulateAM Functions	43

5.3.1	Configure AM Enabled (IVI-C only)	44

5.3.2	Configure AM External Coupling (IVI-C only)	45

5.3.3	Configure AM	46

5.4	IviRFSigGenModulateAM Behavior Model	47

5.5	IviRFSigGenModulateAM Compliance Notes	47

6.	IviRFSigGenModulateFM Extension Group	48

6.1	IviRFSigGenModulateFM Extension Group Overview	48

6.2	IviRFSigGenModulateFM Attributes	48

6.2.1	FM Enabled	49

6.2.2	FM Source	50

6.2.3	FM External Coupling	51

6.2.4	FM Nominal Voltage	53

6.2.5	FM Deviation	54

6.3	IviRFSigGenModulateFM Functions	55

6.3.1	Configure FM Enabled (IVI-C only)	56

6.3.2	Configure FM External Coupling (IVI-C only)	57

6.3.3	Configure FM	58

6.4	IviRFSigGenModulateFM Behavior Model	59

6.5	IviRFSigGenModulateFM Compliance Notes	59

7.	IviRFSigGenModulatePM Extension Group	60

7.1	IviRFSigGenModulatePM Extension Group Overview	60

7.2	IviRFSigGenModulatePM Attributes	60

7.2.1	PM Enabled	61

7.2.2	PM Source	62

7.2.3	PM External Coupling	63

7.2.4	PM Nominal Voltage	65

7.2.5	PM Deviation	66

7.3	IviRFSigGenModulatePM Functions	67

7.3.1	Configure PM Enabled (IVI-C only)	68

7.3.2	Configure PM External Coupling (IVI-C only)	69

7.3.3	Configure PM	70

7.4	IviRFSigGenModulatePM Behavior Model	71

7.5	IviRFSigGenModulatePM Compliance Notes	71

8.	IviRFSigGenAnalogModulationSource Extension Group	72

8.1	IviRFSigGenAnalogModulationSource Extension Group Overview	72

8.2	IviRFSigGenAnalogModulationSource Attributes	72

8.2.1	Analog Modulation Source Count	73

8.2.2	Analog Modulation Source Name (IVI-COM Only)	74

8.3	IviRFSigGenAnalogModulationSource Functions	75

8.3.1	Get Analog Modulation Source Name (IVI-C & IVI.NET only)	76

8.4	IviRFSigGenAnalogModulationSource Behavior Model	78

8.5	IviRFSigGenAnalogModulationSource Compliance Notes	78

9.	IviRFSigGenModulatePulse Extension Group	79

9.1	IviRFSigGenModulatePulse Extension Group Overview	79

9.2	IviRFSigGenModulatePulse Attributes	79

9.2.1	Pulse Modulation Enabled	80

9.2.2	Pulse Modulation Source	81

9.2.3	Pulse Modulation External Polarity	83

9.3	IviRFSigGenModulatePulse Functions	85

9.3.1	Configure Pulse Modulation Enabled (IVI-C only)	86

9.3.2	Configure Pulse Modulation Source (IVI-C only)	87

9.3.3	Configure Pulse Modulation External Polarity (IVI-C only)	88

9.4	IviRFSigGenModulatePulse Behavior Model	89

9.5	IviRFSigGenModulatePulse Compliance Notes	89

10.	IviRFSigGenLFGenerator Extension Group	90

10.1	IviRFSigGenLFGenerator Extension Group Overview	90

10.2	IviRFSigGenLFGenerator Attributes	90

10.2.1	Active LFGenerator	91

10.2.2	LFGenerator Count	92

10.2.3	LFGenerator Name (IVI-COM Only)	93

10.2.4	LFGenerator Frequency	94

10.2.5	LFGenerator Waveform	95

10.3	IviRFSigGenLFGenerator Functions	97

10.3.1	Get LFGenerator Name (IVI-C & IVI.NET only)	98

10.3.2	Set Active LFGenerator (IVI-C Only)	99

10.3.3	Configure LFGenerator	100

10.4	IviRFSigGenLFGenerator Behavior Model	101

10.5	IviRFSigGenLFGenerator Compliance Notes	101

11.	IviRFSigGen LFGeneratorOutput Extension Group	102

11.1	IviRFSigGenLFGeneratorOutput Extension Group Overview	102

11.2	IviRFSigGenLFGeneratorOutput Attributes	102

11.2.1	LFGenerator Output Amplitude	103

11.2.2	LFGenerator Output Enabled	104

11.3	IviRFSigGenLFGeneratorOutput Functions	105

11.3.1	Configure LFGenerator Output	106

11.4	IviRFSigGenLFGeneratorOutput Behavior Model	107

11.5	IviRFSigGenLFGeneratorOutput Compliance Notes	107

12.	IviRFSigGenPulseGenerator Extension Group	108

12.1	IviRFSigGenPulseGenerator Extension Group Overview	108

12.2	IviRFSigGenPulseGenerator Attributes	108

12.2.1	Pulse Internal Trigger Period	109

12.2.2	Pulse Width	110

12.2.3	Pulse Gating Enabled	111

12.2.4	Pulse Trigger Source	112

12.2.5	Pulse External Trigger Slope	114

12.2.6	Pulse External Trigger Delay	116

12.3	IviRFSigGenPulseGenerator Functions	117

12.3.1	Configure Pulse External Trigger	118

12.3.2	Configure Pulse Internal Trigger (IVI-C only)	119

12.3.3	Configure Pulse	120

12.4	IviRFSigGenPulseGenerator Behavior Model	121

12.5	IviRFSigGenPulseGenerator Compliance Notes	121

13.	IviRFSigGenPulseDoubleGenerator Extension Group	122

13.1	IviRFSigGenPulseDoubleGenerator Extension Group Overview	122

13.2	IviRFSigGenPulseDoubleGenerator Attributes	122

13.2.1	Pulse Double Enabled	123

13.2.2	Pulse Double Delay	124

13.3	IviRFSigGenPulseDoubleGenerator Functions	125

13.3.1	Configure Pulse Double	126

13.4	IviRFSigGenPulseDoubleGenerator Behavior Model	127

13.5	IviRFSigGenPulseDoubleGenerator Compliance Notes	127

14.	IviRFSigGenPulseGeneratorOutput Extension Group	128

14.1	IviRFSigGenPulseGeneratorOutput Extension Group Overview	128

14.2	IviRFSigGenPulseGeneratorOutput Attributes	128

14.2.1	Pulse Output Polarity	129

14.2.2	Pulse Output Enabled	131

14.3	IviRFSigGenPulseGeneratorOutput Functions	132

14.3.1	Configure Pulse Output	133

14.4	IviRFSigGenPulseGeneratorOutput Behavior Mode	134

14.5	IviRFSigGenPulseGeneratorOutput Compliance Notes	134

15.	IviRFSigGenSweep Extension Group	135

15.1	IviRFSigGenSweep Overview	135

15.2	IviRFSigGenSweep Attributes	135

15.2.1	Sweep Mode	136

15.2.2	Sweep Trigger Source	139

15.3	IviRFSigGenSweep Functions	141

15.3.1	Configure Sweep	142

15.4	IviRFSigGenSweep Behavior Model	143

15.4.1	Frequency Sweep / Power Sweep	143

15.4.2	Frequency Step / Power Step / List	144

16.	IviRFSigGenFrequencySweep Extension Group	146

16.1	IviRFSigGenFrequencySweep Extension Group Overview	146

16.2	IviRFSigGenFrequencySweep Attributes	146

16.2.1	Frequency Sweep Start	147

16.2.2	Frequency Sweep Stop	148

16.2.3	Frequency Sweep Time	149

16.3	IviRFSigGenFrequencySweep Functions	150

16.3.1	Configure Frequency Sweep Start Stop	151

16.3.2	Configure Frequency Sweep Center Span	152

16.3.3	Configure Frequency Sweep Time (IVI-C only)	153

16.4	IviRFSigGenFrequencySweep Behavior Model	154

16.5	IviRFSigGenFrequencySweep Compliance Notes	154

17.	IviRFSigGenPowerSweep Extension Group	155

17.1	IviRFSigGenPowerSweep Extension Group Overview	155

17.2	IviRFSigGenPowerSweep Attributes	155

17.2.1	Power Sweep Start	156

17.2.2	Power Sweep Stop	157

17.2.3	Power Sweep Time	158

17.3	IviRFSigGenPowerSweep Functions	159

17.3.1	Configure Power Sweep Start Stop	160

17.3.2	onfigure Power Sweep Time (IVI-C only)	161

17.4	IviRFSigGenPowerSweep Behavior Model	162

17.5	IviRFSigGenPowerSweep Compliance Notes	162

18.	IviRFSigGenFrequencyStep Extension Group	163

18.1	IviRFSigGenFrequencyStep Extension Group Overview	163

18.2	IviRFSigGenFrequencyStep Attributes	163

18.2.1	Frequency Step Start	164

18.2.2	Frequency Step Stop	165

18.2.3	Frequency Step Scaling	166

18.2.4	Frequency Step Size	168

18.2.5	Frequency Step Single Step Enabled	169

18.2.6	Frequency Step Dwell	170

18.3	IviRFSigGenFrequencyStep Functions	171

18.3.1	Configure Frequency Step Start Stop	172

18.3.2	Configure Frequency Step Dwell	173

18.3.3	Reset Frequency Step	174

18.4	IviRFSigGenFrequencyStep Behavior Model	175

18.5	IviRFSigGenFrequencyStep Compliance Notes	175

19.	IviRFSigGenPowerStep Extension Group	176

19.1	IviRFSigGenPowerStep Extension Group Overview	176

19.2	IviRFSigGenPowerStep Attributes	176

19.2.1	Power Step Start	177

19.2.2	Power Step Stop	178

19.2.3	Power Step Size	179

19.2.4	Power Step Single Step Enabled	180

19.2.5	Power Step Dwell	181

19.3	IviRFSigGenPowerStep Functions	182

19.3.1	Configure Power Step Start Stop	183

19.3.2	Configure Power Step Dwell	184

19.3.3	Reset Power Step	185

19.4	IviRFSigGenPowerStep Behavior Model	186

19.5	IviRFSigGenPowerStep Compliance Notes	186

20.	IviRFSigGenList Extension Group	187

20.1	IviRFSigGenList Extension Group Overview	187

20.2	IviRFSigGenList Attributes	187

20.2.1	List Selected Name	188

20.2.2	List Single Step Enabled	189

20.2.3	List Dwell	190

20.3	IviRFSigGenList Functions	191

20.3.1	Create Frequency List	192

20.3.2	Create Power List	193

20.3.3	Create Frequency Power List	194

20.3.4	Select List (IVI-C only)	195

20.3.5	Clear All Lists	196

20.3.6	Configure List Dwell	197

20.3.7	Reset List	198

20.4	IviRFSigGenList Behavior Model	199

20.5	IviRFSigGenList Compliance Notes	199

21.	IviRFSigGenALC Extension Group	200

21.1	IviRFSigGenALC Overview	200

21.2	IviRFSigGenALC Attributes	200

21.2.1	ALC Source	201

21.2.2	ALC Bandwidth	203

21.3	IviRFSigGenALC Functions	204

21.3.1	Configure ALC	205

21.4	IviRFSigGenALC Behavior Model	206

22.	IviRFSigGen ReferenceOscillator Extension Group	207

22.1	IviRFSigGenReferenceOscillator Overview	207

22.2	IviRFSigGenReferenceOscillator Attributes	207

22.2.1	Reference Oscillator Source	208

22.2.2	Reference Oscillator External Frequency	210

22.3	IviRFSigGenReferenceOscillator Functions	211

22.3.1	Configure Reference Oscillator	212

22.4	IviRFSigGenReferenceOscillator Behavior Model	213

23.	IviRFSigGenSoftwareTrigger Extension Group	214

23.1	IviRFSigGenSoftwareTrigger Overview	214

23.2	IviRFSigGenSoftwareTrigger Functions	214

23.2.1	Send Software Trigger	214

23.3	IviRFSigGenSoftwareTrigger Behavior Model	214

23.4	IviRFSigGenSoftwareTrigger Compliance Notes	214

24.	IviRFSigGenModulateIQ Extension Group	215

24.1	IviRFSigGenModulateIQ Extension Group Overview	215

24.2	IviRFSigGenModulateIQ Attributes	215

24.2.1	IQ Enabled	216

24.2.2	IQ Nominal Voltage	217

24.2.3	IQ Source	218

24.2.4	IQ Swap Enabled	220

24.3	IviRFSigGenModulateIQ Functions	221

24.3.1	Configure IQ Enabled (IVI-C only)	222

24.3.2	Configure IQ	223

24.3.3	Calibrate IQ	224

24.4	IviRFSigGenModulateIQ Behavior Model	225

25.	IviRFSigGenIQImpairment Extension Group	226

25.1	IviRFSigGenIQImpairment Extension Group Overview	226

25.2	IviRFSigGenIQImpairment Attributes	226

25.2.1	IQ Impairment Enabled	227

25.2.2	IQ I-Offset	228

25.2.3	IQ Q-Offset	229

25.2.4	IQ Ratio	230

25.2.5	IQ Skew	231

25.3	IviRFSigGenIQImpairment Functions	232

25.3.1	Configure IQ Impairment Enabled (IVI-C only)	233

25.3.2	Configure IQ Impairment	234

25.4	IviRFSigGenIQImpairment Behavior Model	235

25.5	IviRFSigGenIQImpairment Compliance Notes	235

26.	IviRFSigGenArbGenerator Extension Group	236

26.1	IviRFSigGenArbGenerator Extension Group Overview	236

26.2	IviRFSigGenArbGenerator Attributes	236

26.2.1	Arb Selected Waveform	237

26.2.2	Arb Clock Frequency	238

26.2.3	Arb Filter Frequency	239

26.2.4	Arb Max Number Waveforms	240

26.2.5	Arb Waveform Quantum	241

26.2.6	Arb Waveform Size Min	242

26.2.7	Arb Waveform Size Max	243

26.2.8	Arb Trigger Source	244

26.2.9	Arb External Trigger Slope	246

26.3	IviRFSigGenArbGenerator Functions	248

26.3.1	Configure Arb	249

26.3.2	Write Arb Waveform	250

26.3.3	Select Arb Waveform (IVI-C only)	251

26.3.4	Clear All Arb Waveforms	252

26.3.5	Query Arb Waveform Capabilities (IVI-C and IVI-COM Only)	253

26.3.6	Configure Arb Trigger Source (IVI-C only)	254

26.3.7	Configure Arb External Trigger Slope (IVI-C only)	255

26.4	IviRFSigGenArbGenerator Behavior Model	256

26.5	IviRFSigGenArbGenerator Compliance Notes	256

27.	IviRFSigGenDigitalModulationBase Extension Group	257

27.1	IviRFSigGenDigitalModulationBase Extension Group Overview	257

27.2	IviRFSigGenDigitalModulationBase Attributes	257

27.2.1	DigitalModulationBase Standard Names (.NET only)	258

27.2.2	DigitalModulationBase Standard Name (IVI-COM only)	259

27.2.3	DigitalModulationBase Standard Count (IVI-C and IVI-COM only)	260

27.2.4	DigitalModulationBase Selected Standard	261

27.2.5	DigitalModulationBase Data Source	262

27.2.6	DigitalModulationBase PRBS Type	264

27.2.7	DigitalModulationBase Selected Bit Sequence	266

27.2.8	DigitalModulationBase Clock Source	267

27.2.9	DigitalModulationBase External Clock Type	269

27.3	IviRFSigGenDigitalModulationBase Functions	271

27.3.1	Get DigitalModulationBase Standard Name (IVI-C only)	272

27.3.2	Select DigitalModulationBase Standard (IVI-C only)	273

27.3.3	Configure DigitalModulationBase Clock Source	274

27.3.4	Configure DigitalModulationBase Data Source (IVI-C only)	275

27.3.5	Configure DigitalModulationBase PRBS Type (IVI-C only)	276

27.3.6	Write DigitalModulationBase Bit Sequence	277

27.3.7	Select DigitalModulationBase Bit Sequence (IVI-C only)	278

27.3.8	Clear All DigitalModulationBase Bit Sequences	279

27.4	IviRFSigGenDigitalModulationBase Behavior Model	280

27.5	IviRFSigGenDigitalModulationBase Compliance Notes	280

28.	IviRFSigGenCDMABase Extension Group	281

28.1	IviRFSigGenCDMABase Extension Group Overview	281

28.2	IviRFSigGenCDMABase Attributes	281

28.2.1	CDMA Standard Names (IVI.NET only)	282

28.2.2	CDMA Standard Name (IVI-COM only)	283

28.2.3	CDMA Standard Count (IVI-C and IVI-COM only)	284

28.2.4	CDMA Selected Standard	285

28.2.5	CDMA Trigger Source	286

28.2.6	CDMA External Trigger Slope	288

28.2.7	CDMA Test Model Names (IVI.NET only)	290

28.2.8	CDMA Test Model Name (IVI-COM only)	291

28.2.9	CDMA Test Model Count (IVI-C and IVI-COM only)	292

28.2.10	CDMA Selected Test Model	293

28.2.11	CDMA Clock Source	294

28.3	IviRFSigGenCDMABase Functions	296

28.3.1	Get CDMA Standard Name (IVI-C only)	297

28.3.2	Select CDMA Standard (IVI-C only)	298

28.3.3	Configure CDMA Clock Source (IVI-C only)	299

28.3.4	Configure CDMA Trigger Source (IVI-C only)	300

28.3.5	Configure CDMA External Trigger Slope (IVI-C only)	301

28.3.6	Get CDMA Test Model Name (IVI-C only)	302

28.3.7	Select CDMA Test Model (IVI-C only)	303

28.4	IviRFSigGenCDMABase Behavior Model	304

28.5	IviRFSigGenCDMABase Compliance Notes	304

29.	IviRFSigGenTDMABase Extension Group	305

29.1	IviRFSigGenTDMABase Extension Group Overview	305

29.2	IviRFSigGenTDMABase Attributes	305

29.2.1	TDMA Standard Names (IVI.NET only)	306

29.2.2	TDMA Standard Name (IVI-COM only)	307

29.2.3	TDMA Standard Count (IVI-C and IVI-COM only)	308

29.2.4	TDMA Selected Standard	309

29.2.5	TDMA Trigger Source	310

29.2.6	TDMA External Trigger Slope	312

29.2.7	TDMA Frame Names (IVI.NET only)	314

29.2.8	TDMA Frame Name (IVI-COM only)	315

29.2.9	TDMA Frame Count (IVI-C and IVI-COM only)	316

29.2.10	TDMA Selected Frame	317

29.2.11	TDMA Clock Source	318

29.2.12	TDMA External Clock Type	320

29.3	IviRFSigGenTDMABase Functions	322

29.3.1	Get TDMA Standard Name (IVI-C only)	323

29.3.2	Select TDMA Standard (IVI-C only)	324

29.3.3	Configure TDMA Clock Source	325

29.3.4	Configure TDMA Trigger Source (IVI-C only)	326

29.3.5	Configure TDMA External Trigger Slope (IVI-C only)	327

29.3.6	Get TDMA Frame Name (IVI-C only)	328

29.3.7	Select TDMA Frame (IVI-C only)	329

29.4	IviRFSigGenTDMABase Behavior Model	330

29.5	IviRFSigGenTDMABase Compliance Notes	330

30.	IviRFSigGen Attribute ID Definitions	331

31.	IviRFSigGen Attribute Value Definitions	335

32.	IviRFSigGen Function Parameter Value Definitions	350

33.	IviRFSigGen Error and Completion Code Value Definitions	351

33.1	IVI.NET IviRFSigGen Exceptions and Warnings	352

33.1.1	ListUnknownException	352

34.	IviRFSigGen Hierarchies	353

34.1	IviRFSigGen .NET Hierarchy	353

34.1.1	IviRFSigGen COM Interfaces	359

34.1.2	.NET Interface Reference Properties	360

34.2	IviRFSigGen COM Hierarchy	361

34.2.1	IviRFSigGen COM Interfaces	368

34.2.2	COM Interface Reference Properties	370

34.2.3	IviRFSigGen COM Category	370

34.3	IviRFSigGen C Function Hierarchy	371

34.4	IviRFSigGen C Attribute Hierarchy	375

Appendix A	Specific Driver Development Guidelines	380

A.1	Introduction	380

A.2	Disabling Unused Extensions	380

A.3	Query Instrument Status	385

Appendix B	Interchangeability Checking Rules	386

B.1	Introduction	386

B.2	When to Perform Interchangeability Checking	386

B.3	Interchangeability Checking Rules	386

IviRFSigGen Class Specification

IviRFSigGen Revision History

This section is an overview of the revision history of the IviRFSigGen specification.

| Table 1. IviRFSigGen Class Specification Revisions |  |  |
| --- | --- | --- |
| Revision Number | Date of Revision | Revision Notes |
| Revision 1.0 | March 2002 | Released version 1.0 of the spec |
| Revision 1.1 | February 2007 | Editorial changes: Correction of misspellings on pages 32, 80,101, 107, 251, 266, 309, and 357 . Interchangeablity checking rules for AM, FM and PM Source corrected |
| Revision 1.1 | October 2008 | Editorial change to update the IVI Foundation contact information in the Important Information section to remove obsolete address information and refer only to the IVI Foundation web site. |
| Revision 1.1 | April 2009 | Editorial change to update repeated capabilities section to include both qualified and unqualified repeated capability names. |
| Revision 2.0 | June 9, 2010 | Incorporated IVI.NET |
| Revision 2.0 | August 25, 2011 | Editorial IVI.NET change. Change references to process-wide locking to AppDomain-wide locking. Add an overload to the Create factory method that takes locking related parameters. |
| Revision 2.0 | September 24, 2015 | Editorial Change – Clarified the use of one-based index for C, and zero-based index for .NET for repeated capabilities in sections 8.3.1 and 10.3.1. |
| Revision 2.0 | October 14, 2016 | Editorial Change – Modified header text for table 33.2 to indicate that the messages do not apply to .NET exceptions. |

- API Versions

| Architecture | Drivers that comply with version 2.0 comply with all of the versions below. |
| --- | --- |
| C | 1.0, 1.1, 2.0 |
| COM | 1.0, 1.1, 2.0 |
| .NET | 2.0 |

Drivers that comply with this version of the specification also comply with earlier, compatible versions of the specification as shown in the table above.  The driver may benefit by advertising that it supports all the API versions listed in the table above.

# Overview of the IviRFSigGen Specification

## Introduction

This specification defines the IVI class for RF signal generators. The IviRFSigGen class is designed to support the typical RF signal generator as well as common extended functionality found in more complex instruments. This section summarizes the IviRFSigGen Specification itself and contains general information that the reader may need in order to understand, interpret, and implement aspects of this specification. These aspects include the following:

- IviRFSigGen Class Overview

- References

- Definitions of Terms and Acronyms

## IviRFSigGen Class Overview

This specification defines the IVI class for RF signal generators (RF SigGens). The IviRFSigGen class is designed to support the typical RF signal generator as well as common extended functionality found in more complex instruments. The IviRFSigGen class conceptualizes a RF signal generator as an instrument that can generate a sinussoidal carrier waveform with a wide variety of modulation types.

The IviRFSigGen class is divided into a base capability group and several extension groups. The base capability group is used to configure the carrier waveform. This includes setting the output frequency and amplitude, enabling or disabling the ALC and the RF output. The IviRFSigGen base capability group is described in Section 4, IviRFSigGenBase Capability Group.

The extension groups for analog and pulse modulation are IviRFSigGenModulateAM, IviRFSigGenModulateFM, IviRFSigGenModulatePM, IviRFSigGenAnalogModulationSource, IviRFSigGenModulatePulse, IviRFSigGenLFGenerator, IviRFSigGenLFGeneratorOutput, IviRFSigGenPulseGenerator, IviRFSigGenPulseDoubleGenerator and IviRFSigGenPulseGeneratorOutput.

The extension groups for sweeping are IviRFSigGenSweep, IviRFSigGenFrequencySweep, IviRFSigGenPowerSweep, IviRFSigGenFrequencyStep, IviRFSigGenPowerStep and IviRFSigGenList.

The extension groups for digital modulation are IviRFSigGenModulateIQ, IviRFSigGenIQImpairments, IviRFSigGenArbGeneration, IviRFSigGenDigitalModulationBase, IviRFSigGenCDMABase and IviRFSigGenTDMABase.

The remaining extension groups are IviRFSigGenALC, IviRFSigGenReferenceOscillator and IviRFSigGenSoftwareTrigger.

## References

Several other documents and specifications are related to this specification. These other related documents are the following:

- IVI-3.1: Architecture Overview Specification

- IVI-3.2: Inherent Capabilities Specification

- IVI-3.3: Standard Cross Class Capabilities

- IVI-3.18: IVI.NET Utility Classes and Interfaces Specification

- IVI-5.0: Glossary

- CCITT-V.52: Characteristics of distortion and error-rate measuring apparatus for data transmission

## Definitions of Terms and Acronyms

This section defines terms and acronyms that are specific to the IviRFSigGen class. Terms of more general interest are defined in IVI-5.0: Glossary.

ALC	Automatic Level Control, Used for controlling the RF SigGens output signal on a constant amplitude.

TDMA	Time Division Multiple Access

CDMA	Code Division Multiple Access

LF	Low Frequency

AM	Amplitude Modulation

FM	Frequency Modulation

PM	Phase Modulation

PRBS	Pseudo Random Bit Sequence

ARB	Arbitrary (Waveform)

# IviRFSigGen Class Capabilities

## Introduction

The IviRFSigGen specification divides signal generator capabilities into a base capabilities group and multiple extension capabilities groups. Each capability group is discussed in a separate section. This section defines names for each capability group and gives an overview of the information presented in each section.

## IviRFSigGen Group Names

The capability group names for the IviRFSigGen class are defined in the following table. The Group Name is used to represent a particular capability group and is returned as one of the possible group names from the IviRFSigGenGroupCapabilities attribute, see IVI-3.3: Standard Cross Class Capabilities Specification.

| Table 2-1. IviRFSigGen Group Names |  |
| --- | --- |
| Group Name | Description |
| IviRFSigGenBaseCapabilities | Base Capabilities: For generators that at minimum support setting the output frequency and amplitude, enabling or disabling the ALC and the RF output. |
| IviRFSigGenModulateAM | Extension Group: For generators with the ability to apply amplitude modulation to an output signal. |
| IviRFSigGenModulateFM | Extension Group: For generators with the ability to apply frequency modulation to an output signal. |
| IviRFSigGenModulatePM | Extension Group: For generators with the ability to apply phase modulation to an output signal. |
| IviRFSigGenAnalogModulationSource | Extension Group: For generators with at least one modulation source. |
| IviRFSigGenModulatePulse | Extension Group: For generators with the ability to apply pulse modulation to an output signal. |
| IviRFSigGenLFGenerator | Extension Group: For generators with an internal LF generator. |
| IviRFSigGenLFGeneratorOutput | Extension Group: For generators with an internal LF generator and a configurable output. |
| IviRFSigGenPulseGenerator | Extension Group: For generators with an internal pulse generator. |
| IviRFSigGenPulseDoubleGenerator | Extension Group: For generators with an internal pulse generator and the ability to generate a double pulse. |
| IviRFSigGenPulseGeneratorOutput | Extension Group: For generators with an internal pulse generator and a configurable output. |
| IviRFSigGenSweep | Extension Group: For generators with the ability to sweep the output frequency and/or power by analog means, discrete steps, or a list of values. |
| IviRFSigGenFrequencySweep | Extension Group: For generators with the ability to sweep the output frequency. |
| IviRFSigGenPowerSweep | Extension Group: For generators with the ability to sweep the output power. |
| IviRFSigGenFrequencyStep | Extension Group: For generators with the ability to sweep the output frequency in discrete steps. |
| IviRFSigGenPowerStep | Extension Group: For generators with the ability to sweep the output power in discrete steps. |
| IviRFSigGenList | Extension Group: For generators with the ability to sweep the output frequency and/or power by alist of values. |
| IviRFSigGenALC | Extension Group: For generators with the ability to use an automatic level control. |
| IviRFSigGenReferenceOscillator | Extension Group: For generators with the ability to use an external reference frequency. |
| IviRFSigGenSoftwareTrigger | Extension Group: For generators with the ability to trigger signal generation. |
| IviRFSigGenModulateIQ | Extension Group: For generators with the ability to apply vector (IQ) modulation to an output signal. |
| IviRFSigGenIQImpairment | Extension Group: For generators with the ability to apply impairment to vector (IQ) modulation. |
| IviRFSigGenArbGenerator | Extension Group: For generators with an arbitrary waveform generator as source for vector (IQ) modulation. |
| IviRFSigGenDigitalModulationBase | Extension Group: For generators with the ability to apply basic Digital Modulation |
| IviRFSigGenCDMABase | Extension Group: For generators with the ability to apply digital CDMA (Code Division Multiple Access) modulation. |
| IviRFSigGenTDMABase | Extension Group: For generators with the ability to apply digital TDMA (Time Division Multiple Access) modulation. |

## Repeated Capability Names

The IviRFSigGen Class Specification defines two repeated capabilities. Refer to the sections of IVI-3.1, Driver Architecture Specification that deal with repeated capabilities. The relevant sections are Section 2.7, Repeated Capabilities, Section 4.1.9, Repeated Capabilities, Section 4.2.5, Repeated Capabilities, Section 4.3.9, Repeated Capabilities, and Section 5.9, Repeated Capability Identifiers and Selectors.

- LFGenerator

- Analog Modulation Source

### LFGenerator

In the configuration store, the name for the LFGenerator repeated capability shall be exactly one of "LFGenerator" or “IviRFSigGenLFGenerator”.  Drivers that implement multiple repeated capabilities with the name “LFGenerator” shall use the latter form to disambiguate the names. The LFGenerator capability is used by the IviRFSigGenLFGenerator and IviRFSigGenLFGeneratorOutput extension groups and shall be available only if those groups are implemented.

When using the LFGenerator capability, it is necessary to set the Active LFGenerator attribute to an active repeated capability identifier before calling LFGenerator functions. All attributes and functions in the IviRFSigGenLFGenerator and IviRFSigGenLFGeneratorOutput extension groups operate on the repeated capability instance value set by the Active LFGenerator attribute.

### Analog Modulation Source

In the configuration store, the name for the analog modulation source repeated capability shall be exactly one of “AnalogModulationSource” or “IviRFSigGenAnalogModulationSource”.  Drivers that implement multiple repeated capabilities with the name “AnalogModulationSource” shall use the latter form to disambiguate the names.

The Analog Modulation Source capability is used by the IviRFSigGenAnalogModulationSource, IviRFSigGenModulateAM, IviRFSigGenModulateFM, and IviRFSigGenModulatePM extension groups and shall be available only if the IviRFSigGenAnalogModulationSource group is implemented.

## Boolean Attribute and Parameter Values

This specification uses True and False as the values for Boolean attributes and parameters. The following table defines the identifiers that are used for True and False in the IVI.NET, IVI-COM, and IVI-C architectures.

| Boolean Value | IVI.NET Identifier | IVI-COM Identifier | IVI-C Identifier |
| --- | --- | --- | --- |
| True | true | VARIANT_TRUE | VI_TRUE |
| False | false | VARIANT_FALSE | VI_FALSE |

## .NET Namespace

The .NET namespace for the IviRFSigGen class is Ivi.RFSigGen.

## .NET IviRFSigGen Session Factory

The IviRFSigGen .NET assembly contains a factory method called Create for creating instances of IviRFSigGen class-compliant IVI.NET drivers from driver sessions and logical names.  Create is a static method accessible from the static IviRFSigGen class.

Refer to IVI-3.5: Configuration Server Specification for a description of how logical names and session names are defined in the configuration store.

Refer to Section 8, IVI.NET Specific Driver Constructor, of IVI-3.2: Inherent Capabilities Specification, for more details on how the idQuery, reset, and options parameters affect the instantiation of the driver.

Refer to Section 4.3.11, Multithread Safety, of IVI-3.1: Driver Architecture Specification for a complete description of IVI.NET driver locking.  Refer to Section 8, Table 8.2 Required Lock Type Behavior for Drivers With the Same Access Key, of  IVI-3.2, Inherent Capability Specification, for an explanation of how the values for lockType and accessKey are used to determine the kind of multithreaded lock to use for the driver instance.

.NET Method Prototype

IIviRFSigGen Ivi.RFSigGen.Create(String name);

IIviRFSigGen Ivi.RFSigGen.Create(String name,

Boolean idQuery,

Boolean reset);

IIviRFSigGen Ivi.RFSigGen.Create(String name,

Boolean idQuery,

Boolean reset,

String options);

IIviRFSigGen Ivi.RFSigGen.Create(String resourceName,

Boolean idQuery,

Boolean reset,

LockType lockType,

String accessKey,

String options);

Parameters

| Inputs | Description | Base Type |  |
| --- | --- | --- | --- |
| name | A session name or a logical name that points to a session that uses an IVI.NET IviRFSigGen class-compliant driver. | String |  |
| idQuery | Specifies whether to verify the ID of the instrument. The default is False. | Boolean |  |
| reset | Specifies whether to reset the instrument. The default is False. | Boolean |  |
| lockType | Specifies whether to use AppDomain-wide locking or machine-wide locking. |  | Ivi.Driver.LockType |
| accessKey | Specifies a user-selectable access key to identify the lock. Driver instances that are created with the same accessKey will be protected from simultaneous access by multiple threads within an AppDomain or across AppDomains, depending upon the value of the lockType parameter. |  | String |
| options | A string that allows the user to specify the initial values of certain inherent attributes. The default is an empty string. | String |  |
| Outputs | Description | Base Type |  |
| Return Value | Interface pointer to the IIviRFSigGen interface of the driver referenced by session. | IIviRFSigGen |  |

      - Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| AppDomain | The lock is AppDomain-wide. |  |  |
|  |  | .NET | Ivi.Driver.LockType.AppDomain |
| Machine | The lock is machine-wide. |  |  |
|  |  | .NET | Ivi.Driver.LockType.Machine |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

Usage

To create a driver that implements the IviRFSigGen instrument class API from the logical name “My LogicalName”, use the following:

IIviRFSigGen rfSigGen = IviRFSigGen.Create(“MyLogicalName”);

In this case, the ID of the instrument will not be verified, the instrument will not be reset, and options will be supplied from the configuration store and/or driver defaults.

# General Requirements

This section describes the general requirements a specific driver shall meet in order to be compliant with this specification.  In addition, it provides general requirements that specific drivers shall meet in order to comply with a capability group, attribute, or function.

## Minimum Class Compliance

To be compliant with the IviRFSigGen Class Specification, an IVI specific driver shall conform to all of the requirements for an IVI class-compliant specific driver specified in IVI-3.1: Driver Architecure Specification. In addition it shall implement the inherent capabilities that IVI-3.2: Inherent Capabilities Specification defines, and the IviRFSigGenBase capability group.

### Disable

Refer to IVI-3.2: Inherent Capabilities Specification for the prototype of this function.

The Disable function shall cause the Signal Generator to apply the minimum amount of power possible at the output terminals. Setting the power level to a very small value or physically disconnecting the signal generator from the output terminals meets this requirement. Other techniques are also allowed.

## Capability Group Compliance

IVI-3.1: Architecture Overview Specification defines the general rules for a specific driver to be compliant with a capability group.

# IviRFSigGenBase Capabilities Group

## Overview

The IviRFSigGenBase capabilities group supports the basic signal generator capabilities.

## IviRFSigGenBase Attributes

The IviRFSigGenBase capabilities group defines the following attributes:

- Frequency

- Power Level

- ALC Enabled

- Output Enabled

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure RF |

.NET Property Name

RF.Frequency

COM Property Name

RF.Frequency

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY

Description

Specifies the frequency of the generated RF output signal. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Level

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure RF |

.NET Property Name

RF.Level

COM Property Name

RF.Level

C Constant Name

IVIRFSIGGEN_ATTR_POWER_LEVEL

Description

Specifies the power level of the RF output signal. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### ALC Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure ALC Enabled |

.NET Property Name

Alc.Enabled

COM Property Name

ALC.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_ALC_ENABLED

Description

If set to True, enables Automatic Level Control (ALC).  If set to False, disables Automatic Level Control (ALC).  Additional ALC attributes are in the ALC extension group.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

If a specific driver implements the value True for this attribute, it shall also implement the IviRFSigGenALC extension capability group

### Output Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Output Enabled |

.NET Property Name

RF.OutputEnabled

COM Property Name

RF.OutputEnabled

C Constant Name

IVIRFSIGGEN_ATTR_OUTPUT_ENABLED

Description

If set to True, the signal the RF signal generator produces appears at the output connector.  If set to False, the signal the RF signal generator produces does not appear at the output connector.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the value True.

## IviRFSigGenBase Functions

In addition to the IVI required functions, the IviRFSigGen Base capability group includes the following functions:

- Configure RF

- Configure ALC Enabled (IVI-C only)

- Configure Output Enabled (IVI-C only)

- Disable All Modulation

- Is Settled

- Wait Until Settled

This section describes the behavior and requirements of each function.

### Configure RF

Description

Configures the frequency and the power level of the RF output signal.

.NET Method Prototype

void RF.Configure (Double frequency,
                   Double powerLevel);

COM Method Prototype

HRESULT RF.Configure ([in] DOUBLE Frequency,
                      [in] DOUBLE PowerLevel);

C Prototype

ViStatus IviRFSigGen_ConfigureRF (ViSession Vi,
                                  ViReal64 Frequency,
                                  ViReal64 PowerLevel);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| Frequency | Specifies the frequency of the generated RF signal. The driver uses this value to set the Frequency attribute. See the attribute description for more details. |
| PowerLevel | Specifies the power level of the generated RF signal. The driver uses this value to set the Power Level attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure ALC Enabled (IVI-C only)

Description

Enables or disables the Automatic Level Control (ALC).

.NET Method Prototype

N/A

(use the Alc.Enabled property)

COM Method Prototype

N/A

(use the ALC.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureALCEnabled (ViSession Vi, ViBoolean ALCEnabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| ALCEnabled | Specifies whether the ALC is used. The driver uses this value to set the ALC Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Output Enabled (IVI-C only)

Description

Enables or disables the RF output signal.

.NET Method Prototype

N/A

(use the RF.OutputEnabled property)

COM Method Prototype

N/A

(use the RF.OutputEnabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureOutputEnabled (ViSession Vi, ViBoolean OutputEnabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| OutputEnabled | Specifies whether the RF output is enabled. The driver uses this value to set the Output Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Disable All Modulation

Description

Disables all currently enabled modulations (e.g. analog, pulse, IQ, and digital modulation).

.NET Method Prototype

void RF.DisableAllModulation();

COM Method Prototype

HRESULT RF.DisableAllModulation();

C Prototype

ViStatus IviRFSigGen_DisableAllModulation (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Is Settled

Description

Queries if the RF output signal is currently settled.

.NET Method Prototype

Boolean RF.IsSettled ();

COM Method Prototype

HRESULT RF.IsSettled ([out, retval] VARIANT_BOOL *Done);

C Prototype

ViStatus IviRFSigGen_IsSettled (ViSession Vi, ViBoolean * Done);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

| Outputs | Description |
| --- | --- |
| Done (C/COM) | Returns True if the output signal is in settled, False otherwise. |
| Return Value (.NET) | Returns true if the output signal is in settled, false otherwise. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Wait Until Settled

Description

This function waits until the state of the RF output signal has settled.

.NET Method Prototype

void RF.WaitUntilSettled (PrecisionTimeSpan maximumTime);

COM Method Prototype

HRESULT RF.WaitUntilSettled ([in] LONG MaxTimeMilliseconds);

C Prototype

ViStatus IviRFSigGen_WaitUntilSettled (ViSession Vi, ViInt32 MaxTimeMilliseconds);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| MaxTimeMilliseconds | Defines the maximum time the function waits for the output to be settled. If the maximum time is exceeded, this function returns the Max Time Exceeded error.The units are milliseconds. |
| MaximumTime | Defines the maximum time the function waits for the output to be settled. If the maximum time is exceeded, this function returns the Max Time Exceeded error. |

Defined Values for MaxTimeMilliseconds Parameter (C and COM)

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Max Time Immediate | Sets timeout to immediate. The function returns immediately. |  |  |  |
|  |  | C | IVIRFSIGGEN_VAL_MAX_TIME_IMMEDIATE |  |
|  |  | COM | IviRFSigGenMaxTimeImmediate |  |
| Max Time Infinite | Sets timeout to infinite. The function waits indefinitely for the settling to complete. |  |  |  |
|  |  | C | IVIRFSIGGEN_VAL_MAX_TIME_INFINITE |  |
|  |  | COM | IviRFSigGenMaxTimeInfinite |  |

Defined Values for MaximumTime Parameter (.NET)

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Zero | Sets timeout to immediate. The function returns immediately. |  |  |  |
|  |  | .NET | PrecisionTimeSpan.Zero |  |
| MaxValue | Sets timeout to infinite. The function waits indefinitely for the settling to complete. |  |  |  |
|  |  | C | PrecisionTimeSpan.MaxValue |  |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return. The table below specifies additional class-defined status codes for this function.

| Completion Codes | Description |
| --- | --- |
| Max Time Exceeded | Error: Maximum time exceeded before the operation completed. |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

Note that the .NET MaxTimeExceededException is defined in IVI-3.2: Inherent Capabilities Specification.

Compliance Notes

An IviRFSigGen specific driver need not implement the Max Time Immediate or the Max Time Infinite defined values for the MaxTimeMilliseconds parameter to be compliant with the IviRFSigGenBase Capability group.

## IviRFSigGenBase Behavior Model

After the user calls the Initialize or Reset functions, the RF signal generator produces an output signal based on its current configuration. All changes to the RF signal generator’s IviRFSigGenBase functions and attributes take place immediately.

If the user executes the Wait Until Settled function, the driver will block any further operation until the function completes (i.e. the signal at the output of the RF signal generator has settled).

# IviRFSigGenModulateAM Extension Group

## IviRFSigGenModulateAM Extension Group Overview

The IviRFSigGenModulateAM Extension Group supports signal generators that can apply amplitude modulation to the RF output signal. The user can enable or disable amplitude modulation, specify the source and coupling of the modulating signal and the modulation depth with linear or logarithmic attenuation.

## IviRFSigGenModulateAM Attributes

The IviRFSigGenModulateAM extension group defines the following attributes:

- AM Enabled

- AM Source

- AM Scaling

- AM External Coupling

- AM Nominal Voltage

- AM Depth

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### AM Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure AM Enabled |

.NET Property Name

AnalogModulation.AM.Enabled

COM Property Name

AnalogModulation.AM.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_AM_ENABLED

Description

If set to True, the RF signal generator applies amplitude modulation to the RF output signal.  If set to False, the RF signal generator does not apply amplitude modulation to the output signal.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### AM Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Configure AM |

.NET Property Name

AnalogModulation.AM.Source

COM Property Name

AnalogModulation.AM.Source

C Constant Name

IVIRFSIGGEN_ATTR_AM_SOURCE

Description

Specifies the source of the signal that is used as the modulating signal. If more than one source is specified, the voltages of all specified sources (internal and external) are summed. Multiple source names are separated by commas.

Defined Values

To get the valid source names, use the Get Modulation Source Name function (IVI-C) or the Analog Modulation Source Name property (IVI-COM) from the IviRFSigGenAnalogModulationSource extension group.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### AM Scaling

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure AM |

.NET Property Name

AnalogModulation.AM.Scaling

.NET Enumeration Name

AMScaling

COM Property Name

AnalogModulation.AM.Scaling

COM Enumeration Name

IviRFSigGenAMScalingEnum

C Constant Name

IVIRFSIGGEN_ATTR_AM_SCALING

Description

Specifies linear or logarithmic characteristic for amplitude modulation. The units of the AM Depth attribute is changed with this setting.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Linear | Enables linear attenuation for amplitude modulation. |  |  |
|  |  | .NET | AMScaling.Linear |
|  |  | C | IVIRFSIGGEN_VAL_AM_SCALING_LINEAR |
|  |  | COM | IviRFSigGenAMScalingLinear |
| Logarithmic | Enables logarithmic attenuation for amplitude modulation. |  |  |
|  |  | .NET | AMScaling.Logarithmic |
|  |  | C | IVIRFSIGGEN_VAL_AM_SCALING_LOGARITHMIC |
|  |  | COM | IviRFSigGenAMScalingLogarithmic |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_AM_SCALING_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_AM_SCALING_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_AM_SCALING_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to AM Scale Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of AM Scaling Specific Ext Base, IVIRFSIGGEN_VAL_AM_SCALING_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_AM_SCALING_CLASS_EXT_BASE.

### AM External Coupling

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure AM External Coupling |

.NET Property Name

AnalogModulation.AM.ExternalCoupling

.NET Enumeration Name

ExternalCoupling

COM Property Name

AnalogModulation.AM.ExternalCoupling

COM Enumeration Name

IviRFSigGenAMExternalCouplingEnum

C Constant Name

IVIRFSIGGEN_ATTR_AM_EXTERNAL_COUPLING

Description

Specifies the coupling of the external source of the modulating signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| AC | The external source is coupled for AC only. |  |  |
|  |  | .NET | ExternalCoupling.AC |
|  |  | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_AC |
|  |  | COM | IviRFSigGenAMExternalCouplingAC |
| DC | The external source is coupled for both DC and AC |  |  |
|  |  | .NET | ExternalCoupling.DC |
|  |  | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_DC |
|  |  | COM | IviRFSigGenAMExternalCouplingDC |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to AM External Coupling Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of AM External Coupling Specific Ext Base, IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_CLASS_EXT_BASE.

### AM Nominal Voltage

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | N/A | None | None |

.NET Property Name

AnalogModulation.AM.NominalVoltage

COM Property Name

AnalogModulation.AM.NominalVoltage

C Constant Name

IVIRFSIGGEN_ATTR_AM_NOMINAL_VOLTAGE

Description

Returns the voltage at which the instrument achieves the amount of modulation specified by the AM Depth attribute. The units are Volts.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### AM Depth

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure AM |

.NET Property Name

AnalogModulation.AM.Depth

COM Property Name

AnalogModulation.AM.Depth

C Constant Name

IVIRFSIGGEN_ATTR_AM_DEPTH

Description

Specifies the extent of modulation the signal generator applies to the RF-signal (carrier waveform) with the modulating signal as a result of summing all sources, internal and external. The amount of the specified modulation depth is achieved with a modulating voltage of AM Nominal Voltage.
If the AM Scaling attribute is set to Linear, then the units are percent (%). If the AM Scaling attribute is set to logarithmic, then the units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenModulateAM Functions

The IviRFSigGenModulateAM extension defines the following functions:

- Configure AM Enabled (IVI-C only)

- Configure AM External Coupling (IVI-C only)

- Configure AM

This section describes the behavior and requirements of each function.

### Configure AM Enabled (IVI-C only)

Description

Configures the signal generator to apply amplitude modulation to the RF output signal.

.NET Method Prototype

N/A

(use the AnalogModulation.AM.Enabled property)

COM Method Prototype

N/A

(use the AnalogModulation.AM.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureAMEnabled (ViSession Vi, ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | Enables or disables amplitude modulation. The driver uses this value to set the AM Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure AM External Coupling (IVI-C only)

Description

Configures the coupling of the external source the signal generator uses for amplitude modulation of the output signal.

.NET Method Prototype

N/A

(use the AnalogModulation.AM.ExternalCoupling property)

COM Method Prototype

N/A

(use the AnalogModulation.AM.ExternalCoupling property)

C Prototype

ViStatus IviRFSigGen_ConfigureAMExternalCoupling (ViSession Vi,
ViInt32 Coupling);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Coupling | Specifies the coupling of the external source. The driver uses this value to set the AM External Coupling attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure AM

Description

Configures the attributes that control the signal generator's amplitude modulation. These attributes are the AM modulation source, scaling and depth.

.NET Method Prototype

void AnalogModulation.AM.Configure (String source,
                                    AMScaling scaling,
                                    Double depth);

COM Method Prototype

HRESULT AnalogModulation.AM.Configure ([in] BSTR Source,
                                       [in] IviRFSigGenAMScalingEnum Scaling,
                                       [in] DOUBLE Depth);

C Prototype

ViStatus IviRFSigGen_ConfigureAM (ViSession Vi, ViConstString Source,
ViInt32 Scaling,
ViReal64 Depth);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | Specifies the source of the signal that the signal generator uses to modulate the output signal. The driver uses this value to set the AM Source attribute. See the attribute description for more details. |
| Scaling | Specifies the scaling of the modulation. The driver uses this value to set the AM Scaling attribute. See the attribute description for more details. |
| Depth | Specifies the extent of modulation. The driver uses this value to set the AM Depth attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenModulateAM Behavior Model

The IviRFSigGenModulateAM Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenModulateAM settings.

## IviRFSigGenModulateAM Compliance Notes

- If a specific driver implements the IviRFSigGenModulateAM Extension Group, it shall also implement the IviRFSigGenAnalogModulationSource Extension Group.

- If the AM source attribute is not set to an external modulation source, the AM External Coupling attribute need not to be in a user specified state.

# IviRFSigGenModulateFM Extension Group

## IviRFSigGenModulateFM Extension Group Overview

The IviRFSigGenModulateFM Extension Group supports signal generators that can apply frequency modulation to the RF output signal. The user can enable or disable frequency modulation, specify the source and coupling of the modulating signal and the peak frequency deviation.

## IviRFSigGenModulateFM Attributes

The IviRFSigGenModulateFM extension group defines the following attributes:

- FM Enabled

- FM Source

- FM External Coupling

- FM Nominal Voltage

- FM Deviation

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### FM Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure FM Enabled |

.NET Property Name

AnalogModulation.FM.Enabled

COM Property Name

AnalogModulation.FM.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_FM_ENABLED

Description

If set to True, the RF signal generator applies frequency modulation to the RF output signal.  If set to False, the RF signal generator does not apply frequency modulation to the RF output signal.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### FM Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Configure FM |

.NET Property Name

AnalogModulation.FM.Source

COM Property Name

AnalogModulation.FM.Source

C Constant Name

IVIRFSIGGEN_ATTR_FM_SOURCE

Description

Specifies the source of the signal that is used as the modulating signal. If more than one source is specified, the voltages of all sources (internal and external) are summed. Multiple source names are separated by commas.

Defined Values

To get the values (names) use the Get Modulation Source Name function  (IVI-C) or the Analog Modulation Source Name property (IVI-COM) from the IviRFSigGenAnalogModulationSource extension group.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### FM External Coupling

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure FM External Coupling |

.NET Property Name

AnalogModulation.FM.ExternalCoupling

.NET Enumeration Name

ExternalCoupling

COM Property Name

AnalogModulation.FM.ExternalCoupling

COM Enumeration Name

IviRFSigGenFMExternalCouplingEnum

C Constant Name

IVIRFSIGGEN_ATTR_FM_EXTERNAL_COUPLING

Description

Specifies the coupling of the external source of the modulating signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| AC | The external source is coupled for AC only. |  |  |
|  |  | .NET | ExternalCoupling.AC |
|  |  | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_AC |
|  |  | COM | IviRFSigGenFMExternalCouplingAC |
| DC | The external source is coupled for both DC and AC |  |  |
|  |  | .NET | ExternalCoupling.DC |
|  |  | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_DC |
|  |  | COM | IviRFSigGenFMExternalCouplingDC |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to FM External Coupling Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of FM External Coupling Specific Ext Base, IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_CLASS_EXT_BASE.

### FM Nominal Voltage

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | N/A | None | None |

.NET Property Name

AnalogModulation.FM.NominalVoltage

COM Property Name

AnalogModulation.FM.NominalVoltage

C Constant Name

IVIRFSIGGEN_ATTR_FM_NOMINAL_VOLTAGE

Description

Returns the voltage at which the instrument achieves the amount of modulation specified by the FM Deviation attribute. The units are Volts.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### FM Deviation

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure FM |

.NET Property Name

AnalogModulation.FM.Deviation

COM Property Name

AnalogModulation.FM.Deviation

C Constant Name

IVIRFSIGGEN_ATTR_FM_DEVIATION

Description

Specifies the extent of modulation (peak frequency deviation) the signal generator applies to the RF-signal (carrier waveform) with the modulating signal as a result of summing all sources, internal and external. The amount of the specified FM modulation deviation is achieved with a modulating voltage of FM Nominal Voltage.The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenModulateFM Functions

The IviRFSigGenModulateFM extension group defines the following functions:

- Configure FM Enabled (IVI-C only)

- Configure FM External Coupling (IVI-C only)

- Configure FM

This section describes the behavior and requirements of each function.

### Configure FM Enabled (IVI-C only)

Description

Configures the signal generator to apply frequency modulation to the RF output signal.

.NET Method Prototype

N/A

(use the AnalogModulation.FM.Enabled property)

COM Method Prototype

N/A

(use the AnalogModulation.FM.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureFMEnabled (ViSession Vi, ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | Enables or disables frequency modulation. The driver uses this value to set the FM Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure FM External Coupling (IVI-C only)

Description

Configures the coupling of the external source the generator uses for frequency modulation of the output signal.

.NET Method Prototype

N/A

(use the AnalogModulation.FM.ExternalCoupling property)

COM Method Prototype

N/A

(use the AnalogModulation.FM.ExternalCoupling property)

C Prototype

ViStatus IviRFSigGen_ConfigureFMExternalCoupling (ViSession Vi,
           ViInt32 Coupling);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Coupling | Specifies the coupling of the external source. The driver uses this value to set the FM External Coupling attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure FM

Description

Configures the attributes that control the signal generator's frequency modulation. The attributes are the modulation deviation and the modulating source(s).

.NET Method Prototype

void AnalogModulation.FM.Configure (String source,
                                    Double deviation);

COM Method Prototype

HRESULT AnalogModulation.FM.Configure ([in] BSTR Source,
                                       [in] DOUBLE Deviation);

C Prototype

ViStatus IviRFSigGen_ConfigureFM (ViSession Vi, ViConstString Source,
ViReal64 Deviation);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | Specifies the modulating source. The driver uses this value to set the FM Source attribute. See the attribute description for more details. |
| Deviation | Specifies the extent of modulation. The driver uses this value to set the FM Deviation attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenModulateFM Behavior Model

The IviRFSigGenModulateFM Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenModulateFM settings.

## IviRFSigGenModulateFM Compliance Notes

- If a specific driver implements the IviRFSigGenModulateFM Extension Group, it shall also implement the IviRFSigGenAnalogModulationSource Extension Group.

- If the FM Source attribute is not set to an external modulation source, the FM External Coupling attribute need not to be in a user specified state.

# IviRFSigGenModulatePM Extension Group

## IviRFSigGenModulatePM Extension Group Overview

The IviRFSigGenModulatePM Extension Group supports signal generators that can apply phase modulation to the RF output signal. The user can enable or disable phase modulation, specify the source and coupling of the modulating signal and the peak phase deviation.

## IviRFSigGenModulatePM Attributes

The IviRFSigGenModulatePM extension group defines the following attributes:

- PM Enabled

- PM Source

- PM External Coupling

- PM Nominal Voltage

- PM Deviation

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### PM Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure PM Enabled |

.NET Property Name

AnalogModulation.PM.Enabled

COM Property Name

AnalogModulation.PM.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_PM_ENABLED

Description

If set to True, the RF signal generator applies phase modulation to the RF output signal.  If set to False, the RF signal generator does not apply phase modulation to the RF output signal.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### PM Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Configure PM |

.NET Property Name

AnalogModulation.PM.Source

COM Property Name

AnalogModulation.PM.Source

C Constant Name

IVIRFSIGGEN_ATTR_PM_SOURCE

Description

Specifies the source of the signal that is used as the modulating signal. If more than one source is specified, the voltages of all sources (internal and external) are summed. Multiple source names are separated by commas.

Defined Values

To get the values (names) use the Get Modulation Source Name function  (IVI-C) or the Analog Modulation Source Name property (IVI-COM) from the IviRFSigGenAnalogModulationSource extension group.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### PM External Coupling

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure PM External Coupling |

.NET Property Name

AnalogModulation.PM.ExternalCoupling

.NET Enumeration Name

ExternalCoupling

COM Property Name

AnalogModulation.PM.ExternalCoupling

COM Enumeration Name

IviRFSigGenPMExternalCouplingEnum

C Constant Name

IVIRFSIGGEN_ATTR_PM_EXTERNAL_COUPLING

Description

Specifies the coupling of the external source of the modulating signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| AC | The external source is coupled for AC only. |  |  |
|  |  | .NET | ExternalCoupling.AC |
|  |  | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_AC |
|  |  | COM | IviRFSigGenPMExternalCouplingAC |
| DC | The external source is coupled for both DC and AC |  |  |
|  |  | .NET | ExternalCoupling.DC |
|  |  | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_DC |
|  |  | COM | IviRFSigGenPMExternalCouplingDC |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to PM External Coupling Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of AM External Coupling Specific Ext Base, IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_CLASS_EXT_BASE.

### PM Nominal Voltage

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | N/A | None | None |

.NET Property Name

AnalogModulation.PM.NominalVoltage

COM Property Name

AnalogModulation.PM.NominalVoltage

C Constant Name

IVIRFSIGGEN_ATTR_PM_NOMINAL_VOLTAGE

Description

Returns the voltage at which the instrument achieves the amount of modulation specified by the PM Deviation attribute. The units are Volts.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### PM Deviation

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure PM |

.NET Property Name

AnalogModulation.PM.Deviation

COM Property Name

AnalogModulation.PM.Deviation

C Constant Name

IVIRFSIGGEN_ATTR_PM_DEVIATION

Description

Specifies the extent of modulation (peak phase deviation) the signal generator applies to the RF-signal (carrier waveform) with the modulating signal as a result of summing all sources, internal and external. The amount of the specified PM modulation deviation is achieved with a modulating voltage of PM Nominal Voltage. The units are radians.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenModulatePM Functions

The IviRFSigGenModulatePM extension group defines the following functions:

- Configure PM Enabled (IVI-C only)

- Configure PM External Coupling (IVI-C only)

- Configure PM

This section describes the behavior and requirements of each function.

### Configure PM Enabled (IVI-C only)

Description

Configures the signal generator to apply phase modulation to the RF output signal.

.NET Method Prototype

N/A

(use the AnalogModulation.PM.Enabled property)

COM Method Prototype

N/A

(use the AnalogModulation.PM.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigurePMEnabled (ViSession Vi, ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | Enables or disables phase modulation. The driver uses this value to set the PM Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure PM External Coupling (IVI-C only)

Description

Configures the coupling of the external source for phase modulation.

.NET Method Prototype

N/A

(use the AnalogModulation.PM.ExternalCoupling property)

COM Method Prototype

N/A

(use the AnalogModulation.PM.ExternalCoupling property)

C Prototype

ViStatus IviRFSigGen_ConfigurePMExternalCoupling (ViSession Vi,
           ViInt32 Coupling);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Coupling | Specifies the coupling of the external source. The driver uses this value to set the PM External Coupling attribute. See the attribute description for more details. |

Return Values (C)

### Configure PM

Description

Configures the attributes that control the signal generator's phase modulation. The attributes are the modulation deviation and the modulating source(s).

.NET Method Prototype

void AnalogModulation.PM.Configure (String source,
                                    Double deviation);

COM Method Prototype

HRESULT AnalogModulation.PM.Configure ([in] BSTR Source,
                                       [in] DOUBLE Deviation);

C Prototype

ViStatus IviRFSigGen_ConfigurePM (ViSession Vi,
ViConstString Source,
ViReal64 Deviation);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | Specifies the source of the signal the signal generator uses to modulate the output signal. The driver uses this value to set the PM Source attribute. See the attribute description for more details. |
| Deviation | Specifies the extent of modulation. The driver uses this value to set the PM Deviation attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenModulatePM Behavior Model

The IviRFSigGenModulatePM Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenModulatePM settings.

## IviRFSigGenModulatePM Compliance Notes

- If a specific driver implements the IviRFSigGenModulatePM Extension Group, it shall also implement the IviRFSigGenAnalogModulationSource Extension Group.

- If the PM source attribute is not set to an external modulation source, the PM External Coupling attribute need not to be in a user specified state.

# IviRFSigGenAnalogModulationSource Extension Group

## IviRFSigGenAnalogModulationSource Extension Group Overview

The source of the modulating signal is a repeated capability. One or more internal sources (LF Generators) and/or one and more external sources can be combined. The voltage of all signals is summed before modulating the RF Signal. This applies to the following extension groups:

- IviRFSigGenModulateAM

- IviRFSigGenModulateFM

- IviRFSigGenModulatePM

Typically, all the LFGenerators defined in the IviRFSigGenLFGenerator extension group have a corresponding Analog Modulation Source.

## IviRFSigGenAnalogModulationSource Attributes

The IviRFSigGenAnalogModulationSource extension group defines the following attributes:

- Analog Modulation Source Count

- Analog Modulation Source Name (IVI-COM only)

This section describes the behavior and requirements of this attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Analog Modulation Source Count

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None |  |

.NET Property Name

AnalogModulation.Source.Count

COM Property Name

AnalogModulation.Source.Count

C Constant Name

IVIRFSIGGEN_ATTR_ANALOG_MODULATION_SOURCE_COUNT

Description

Specifies how many analog modulation sources are available.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Analog Modulation Source Name (IVI-COM Only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A
(Use the Get Analog Modulation Source Name function.)

COM Property Name

HRESULT AnalogModulation.Source.Name ([in] LONG Index,
                                      [out,retval] BSTR* Name);

C Constant Name

N/A
(Use the Get Analog Modulation Source Name function.)

Description

This property returns the Analog Modulation Source identifier that corresponds to the one-based index that the user specifies.  If the driver defines a qualified analog modulation source name, this property returns the qualified name.  If the value that the user pass for the Index parameter is less than one or greater than the value of the Analog Modulation Source Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the analog modulation source name. | ViChar[] |

## IviRFSigGenAnalogModulationSource Functions

The IviRFSigGenAnalogModulationSource extension group defines the following functions:

- Get Analog Modulation Source Name (IVI-C & IVI.NET only)

This section describes the behavior and requirements of each function.

### Get Analog Modulation Source Name (IVI-C & IVI.NET only)

Description

This function returns the specific driver defined analog modulation source name that corresponds to the index that the user specifies.  If the driver defines a qualified analog modulation source name, this property returns the qualified name.  In C, the index is one-based.  In .NET, the index is zero-based.

.NET Method Prototype

String AnalogModulation.Source.GetName (Int32 index);

COM Method Prototype

N/A

(Use AnalogModulation.Source.Name property)

C Prototype

ViStatus IviRFSigGen_GetAnalogModulationSourceName (ViSession Vi,
                                                    ViInt32 Index,
                                                    ViInt32 NameBufferSize,
                                                    ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | An index (one-based for IVI-C, zero-based for IVI.NET) that defines which name to return. |
| NameBufferSize | The number of bytes in the viChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name (C) | A user-allocated buffer into which the driver stores the Analog Modulation Source name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |
| Return Value (.NET) | The Analog Modulation Source name corresponding to the user specified index. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

For an instrument with only one modulation source, that is the Analog Modulation Source Count attribute is one, the driver may return an empty string in the Name parameter.

## IviRFSigGenAnalogModulationSource Behavior Model

The IviRFSigGenAnalogModulationSource Extension Group follows the behavior model of the IviRFSigGenBase capability group.

## IviRFSigGenAnalogModulationSource Compliance Notes

If the driver supports this extension group it shall support the IviRFSigGenBase capabilities and at least one of the following extension groups:

- IviRFSigGenModulateAM

- IviRFSigGenModulateFM

- IviRFSigGenModulatePM

# IviRFSigGenModulatePulse Extension Group

## IviRFSigGenModulatePulse Extension Group Overview

The IviRFSigGenModulatePulse Extension Group supports signal generators that can apply pulse modulation to the RF output signal. The user can enable or disable pulse modulation, and specify the source and the polarity of the modulating signal.

## IviRFSigGenModulatePulse Attributes

The IviRFSigGenModulatePulse extension group defines the following attributes:

- Pulse Modulation Enabled

- Pulse Modulation Source

- Pulse Modulation External Polarity

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Pulse Modulation Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Pulse Modulation Enabled |

.NET Property Name

PulseModulation.Enabled

COM Property Name

PulseModulation.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_MODULATION_ENABLED

Description

If set to True, enables pulse modulation of the RF output signal.  If set to False, disables pulse modulation of the RF output signal .

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### Pulse Modulation Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Pulse Modulation Source |

.NET Property Name

PulseModulation.Source

.NET Enumeration Name

PulseModulationSource

COM Property Name

PulseModulation.Source

COM Enumeration Name

IviRFSigGenPulseModulationSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_MODULATION_SOURCE

Description

Specifies the source of the signal that is used as the modulating signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The internal pulse generator (IviRFSigGenPulseGenerator Extension Group) is used for modulation. |  |  |
|  |  | .NET | PulseModulationSource.Internal |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenPulseModulationSourceInternal |
| External | An external generator is used for modulation. |  |  |
|  |  | .NET | PulseModulationSource.External |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenPulseModulationSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Pulse Modulation Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Pulse ModulationSource Specific Ext Base, IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_CLASS_EXT_BASE.

### Pulse Modulation External Polarity

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Pulse Modulation External Polarity |

.NET Property Name

PulseModulationExternalPolarity PulseModulation.ExternalPolarity

.NET Enumeration Name

PulseModulationExternalPolarity

COM Property Name

PulseModulation.ExternalPolarity

COM Enumeration Name

IviRFSigGenPulseModulationExternalPolarityEnum

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_MODULATION_EXTERNAL_POLARITY

Description

Specifies the polarity of the external source signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Normal | The signal generator modulates the carrier signal with normal pulse polarity. Increasing the positive pulse voltage level results in higher RF level. |  |  |
|  |  | .NET | PulseModulationExternalPolarity.Normal |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_NORMAL |
|  |  | COM | IviRFSigGenPulseModulationExternalPolarityNormal |
| Inverse | The signal generator modulates the carrier signal with inverted pulse polarity. Increasing the positive pulse voltage level results in lower RF level. |  |  |
|  |  | .NET | PulseModulationExternalPolarity.Inverse |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_INVERSE |
|  |  | COM | IviRFSigGenPulseModulationExternalPolarityInverse |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Pulse Modulation External Polarity Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Pulse Modulation Specific Ext Base, IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_CLASS_EXT_BASE.

## IviRFSigGenModulatePulse Functions

The IviRFSigGenPulseModulation extension group defines the following functions:

- Configure Pulse Modulation Enabled (IVI-C only)

- Configure Pulse Modulation Source (IVI-C only)

- Configure Pulse Modulation External Polarity (IVI-C only)

This section describes the behavior and requirements of each function.

### Configure Pulse Modulation Enabled (IVI-C only)

Description

Configures the signal generator to apply pulse modulation to the RF output signal.

.NET Method Prototype

N/A

(use the PulseModulation.Enabled property)

COM Method Prototype

N/A

(use the PulseModulation.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigurePulseModulationEnabled (ViSession Vi, ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | True enables pulse modulation. The driver uses this value to set the Pulse Modulation Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Pulse Modulation Source (IVI-C only)

Description

Configures the source the signal generator uses for pulse modulation of the RF output signal.

.NET Method Prototype

N/A

(use the PulseModulation.Source property)

COM Method Prototype

N/A

(use the PulseModulation.Source property)

C Prototype

ViStatus IviRFSigGen_ConfigurePulseModulationSource (ViSession Vi, ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | Specifies the source of the signal, the signal generator uses to modulate the output signal. The driver uses this value to set the Pulse Modulation Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Pulse Modulation External Polarity (IVI-C only)

Description

Specifies the polarity of the external source signal.

.NET Method Prototype

N/A

(use the PulseModulation.ExternalPolarity property)

COM Method Prototype

N/A

(use the PulseModulation.ExternalPolarity property)

C Prototype

ViStatus IviRFSigGen_ConfigurePulseModulationExternalPolarity (ViSession Vi,
 ViInt32 Polarity);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Polarity | Specifies the polarity of the signal used as the external modulation source. The driver uses this value to set the Pulse Modulation External Polarity attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenModulatePulse Behavior Model

The IviRFSigGenModulatePulse Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenModulatePulse settings.

## IviRFSigGenModulatePulse Compliance Notes

- If a specific driver supports the value Internal for the Pulse Modulation Source attribute, it shall also implement the IviRFSigGenPulseGenerator Extension Group.

- If a specific driver does not support an external pulse modulation source, it need not support the Configure Pulse Modulation External Polarity function and the Pulse Modulation External Polarity attribute.

# IviRFSigGenLFGenerator Extension Group

## IviRFSigGenLFGenerator Extension Group Overview

The IviRFSigGenLFGenerator Extension Group supports the LF generator (within the RF signal generator), that is normally used as a source for the modulators. To use the LF generator as a modulation source, the modulation source in the ModulateAM extensions group, ModulateFM extension group or ModulatePM extension group should be configured. The user can set the frequency and the waveform of the LF signal.

## IviRFSigGenLFGenerator Attributes

The IviRFSigGenLFGenerator extension group defines the following attributes:

- Active LFGenerator

- LFGenerator Count

- LFGenerator Name (IVI-COM only)

- LFGenerator Frequency

- LFGenerator Waveform

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Active LFGenerator

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Set Active LFGenerator |

.NET Property Name

LFGenerator.ActiveLFGenerator

COM Property Name

LFGenerator.ActiveLFGenerator

C Constant Name

IVIRFSIGGEN_ATTR_ACTIVE_LF_GENERATOR

Description

Specifies the LF generator which is currently active. The values for this attribute correspond to the LFGenerator repeated capability.  If the driver defines a qualified LF generator name, this property returns the qualified name.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### LFGenerator Count

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

LFGenerator.Count

COM Property Name

LFGenerator.Count

C Constant Name

IVIRFSIGGEN_ATTR_LF_GENERATOR_COUNT

Description

Specifies the number of LF generator sources available for a particular instrument.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### LFGenerator Name (IVI-COM Only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A
(Use the Get LFGenerator Name function.)

COM Property Name

HRESULT LFGenerator.Name ([in] LONG Index,
                          [out,retval] BSTR* Name);

C Constant Name

N/A
(Use the Get LFGenerator Name function.)

Description

This property returns the LFGenerator identifier that corresponds to the one-based index that the user specifies.  If the driver defines a qualified LFGenerator name, this property returns the qualified name. If the value that the user pass for the Index parameter is less than one or greater than the value of the LFGenerator Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the LFGenerator name. | ViChar[] |

### LFGenerator Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure LF Generator |

.NET Property Name

LFGenerator.Frequency

COM Property Name

LFGenerator.Frequency

C Constant Name

IVIRFSIGGEN_ATTR_LF_GENERATOR_FREQUENCY

Description

Specifies the frequency of the active LF generator. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### LFGenerator Waveform

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure LF Generator |

.NET Property Name

LFGenerator.Waveform

.NET Enumeration Name

LFGeneratorWaveform

COM Property Name

LFGenerator.Waveform

COM Enumeration Name

IviRFSigGenLFGeneratorWaveformEnum

C Constant Name

IVIRFSIGGEN_ATTR_LF_GENERATOR_WAVEFORM

Description

Specifies the waveform of the active LF generator.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Sine | Configures the LF generator to produce a sinusoidal waveform |  |  |
|  |  | .NET | LFGeneratorWaveform.Sine |
|  |  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SINE |
|  |  | COM | IviRFSigGenLFGeneratorWaveformSine |
| Square | Configures the LF generator to produce a square waveform |  |  |
|  |  | .NET | LFGeneratorWaveform.Square |
|  |  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SQUARE |
|  |  | COM | IviRFSigGenLFGeneratorWaveformSquare |
| Triangle | Configures the LF generator to produce a triangle waveform |  |  |
|  |  | .NET | LFGeneratorWaveform.Triangle |
|  |  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_TRIANGLE |
|  |  | COM | IviRFSigGenLFGeneratorWaveformTriangle |
| Ramp Up | Configures the LF generator to produce a rising ramp waveform |  |  |
|  |  | .NET | LFGeneratorWaveform.RampUp |
|  |  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_RAMP_UP |
|  |  | COM | IviRFSigGenLFGeneratorWaveformRampUp |
| Ramp Down | Configures the LF generator to produce a falling ramp waveform |  |  |
|  |  | .NET | LFGeneratorWaveform.RampDown |
|  |  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_RAMP_DOWN |
|  |  | COM | IviRFSigGenLFGeneratorWaveformRampDown |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to LFGenerator Waveform Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of LFGenerator Specific Ext Base, IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_CLASS_EXT_BASE.

## IviRFSigGenLFGenerator Functions

The IviRFSigGenLFGenerator extension group defines the following function:

- Get LFGenerator Name (IVI-C & IVI.NET)

- Set Active LFGenerator (IVI-C only)

- Configure LFGenerator

This section describes the behavior and requirements of this function.

### Get LFGenerator Name (IVI-C & IVI.NET only)

Description

This function returns the specific driver defined LF generator source name that corresponds to the index that the user specifies.  If the driver defines a qualified LF generator name, this property returns the qualified name.  In C, the index is one-based.  In .NET, the index is zero-based.

.NET Method Prototype

String LFGenerator.GetName (Int32 index);

COM Method Prototype

N/A

(Use LFGenerator.Name property)

C Prototype

ViStatus IviRFSigGen_GetLFGeneratorName (ViSession Vi,
                                         ViInt32 Index,
                                         ViInt32 NameBufferSize,
                                         ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | An index (one-based for IVI-C, zero-based for IVI.NET) that defines which name to return. |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name (C) | A user-allocated buffer into which the driver stores the LFGenerator name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |
| Return Value (.NET) | The LFGenerator name corresponding to the user-specified index. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

Compliance Notes

For an instrument with only one LF generator source, that is the LFGenerator Count attribute is one, the driver may return an empty string in the Name parameter.

### Set Active LFGenerator (IVI-C Only)

Description

This function selects one of the available LF generator sources, and makes it the active LFGenerator.

.NET Method Prototype

N/A

(Use the Active LF Generator property.)

COM Method Prototype

N/A

(Use the Active LF Generator property.)

C Function Prototype

ViStatus IviRFSigGen_SetActiveLFGenerator (ViSession Vi,
                                           ViConstString ActiveLFGenerator);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| ActiveLFGenerator | LF generator source to be selected. The driver uses this value to set the Active LFGenerator attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:Inherent Capabilities Specification defines general status codes that this function can return.

### Configure LFGenerator

Description

Configures the LF generator output frequency and  waveform.

.NET Method Prototype

void LFGenerator.Configure (Double frequency,
                            LFGeneratorWaveform waveform);

COM Method Prototype

HRESULT LFGenerator.Configure ([in] DOUBLE Frequency,
                  [in] IviRFSigGenLFGeneratorWaveformEnum Waveform);

C Prototype

ViStatus IviRFSigGen_ConfigureLFGenerator (ViSession Vi, 
 ViReal64 Frequency,
ViInt32 Waveform);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Frequency | Specifies the frequency of the LF generator. The driver uses this value to set the LFGenerator Frequency attribute. See the attribute description for more details. |
| Waveform | Specifies the waveform of the LF generator. The driver uses this value to set the LFGenerator Waveform attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenLFGenerator Behavior Model

The IviRFSigGenLFGenerator Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenLFGenerator settings.

## IviRFSigGenLFGenerator Compliance Notes

- If a specific driver implements the IviRFSigGenLFGenerator Extension Group, it shall also implement the IviRFSigGenAnalogModulationSource Extension Group.

# IviRFSigGen LFGeneratorOutput Extension Group

## IviRFSigGenLFGeneratorOutput Extension Group Overview

The IviRFSigGenLFGenerator Output Extension Group supports the output of the LF generator, if it is used as a source for external devices. The user can set the amplitude  and enable or disable the output. This extension group requires the LFGenerator extension group.

## IviRFSigGenLFGeneratorOutput Attributes

- LFGenerator Output Amplitude

- LFGenerator Output Enabled

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### LFGenerator Output Amplitude

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure LFGenerator Output |

.NET Property Name

LFGenerator.Output.Amplitude

COM Property Name

LFGenerator.Output.Amplitude

C Constant Name

IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_AMPLITUDE

Description

Specifies the output voltage the of the LF generator. The units are Volts, peak-to-peak.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### LFGenerator Output Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure LF Output |

.NET Property Name

LFGenerator.Output.Enabled

COM Property Name

LFGenerator.Output.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_ENABLED

Description

If set to True, the LF generator applies a signal to the output.  If set to False, the LF generator does not apply a signal to the output.

Compliance Notes

- Instrument drivers shall support the value True.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenLFGeneratorOutput Functions

The IviRFSigGenLFGeneratorOutput extension group defines the following function:

- Configure LFGenerator Output

This section describes the behavior and requirements of this function.

### Configure LFGenerator Output

Description

Configures the attributes of the LF generator output (within the RF signal generator).

.NET Method Prototype

void LFGenerator.Output.Configure (Double amplitude,
                                   Boolean enabled );

COM Method Prototype

HRESULT LFGenerator.Output.Configure ([in] DOUBLE Amplitude,
                                      [in] VARIANT_BOOL Enabled );

C Prototype

ViStatus IviRFSigGen_ConfigureLFGeneratorOutput (ViSession Vi, 
ViReal64 Amplitude,
ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Amplitude | Specifies the output voltage of the LF generator output signal. The driver uses this value to set the LFGenerator Output Amplitude attribute. See the attribute description for more details. |
| Enabled | Specifies whether the LF generator applies a signal to the output. The driver uses this value to set the LFGenerator Output Enabled attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenLFGeneratorOutput Behavior Model

The IviRFSigGenLFGeneratorOutput Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenLFGeneratorOutput settings.

## IviRFSigGenLFGeneratorOutput Compliance Notes

- If a specific driver implements the IviRFSigGenLFGeneratorOutput Extension Group, it shall also implement the IviRFSigGenLFGenerator Extension Group.

# IviRFSigGenPulseGenerator Extension Group

## IviRFSigGenPulseGenerator Extension Group Overview

The IviRFSigGenPulseGenerator Extension Group supports the pulse generator within the signal generator that is normally used as a source for the pulse modulator. The user can set the pulse period, width and delay. The pulse can be triggered, so source and external trigger polarity can be set.

Double pulse can be set with the functions and attributes of the IviRFSigGenPulseDoubleGenerator extension group.

The output of the pulse generator can be set with the functions and attributes of the IviRFSigGenPulseGeneratorOutput extension group.

## IviRFSigGenPulseGenerator Attributes

The IviRFSigGenPulseGenerator extension group defines the following attributes:

- Pulse Internal Trigger Period

- Pulse Width

- Pulse Gating Enabled

- Pulse Trigger Source

- Pulse External Trigger Slope

- Pulse External Trigger Delay

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Pulse Internal Trigger Period

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) Ivi.Driver.PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Pulse Internal Trigger |

.NET Property Name

Ivi.Driver.PrecisionTimeSpan PulseGenerator.InternalTriggerPeriod

COM Property Name

PulseGenerator.InternalTriggerPeriod

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_INTERNAL_TRIGGER_PERIOD

Description

Specifies the period of the pulse generator output signal when Pulse Trigger Source is set to Internal. For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of Precision Time Span.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Pulse Width

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) Ivi.Driver.PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Pulse |

.NET Property Name

Ivi.Driver.PrecisionTimeSpan PulseGenerator.Width

COM Property Name

PulseGenerator.Width

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_WIDTH

Description

Specifies the width of the output pulse. For C and COM, the units are seconds. For .NET, the units are implicit in the definition of Precision Time Span.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Pulse Gating Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Pulse |

.NET Property Name

PulseGenerator.GatingEnabled

COM Property Name

PulseGenerator.GatingEnabled

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_GATING_ENABLED

Description

If set to True, enables the pulse trigger gating.  If set to False, disables the pulse trigger gating.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### Pulse Trigger Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure Pulse |
| ViString (.NET) | R/W | N/A | None | Configure Pulse |

.NET Property Name

PulseGenerator.TriggerSource

COM Property Name

PulseGenerator.TriggerSource

COM Enumeration Name

IviRFSigGenPulseTriggerSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_TRIGGER_SOURCE

Description

Specifies the source of the trigger signal the pulse generator uses to generate one pulse.

Defined Values

In IVI.NET the trigger source is a string.  If an IVI driver supports a trigger source and the trigger source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that trigger source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger source strings for trigger sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | No external trigger is used. The pulse period is specified with Pulse Internal Trigger Period. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenPulseTriggerSourceInternal |
| External | The pulse is started with a trigger after the delay time specified with Pulse External Trigger Delay. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenPulseTriggerSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Pulse Trigger Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Pulse Trigger Source Specific Ext Base, IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_CLASS_EXT_BASE.

### Pulse External Trigger Slope

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure Pulse External Trigger |

.NET Property Name

PulseGenerator.ExternalTriggerSlope

.NET Enumeration Name

Slope

COM Property Name

PulseGenerator.ExternalTriggerSlope

COM Enumeration Name

IviRFSigGenPulseExternalTriggerSlopeEnum

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_SLOPE

Description

Specifies whether the event occurs on the rising or falling edge of the trigger input signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Positive | Enables rising edge triggering. |  |  |
|  |  | .NET | Slope.Positive |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_POSITIVE |
|  |  | COM | IviRFSigGenPulseExternalTriggerSlopePositive |
| Negative | Enables falling edge triggering. |  |  |
|  |  | .NET | Slope.Negative |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_NEGATIVE |
|  |  | COM | IviRFSigGenPulseExternalTriggerSlopeNegative |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Pulse External Trigger Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Pulse External Trigger Specific Ext Base, IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE.

### Pulse External Trigger Delay

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Pulse External Trigger |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Pulse External Trigger |

.NET Property Name

PulseGenerator.ExternalTriggerDelay

COM Property Name

PulseGenerator.ExternalTriggerDelay

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_DELAY

Description

Specifies the delay for starting the output pulse with respect to the trigger input.

For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenPulseGenerator Functions

The IviRFSigGenPulseGenerator extension group defines the following functions:

- Configure Pulse External Trigger

- Configure Pulse Internal Trigger (IVI-C only)

- Configure Pulse

This section describes the behavior and requirements of each function.

### Configure Pulse External Trigger

Description

Configures the triggering of the pulse generator within the RF signal generator. Specifies the external trigger slope and the delay time for starting the pulse after the trigger pulse.

.NET Method Prototype

void PulseGenerator.ConfigureExternalTrigger (

Slope slope,

Ivi.Driver.PrecisionTimeSpan delay);

COM Method Prototype

HRESULT PulseGenerator.ConfigureExternalTrigger (
                    [in] IviRFSigGenPulseExternalTriggerSlopeEnum Slope,
                    [in] DOUBLE Delay);

C Prototype

ViStatus IviRFSigGen_ConfigurePulseExternalTrigger (ViSession Vi,
 ViInt32 Slope,
 ViReal64 Delay);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Slope | Specifies the polarity of the trigger input to start one pulse. The driver uses this value to set the Pulse External Trigger Slope attribute. See the attribute description for more details. |
| Delay | Specifies the delay for starting the output pulse with respect to the trigger input. The driver uses this value to set the Pulse External Trigger Delay attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure Pulse Internal Trigger (IVI-C only)

Description

Configures the pulse generator within the RF signal generator. Specifies the period time (repetition rate) in case of internal trigger (free run) mode.

.NET Method Prototype

N/A

(use the PulseGenerator.InternalTriggerPeriod property)

COM Method Prototype

N/A

(use the PulseGenerator.InternalTriggerPeriod property)

C Prototype

ViStatus IviRFSigGen_ConfigurePulseInternalTrigger (ViSession Vi,
ViReal64 Period);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Period | Specifies the period (repetition rate) for the pulse. The driver uses this value to set the Pulse Internal Trigger Period attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Pulse

Description

Configures the trigger source, pulse width and gating enabled for the pulse generator.

.NET Method Prototype

void PulseGenerator.Configure (String pulseTriggerSource,
                               Ivi.Driver.PrecisionTimeSpan pulseWidth,
                               Boolean gatingEnabled);

COM Method Prototype

HRESULT PulseGenerator.Configure (
                    [in] IviRFSigGenPulseTriggerSourceEnum PulseTriggerSource,
                    [in] DOUBLE PulseWidth,
                    [in] VARIANT_BOOL GatingEnabled);

C Prototype

ViStatus IviRFSigGen_ConfigurePulse (ViSession Vi, ViInt32 PulseTriggerSource,
ViReal64 PulseWidth, ViBoolean GatingEnabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| PulseTriggerSource | Specifies the source of the signal the pulse generator uses to generate one pulse. The driver uses this value to set the Pulse Trigger Source attribute. See the attribute description for more details. |
| PulseWidth | Specifies the width of the output pulse. The driver uses this value to set the Pulse Width attribute. See the attribute description for more details. |
| GatingEnabled | Enables or disables gating. The driver uses this value to set the Pulse Gating Enabled attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenPulseGenerator Behavior Model

The IviRFSigGenPulseGenerator Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenPulseGenerator settings.

## IviRFSigGenPulseGenerator Compliance Notes

- If an instrument driver implements the IviRFSigGenPulseGenerator Extension Group, it shall also implement the IviRFSigGenModulatePulse Extension Group.

- If an instrument driver does not implement the value Pulse Trigger Source External for the Pulse Trigger Source attribute, it shall not implement the Pulse External Trigger Delay attribute, the Pulse External Trigger Slope attribute and the Configure Pulse External Trigger function.

- If an instrument driver does not implement the value Pulse Trigger Source Internal for the Pulse Trigger Source attribute, it shall not implement the Pulse Internal Trigger Period attribute and the Configure Internal Trigger function.

# IviRFSigGenPulseDoubleGenerator Extension Group

## IviRFSigGenPulseDoubleGenerator Extension Group Overview

The IviRFSigGenPulseDoubleGenerator Extension Group extends the IviRFSigGenPulseGenerator Extension Group to support double pulse generation.

## IviRFSigGenPulseDoubleGenerator Attributes

The IviRFSigGenPulseDoubleGenerator extension group defines the following attributes:

- Pulse Double Enabled

- Pulse Double Delay

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Pulse Double Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Pulse Double |

.NET Property Name

PulseGenerator.DoublePulse.Enabled

COM Property Name

PulseGenerator.Double.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_DOUBLE_ENABLED

Description

If set to True, the double pulse mode is enabled.  If set to False, the double pulse mode is disabled.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

Instrument drivers shall support the values True and False.

### Pulse Double Delay

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Pulse Double |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Pulse Double |

.NET Property Name

PulseGenerator.DoublePulse.Delay

COM Property Name

PulseGenerator.Double.Delay

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_DOUBLE_DELAY

Description

Specifies the delay of the second pulse from the falling edge of the first pulse. The second pulse has the same width as the first. For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenPulseDoubleGenerator Functions

The IviRFSigGenPulseGenerator extension group defines the following function:

- Configure Pulse Double

This section describes the behavior and requirements of this function.

### Configure Pulse Double

Description

Configures the pulse generator within the RF signal generator. Specifies double pulse state and delay.

.NET Method Prototype

void PulseGenerator.DoublePulse.Configure (Boolean enabled,
                                      Ivi.Driver.PrecisionTimeSpan delay);

COM Method Prototype

HRESULT PulseGenerator.Double.Configure ([in] VARIANT_BOOL Enabled,
                                         [in] DOUBLE Delay);

C Prototype

ViStatus IviRFSigGen_ConfigurePulseDouble (ViSession Vi, ViBoolean Enabled,
ViReal64 Delay);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | Enables or disables double pulse mode. The driver uses this value to set the Pulse Double Enabled attribute. See the attribute description for more details. |
| Delay | Specifies the delay of the second pulse. The driver uses this value to set the Pulse Double Delay attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenPulseDoubleGenerator Behavior Model

The IviRFSigGenPulseDoubleGenerator Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenPulseDoubleGenerator settings.

## IviRFSigGenPulseDoubleGenerator Compliance Notes

- If a specific driver implements the IviRFSigGenPulseDoubleGenerator Extension Group, it shall also implement the IviRFSigGenPulseGenerator Extension Group.

# IviRFSigGenPulseGeneratorOutput Extension Group

## IviRFSigGenPulseGeneratorOutput Extension Group Overview

The IviRFSigGenPulseGeneratorOutput Extension Group requires the IviRFSigGenPulseGenerator extension group. The output of the pulse generator can be enabled or disabled and its polarity set.

## IviRFSigGenPulseGeneratorOutput Attributes

The IviRFSigGenPulseGeneratorOutput extension group defines the following attributes:

- Pulse Output Polarity

- Pulse Output Enabled

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Pulse Output Polarity

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Pulse Output |

.NET Property Name

PulseGenerator.Output.Polarity

.NET Enumeration Name

PulseOutputPolarity

COM Property Name

PulseGenerator.Output.Polarity

COM Enumeration Name

IviRFSigGenPulseOutputPolarityEnum

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_OUTPUT_POLARITY

Description

Specifies the polarity of the output signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Normal | Specifies normal polarity. |  |  |
|  |  | .NET | PulseOutputPolarity.Normal |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_NORMAL |
|  |  | COM | IviRFSigGenPulseOutputPolarityNormal |
| Inverse | Specifies inverted polarity. |  |  |
|  |  | .NET | PulseOutputPolarity.Inverse |
|  |  | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_INVERSE |
|  |  | COM | IviRFSigGenPulseOutputPolarityInverse |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Pulse Output Polarity Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Pulse Output Polarity Specific Ext Base, IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_CLASS_EXT_BASE.

### Pulse Output Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Pulse Output |

.NET Property Name

PulseGenerator.Output.Enabled

COM Property Name

PulseGenerator.Output.Enabled

COM Enumeration Name

N/A

C Constant Name

IVIRFSIGGEN_ATTR_PULSE_OUTPUT_ENABLED

Description

If set to True, the pulse generator’s external output is enabled.  If set to False, the pulse generator’s external output is disabled.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

Instrument drivers shall support the values True and False.

## IviRFSigGenPulseGeneratorOutput Functions

The IviRFSigGenPulseGenerator extension group defines the following functions:

- Configure Pulse Output

This section describes the behavior and requirements of this function.

### Configure Pulse Output

Description

Configures the output and polarity of the pulse generator within the RF signal generator.

.NET Method Prototype

void PulseGenerator.Output.Configure(PulseOutputPolarity polarity,
                                     Boolean enabled);

COM Method Prototype

HRESULT PulseGenerator.Output.Configure(
                           [in] IviRFSigGenPulseOutputPolarityEnum Polarity,
                           [in] VARIANT_BOOL Enabled);

C Prototype

ViStatus IviRFSigGen_ConfigurePulseOutput (ViSession Vi, ViInt32 Polarity,
ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Polarity | Specifies the polarity of the output signal. The driver uses this value to set the Pulse Output Polarity attribute. See the attribute description for more details. |
| Enabled | Enables or disables the external output of the pulse generator. The driver uses this value to set the Pulse Output Enabled attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenPulseGeneratorOutput Behavior Mode

The IviRFSigGenPulseGeneratorOutput Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenPulseGeneratorOutput settings.

## IviRFSigGenPulseGeneratorOutput Compliance Notes

- If a specific driver implements the IviRFSigGenPulseGeneratorOutput Extension Group, it shall also implement the IviRFSigGenPulseGenerator Extension Group.

# IviRFSigGenSweep Extension Group

## IviRFSigGenSweep Overview

The IviRFSigGenSweep extension group supports signal generators with the ability to sweep (or step) the frequency or the power of the RF output signal.

## IviRFSigGenSweep Attributes

The IviRFSigGenSweep extension group defines the following attributes:

- Sweep Mode

- Sweep Trigger Source

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Sweep Mode

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Sweep |

.NET Property Name

Sweep.Mode

.NET Enumeration Name

SweepMode

COM Property Name

Sweep.Mode

COM Enumeration Name

IviRFSigGenSweepModeEnum

C Constant Name

IVIRFSIGGEN_ATTR_SWEEP_MODE

Description

Specifies the sweep mode applied to the output signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| None | The RF output of the signal generator is a non-swept signal (Continuous Wave). Frequency and power level settings from the base capability group are used. |  |  |
|  |  | .NET | SweepMode.None |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_NONE |
|  |  | COM | IviRFSigGenSweepModeNone |
| Frequency Sweep | The signal generator sweeps the RF output signal’s frequency in an analog form (non-stepped). Refer to IviRFSigGenFrequencySweep extension group. |  |  |
|  |  | .NET | SweepMode.FrequencySweep |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_FREQUENCY_SWEEP |
|  |  | COM | IviRFSigGenSweepModeFrequencySweep |
| Power Sweep | The signal generator sweeps the RF output signal’s power in an analog form (non-stepped). Refer to IviRFSigGenPowerSweep extension group. |  |  |
|  |  | .NET | SweepMode.PowerSweep |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_POWER_SWEEP |
|  |  | COM | IviRFSigGenSweepModePowerSweep |
| Frequency Step | The signal generator sweeps the RF output signal’s frequency in steps. Refer to IviRFSigGenFrequencyStep extension group. |  |  |
|  |  | .NET | SweepMode.FrequencyStep |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_FREQUENCY_STEP |
|  |  | COM | IviRFSigGenSweepModeFrequencyStep |
| Power Step | The signal generator sweeps the RF output signal’s power level in steps. Refer to IviRFSigGenPowerStep extension group. |  |  |
|  |  | .NET | SweepMode.PowerStep |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_POWER_STEP |
|  |  | COM | IviRFSigGenSweepModePowerStep |
| List | The signal generator uses two lists with frequency and power level values to sweep the RF output signal. Refer to IviRFSigGenList extension group. |  |  |
|  |  | .NET | SweepMode.List |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_LIST |
|  |  | COM | IviRFSigGenSweepModeList |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- A specific driver shall implement the defined value None and at least one of the other defined values.

- If a specific driver implements the defined value Frequency Sweep it shall implement the IviRFSigGenFrequencySweep Extension group.

- If a specific driver implements the defined value Power Sweep it shall implement the IviRFSigGenPowerSweep Extension group.

- If a specific driver implements the defined value Frequency Step it shall implement the IviRFSigGenFrequencyStep Extension group.

- If a specific driver implements the defined value Power Step it shall implement the IviRFSigGenPowerStep extension group.

- If a specific driver implements the defined value List it shall implement the IviRFSigGenList Extension group.

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_SWEEP_MODE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_SWEEP_MODE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_SWEEP_MODE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Sweep Mode Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of AM Sweep Mode Specific Ext Base, IVIRFSIGGEN_VAL_SWEEP_MODE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_SWEEP_MODE_CLASS_EXT_BASE.

### Sweep Trigger Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure Sweep |
| ViString (.NET) | R/W | N/A | None | Configure Sweep |

.NET Property Name

Sweep.TriggerSource

COM Property Name

Sweep.TriggerSource

COM Enumeration Name

IviRFSigGenSweepTriggerSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_SWEEP_TRIGGER_SOURCE

Description

Specifies the trigger used to start a sweep operation.

Defined Values

In IVI.NET the trigger source is a string.  If an IVI driver supports a trigger source and the trigger source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that trigger source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger source strings for trigger inputs that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Immediate | The sweep system does not wait for a trigger of any kind, so it is running continuously. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_IMMEDIATE |
|  |  | COM | IviRFSigGenSweepTriggerSourceImmediate |
| External | The sweep is started with an external signal. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenSweepTriggerSourceExternal |
| Software Trigger | The sweep is started with a software programmable trigger. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SOFTWARE |
|  |  | COM | IviRFSigGenSweepTriggerSourceSoftware |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If a specific driver implements the value Software Trigger, it shall also implement the IviRFSigGenSoftwareTrigger Extension Group.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Sweep Trigger Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Sweep Trigger Source Specific Ext Base, IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_CLASS_EXT_BASE.

## IviRFSigGenSweep Functions

The IviRFSigGenSweep extension group defines the following functions:

- Configure Sweep

This section describes the behavior and requirements of each function.

### Configure Sweep

Description

Configures the signal generator whether the RF output signal is a continuous wave or the frequency, the power level or both are swept or stepped.

.NET Method Prototype

void Sweep.Configure (SweepMode mode,
                      String triggerSource);

COM Method Prototype

HRESULT Sweep.Configure([in] IviRFSigGenSweepModeEnum Mode,
                        [in] IviRFSigGenSweepTriggerSourceEnum TriggerSource);

C Prototype

ViStatus IviRFSigGen_ConfigureSweep (ViSession Vi, ViInt32 Mode,
ViInt32 TriggerSource);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| Mode | Specifies the sweep mode of the RF signal generator. The driver uses this value to set the Sweep Mode attribute. See the attribute description for more details. |
| TriggerSource | Specifies the way to start the sweep or running it continuously. The driver uses this value to set the Sweep Trigger Source attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenSweep Behavior Model

The following behavior models show the relationship between the IviRFSigGenSweep extension group and RF Signal Generator behavior.

### Frequency Sweep / Power Sweep

Setting the Sweep Trigger attribute to Immediate will continuously generate sweeps (either frequency or power sweeps). The duration of one sweep from start to stop is defined with Frequency Sweep Time or Power Sweep Time. Setting the Sweep Trigger attribute to External or Software Trigger will delay the start of the next sweep until the specified trigger event occurs.

Trigger Immediate:					Trigger External / Software:

### Frequency Step / Power Step / List

Setting the Sweep Trigger attribute to Immediate will continuously generate steps (frequency or power) either from start to stop with fixed increments (steps) or from a list of arbitrary values. The duration of one step is defined with Dwell Time. Setting the Sweep Trigger attribute to External or Software Trigger will delay the start of the next step until the specified trigger event occurs.

Trigger Immediate:				Trigger External / Software:

Single Step and Trigger External / Software:

In addition the following rules apply:

- When a call to Configure RF of the base capability group is made, the Sweep Mode attribute is set to None, hence stopping any sweeps that may have been in progress.

- When the Sweep Mode attribute is set to Frequency Sweep, Frequency Step or List Mode with frequency or frequency and power list selected, setting Frequency of the base capability group will set the Sweep Mode attribute to None. Setting the value of Power Level will control the power level of the swept signal.

- When the Sweep Mode attribute is set to Power Sweep, Power Step or List Mode with power or frequency and power level list selected, setting Power Level of the base capability group will set the Sweep Mode attribute to None. Setting the value of  Frequency will control the frequency of the swept signal.

- When the Sweep Mode attribute is set to List and a frequency and power level list is selected, setting Frequency or Power Level of the base capability group will set the Sweep Mode attribute to None.

- When the Sweep Mode attribute is changed from any sweep mode to None, the signal generator will generate the signal accordingly to the previously set values of Frequency and Power Level of the base capability group.

# IviRFSigGenFrequencySweep Extension Group

## IviRFSigGenFrequencySweep Extension Group Overview

The IviRFSigGenFrequencySweep Extension Group supports signal generators that can apply a frequency sweep to the output signal.  The user may configure the sweep with start and stop, or center and span frequencies. The sweep time is also configurable.

In order to support this extension group, a driver shall first support the IviRFSigGenSweep Extension Group. This extension group is active when the Sweep Mode attribute is set to Frequency Sweep.

## IviRFSigGenFrequencySweep Attributes

The IviRFSigGenFrequencySweep extension group defines the following attributes:

- Frequency Sweep Start

- Frequency Sweep Stop

- Frequency Sweep Time

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Frequency Sweep Start

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency Sweep Start Stop, Configure Frequency Sweep Center Span |

.NET Property Name

Sweep.FrequencySweep.Start

COM Property Name

Sweep.FrequencySweep.Start

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_START

Description

Specifies the start frequency of the sweep. If the stop frequency is less than the start frequency, the frequency decreases during the sweep. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Sweep Stop

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency Sweep Start Stop Configure Frequency Sweep Center Span |

.NET Property Name

Sweep.FrequencySweep.Stop

COM Property Name

Sweep.FrequencySweep.Stop

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_STOP

Description

Specifies the stop frequency of the sweep. If the stop frequency is less than the start frequency, the frequency decreases during the sweep. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Sweep Time

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Frequency Sweep Time |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Frequency Sweep Time |

.NET Property Name

Sweep.FrequencySweep.Time

COM Property Name

Sweep.FrequencySweep.Time

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_TIME

Description

Specifies the duration of one sweep from start to stop frequency. For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenFrequencySweep Functions

The IviRFSigGenFrequencySweep extension group defines the following functions:

- Configure Frequency Sweep Start Stop

- Configure Frequency Sweep Center Span

- Configure Frequency Sweep Time (IVI-C only)

This section describes the behavior and requirements of each function.

### Configure Frequency Sweep Start Stop

Description

Configures the attributes that control the sweep frequencies of the generator's output signal. These attributes are start and stop frequency. If the stop frequency is less than the start frequency, the frequency decreases during the sweep.

.NET Method Prototype

void Sweep.FrequencySweep.ConfigureStartStop (Double start,
                                              Double stop);

COM Method Prototype

HRESULT Sweep.FrequencySweep.ConfigureStartStop ([in] DOUBLE Start,
                                                 [in] DOUBLE Stop);

C Prototype

ViStatus IviRFSigGen_ConfigureFrequencySweepStartStop (ViSession Vi,
ViReal64 Start,
ViReal64 Stop);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Start | Specifies the start frequency of the sweep. The driver uses this value to set the Frequency Sweep Start attribute. See the attribute description for more details. |
| Stop | Specifies the stop frequency of the sweep. The driver uses this value to set the Frequency Sweep Stop attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure Frequency Sweep Center Span

Description

Configures the attributes that control the sweep frequencies of the generator's output signal. These attributes are center and span frequency. This function modifies the attributes as follows:

Frequency Sweep Start = Center – Span / 2

Frequency Sweep Stop = Center + Span / 2

.NET Method Prototype

void Sweep.FrequencySweep.ConfigureCenterSpan (Double center,
                                               Double span);

COM Method Prototype

HRESULT Sweep.FrequencySweep.ConfigureCenterSpan ([in] DOUBLE Center,
                                                  [in] DOUBLE Span);

C Prototype

ViStatus IviRFSigGen_ConfigureFrequencySweepCenterSpan (ViSession Vi,
ViReal64 Center,
ViReal64 Span);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Center | Specifies the center frequency of the sweep. The driver uses this value to set the Frequency Sweep Start and Frequency Sweep Stop attribute. See the attribute description and function description for more details. |
| Span | Specifies the frequency span of the sweep. The driver uses this value to set therequency Sweep Start and Frequency Sweep Stop attribute. See the attribute description and function description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure Frequency Sweep Time (IVI-C only)

Description

Configures the duration of one frequency sweep.

.NET Method Prototype

N/A

(use the Sweep.FrequencySweep.Time property)

COM Method Prototype

N/A

(use the Sweep.FrequencySweep.Time property)

C Prototype

ViStatus IviRFSigGen_ConfigureFrequencySweepTime (ViSession Vi, ViReal64 Time);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Time | Specifies the duration of the frequency sweep. The driver uses this value to set the Frequency Sweep Time attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenFrequencySweep Behavior Model

The IviRFSigGenFrequencySweep Extension Group follows the behavior model of the IviRFSigGenSweep capability group.

## IviRFSigGenFrequencySweep Compliance Notes

- If a specific driver implements the IviRFSigGenFrequencySweep Extension Group, it shall also implement the IviRFSigGenSweep Extension Group and support the defined value Frequency Sweep for the Sweep Mode attribute.

# IviRFSigGenPowerSweep Extension Group

## IviRFSigGenPowerSweep Extension Group Overview

The IviRFSigGenPowerSweep Extension Group supports signal generators that can apply a power sweep to the output signal.  The user may configure the sweep with start and stop power. The sweep time is also configurable.

In order to support this extension group, a driver shall first support the IviRFSigGenSweep Extension Group. This extension group is active when the Sweep Mode attribute is set to Power Sweep.

## IviRFSigGenPowerSweep Attributes

The IviRFSigGenPowerSweep extension group defines the following attributes:

- Power Sweep Start

- Power Sweep Stop

- Power Sweep Time

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Power Sweep Start

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Power Sweep Start Stop |

.NET Property Name

Sweep.PowerSweep.Start

COM Property Name

Sweep.PowerSweep.Start

C Constant Name

IVIRFSIGGEN_ATTR_POWER_SWEEP_START

Description

Specifies the start power of the sweep. If the stop power is less than the start power, the power decreases in value during the sweep. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Sweep Stop

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Power Sweep Start Stop |

.NET Property Name

Sweep.PowerSweep.Stop

COM Property Name

Sweep.PowerSweep.Stop

C Constant Name

IVIRFSIGGEN_ATTR_POWER_SWEEP_STOP

Description

Specifies the stop power of the sweep. If the stop power is less than the start power, the power decreases in value during the sweep. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Sweep Time

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Power Sweep Time |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Power Sweep Time |

.NET Property Name

Sweep.PowerSweep.Time

COM Property Name

Sweep.PowerSweep.Time

COM Enumeration Name

N/A

C Constant Name

IVIRFSIGGEN_ATTR_POWER_SWEEP_TIME

Description

Specifies the duration of one sweep from start to stop power. For C and COM the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenPowerSweep Functions

The IviRFSigGenPowerSweep extension group defines the following functions:

- Configure Power Sweep Start Stop

- Configure Power Sweep Time (IVI-C only)

This section describes the behavior and requirements of each function.

### Configure Power Sweep Start Stop

Description

Configures the attributes that control the power sweep of the generator's output signal. These attributes are start and stop power. If the stop power is less than the start power, the power decreases in value during the sweep.

.NET Method Prototype

void Sweep.PowerSweep.ConfigureStartStop (Double start,
                                          Double stop);

COM Method Prototype

HRESULT Sweep.PowerSweep.ConfigureStartStop ([in] DOUBLE Start,
                                             [in] DOUBLE Stop);

C Prototype

ViStatus IviRFSigGen_ConfigurePowerSweepStartStop (ViSession Vi,
ViReal64 Start,
ViReal64 Stop);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Start | Specifies the start power of the sweep. The driver uses this value to set the Power Sweep Start attribute. See the attribute description for more details. |
| Stop | Specifies the stop power of the sweep. The driver uses this value to set the Power Sweep Stop attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### onfigure Power Sweep Time (IVI-C only)

Description

Configures the duration of one power sweep.

.NET Method Prototype

N/A

(use the Sweep.PowerSweep.Time property)

COM Method Prototype

N/A

(use the Sweep.PowerSweep.Time property)

C Prototype

ViStatus IviRFSigGen_ConfigurePowerSweepTime (ViSession Vi, ViReal64 Time);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Time | Specifies the duration of the power sweep. The driver uses this value to set the Power Sweep Time attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenPowerSweep Behavior Model

The IviRFSigGenPowerSweep Extension Group follows the behavior model of the IviRFSigGenSweep capability group.

## IviRFSigGenPowerSweep Compliance Notes

- If a specific driver implements the IviRFSigGenPowerSweep Extension Group, it shall also implement the IviRFSigGenSweep Extension Group and support the defined value Power Sweep for the Sweep Mode attribute.

# IviRFSigGenFrequencyStep Extension Group

## IviRFSigGenFrequencyStep Extension Group Overview

The IviRFSigGenFrequencyStep Extension Group supports signal generators that can vary (sweep) the frequency of the RF output signal in steps. The user can specify the start, stop and step frequency and set linear or logarithmic spacing. Setting single step and dwell time are also included.
This extension group requires the Sweep Extension Group. Frequency stepping is enabled by setting the Sweep Mode to Frequency Step in the IviRFSigGenSweep Extension Group.

## IviRFSigGenFrequencyStep Attributes

The IviRFSigGenFrequencyStep extension group defines the following attributes:

- Frequency Step Start

- Frequency Step Stop

- Frequency Step Scaling

- Frequency Step Size

- Frequency Step Single Step Enabled

- Frequency Step Dwell

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Frequency Step Start

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency Step Start Stop |

.NET Property Name

Sweep.FrequencyStep.Start

COM Property Name

Sweep.FrequencyStep.Start

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_START

Description

Specifies the start frequency of the stepped sweep. If the stop frequency is less than the start frequency, the frequency decreases during the sweep. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Step Stop

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency Step Start Stop |

.NET Property Name

Sweep.FrequencyStep.Stop

COM Property Name

Sweep.FrequencyStep.Stop

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_STOP

Description

Specifies the stop frequency of the stepped sweep. If the stop frequency is less than the start frequency, the frequency decreases during the sweep. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Step Scaling

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Frequency Step Start Stop |

.NET Property Name

Sweep.FrequencyStep.Scaling

COM Enumeration Name

FrequencyStepScaling

COM Property Name

Sweep.FrequencyStep.Scaling

COM Enumeration Name

IviRFSigGenFrequencyStepScalingEnum

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SCALING

Description

Specifies the spacing of the steps.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Linear | Enables linear scaling. |  |  |
|  |  | .NET | FrequencyStepScaling.Linear |
|  |  | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_LINEAR |
|  |  | COM | IviRFSigGenFrequencyStepScalingLinear |
| Logarithmic | Enables logarithmic scaling. |  |  |
|  |  | .NET | FrequencyStepScaling.Logarithmic |
|  |  | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_LOGARITHMIC |
|  |  | COM | IviRFSigGenFrequencyStepScalingLogarithmic |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Frequency Step Scaling Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Frequency Step Scaling Specific Ext Base, IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_CLASS_EXT_BASE.

### Frequency Step Size

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency Step Start Stop |

.NET Property Name

Sweep.FrequencyStep.Size

COM Property Name

Sweep.FrequencyStep.Size

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SIZE

Description

Specifies the step size. The units are Hertz if Frequency Step Scaling attribute is set to Linear and is unitless (factor) if Frequency Step Scaling attribute is set to Logarithmic.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Step Single Step Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Frequency Step Dwell |

.NET Property Name

Sweep.FrequencyStep.SingleStepEnabled

COM Property Name

Sweep.FrequencyStep.SingleStepEnabled

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SINGLE_STEP_ENABLED

Description

If set to True, Frequency Step Single Step is enabled, and the frequency sweep will advance when the next trigger event occurs.  If set to False, Frequency Step Single Step is disabled, and the frequency sweep will advance immediatly after the dwell time ends.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Frequency Step Dwell

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Frequency Step Dwell |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Frequency Step Dwell |

.NET Property Name

Sweep.FrequencyStep.Dwell

COM Property Name

Sweep.FrequencyStep.Dwell

C Constant Name

IVIRFSIGGEN_ATTR_FREQUENCY_STEP_DWELL

Description

Specifies the duration of one step. Dwell time starts immediately after trigger or next step; no settling time is added. This attribute is ignored if the Frequency Step Single Step Enabled attribute is set to True.

For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenFrequencyStep Functions

The IviRFSigGenFrequencyStep extension group defines the following functions:

- Configure Frequency Step Start Stop

- Confiugure Frequency Step Dwell

- Reset Frequency Step

This section describes the behavior and requirements of each function.

### Configure Frequency Step Start Stop

Description

Configures the attributes that control the step frequencies of the generator's RF output signal. These attributes are start and stop frequency, step size and lin/log scaling. If the stop frequency is less than the start frequency, the frequency decreases during the sweep.

.NET Method Prototype

void Sweep.FrequencyStep.ConfigureStartStop (Double start,
                                             Double stop),
                                             FrequencyStepScaling scaling,
                                             Double stepSize);

COM Method Prototype

HRESULT Sweep.FrequencyStep.ConfigureStartStop ([in] DOUBLE Start,
                               [in] DOUBLE Stop),
                               [in] IviRFSigGenFrequencyStepScalingEnum Scaling,
                               [in] DOUBLE StepSize);

C Prototype

ViStatus IviRFSigGen_ConfigureFrequencyStepStartStop (ViSession Vi,
ViReal64 Start,
ViReal64 Stop,
ViInt32 Scaling,
ViReal64 StepSize);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Start | Specifies the start frequency of the step sweep. The driver uses this value to set the Frequency Step Start attribute. See the attribute description for more details. |
| Stop | Specifies the stop frequency of the step sweep. The driver uses this value to set the Frequency Step Stop attribute. See the attribute description for more details. |
| Scaling | Specifies the scaling of the step sweep. The driver uses this value to set the Frequency Step Scaling attribute. See the attribute description for more details. |
| StepSize | Specifies the size of one step. The driver uses this value to set the Frequency Step Size attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure Frequency Step Dwell

Description

Configures the attributes that control frequency stepping.

.NET Method Prototype

void Sweep.FrequencyStep.ConfigureDwell (Boolean singleStepEnabled
                                         PrecisionTimeSpan dwell);

COM Method Prototype

HRESULT Sweep.FrequencyStep.ConfigureDwell ([in] VARIANT_BOOL SingleStepEnabled
                                      [in] DOUBLE Dwell);

C Prototype

ViStatus IviRFSigGen_ConfigureFrequencyStepDwell (ViSession Vi,
ViBoolean SingleStepEnabled,
ViReal64 Dwell);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| SingleStepEnabled | Specifies whether the trigger initiates the next step. The driver uses this value to set the Frequency Step Single Step Enabled attribute. See the attribute description for more details. |
| Dwell | Specifies the duration of one frequency step. The driver uses this value to set the Frequency Step Dwell attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Reset Frequency Step

Description

Resets the current frequency step to the frequency step start value.

.NET Method Prototype

void Sweep.FrequencyStep.Reset ();

COM Method Prototype

HRESULT Sweep.FrequencyStep.Reset ();

C Prototype

ViStatus IviRFSigGen_ResetFrequencyStep (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenFrequencyStep Behavior Model

The IviRFSigGenFrequencyStep Extension Group follows the behavior model of the IviRFSigGenSweep capability group.

## IviRFSigGenFrequencyStep Compliance Notes

- If a specific driver implements the IviRFSigGenFrequencyStep Extension Group, it shall also implement the IviRFSigGenSweep Extension Group and support the defined value Frequency Step for the Sweep Mode attribute.

# IviRFSigGenPowerStep Extension Group

## IviRFSigGenPowerStep Extension Group Overview

The IviRFSigGenPowerStep Extension Group supports signal generators that can vary (sweep) the power of the RF output signal in steps. The user can enable or disable stepping, specify the start, stop and step power. Setting single step and dwell time are also included.
This extension group requires the Sweep Extension Group. Power stepping is enabled by setting the Sweep Mode to Power Step in the IviRFSigGenSweep Extension Group.

## IviRFSigGenPowerStep Attributes

The IviRFSigGenPowerStep extension group defines the following attributes:

- Power Step Start

- Power Step Stop

- Power Step Size

- Power Step Single Step Enabled

- Power Step Dwell

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Power Step Start

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Power Step Start Stop |

.NET Property Name

Sweep.PowerStep.Start

COM Property Name

Sweep.PowerStep.Start

C Constant Name

IVIRFSIGGEN_ATTR_POWER_STEP_START

Description

Specifies the start power of the stepped sweep. If the stop power is less than the start power, the power decreases in value during the sweep. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Step Stop

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure PowerStep Start Stop |

.NET Property Name

Sweep.PowerStep.Stop

COM Property Name

Sweep.PowerStep.Stop

C Constant Name

IVIRFSIGGEN_ATTR_POWER_STEP_STOP

Description

Specifies the stop power of the stepped sweep. If the stop power is less than the start power, the power decreases in value during the sweep. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Step Size

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Power Step Start Stop |

.NET Property Name

Sweep.PowerStep.Size

COM Property Name

Sweep.PowerStep.Size

C Constant Name

IVIRFSIGGEN_ATTR_POWER_STEP_SIZE

Description

Specifies the step size. The units are dBm.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Step Single Step Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure Power Step Dwell |

.NET Property Name

Sweep.PowerStep.SingleStepEnabled

COM Property Name

Sweep.PowerStep.SingleStepEnabled

C Constant Name

IVIRFSIGGEN_ATTR_POWER_STEP_SINGLE_STEP_ENABLED

Description

If set to True, the power sweep will advance when the next trigger event occurs.  If set to False, the power sweep will advance immediately after the dwell time ends.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Power Step Dwell

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure Power Step Dwell |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure Power Step Dwell |

.NET Property Name

Sweep.PowerStep.Dwell

COM Property Name

Sweep.PowerStep.Dwell

C Constant Name

IVIRFSIGGEN_ATTR_POWER_STEP_DWELL

Description

Specifies the duration of one step.  This attribute is ignored if Power Step Single Step Enabled is set to True.

For C and COM, the units are seconds.  For .NET, the units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenPowerStep Functions

The IviRFSigGenPowerStep extension group defines the following functions:

- Configure Power Step Start Stop

- Confiugure Power Step Dwell

- Reset Power Step

This section describes the behavior and requirements of each function.

### Configure Power Step Start Stop

Description

Configures the attributes that control the power steps of the generator's RF output signal. These attributes are start and stop power and step size. If the stop power is less than the start power, the power decreases in value during the sweep.

.NET Method Prototype

void Sweep.PowerStep.ConfigureStartStop (Double start,
                                         Double stop,
                                         Double stepSize);

COM Method Prototype

HRESULT Sweep.PowerStep.ConfigureStartStop ([in] DOUBLE Start,
                                      [in] DOUBLE Stop,
                                      [in] DOUBLE StepSize);

C Prototype

ViStatus IviRFSigGen_ConfigurePowerStepStartStop (ViSession Vi,
ViReal64 Start,
ViReal64 Stop,
ViReal64 StepSize);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Start | Specifies the start power of the stepping. The driver uses this value to set the PowerStep Start attribute. See the attribute description for more details. |
| Stop | Specifies the stop power of the stepping. The driver uses this value to set the Power Step Stop attribute. See the attribute description for more details. |
| StepSize | Specifies the size of one step. The driver uses this value to set the Power Step Size attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure Power Step Dwell

Description

Configures the attributes that control power stepping.

.NET Method Prototype

void Sweep.PowerStep.ConfigureDwell (Boolean singleStepEnabled,
                                     PrecisionTimeSpan dwell);

COM Method Prototype

HRESULT Sweep.PowerStep.ConfigureDwell ([in] VARIANT_BOOL SingleStepEnabled,
                                        [in] DOUBLE Dwell);

C Prototype

ViStatus IviRFSigGen_ConfigurePowerStepDwell (ViSession Vi,
ViBoolean SingleStepEnabled,
ViReal64 Dwell);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| SingleStepEnabled | Specifies whether the trigger initiates the next step. The driver uses this value to set the Power Step Single Step Enabled attribute. See the attribute description for more details. |
| Dwell | Specifies the duration of one power step. The driver uses this value to set the Power Step Dwell attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Reset Power Step

Description

Resets the current power step to the power step start value.

.NET Method Prototype

void Sweep.PowerStep.Reset ();

COM Method Prototype

HRESULT Sweep.PowerStep.Reset ();

C Prototype

ViStatus IviRFSigGen_ResetPowerStep (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenPowerStep Behavior Model

The IviRFSigGenPowerStep Extension Group follows the behavior model of the IviRFSigGenSweep capability group.

## IviRFSigGenPowerStep Compliance Notes

- If a specific driver implements the IviRFSigGenPowerStep Extension Group, it shall also implement the IviRFSigGenSweep Extension Group and support the defined value Power Step for the Sweep Mode attribute.

# IviRFSigGenList Extension Group

## IviRFSigGenList Extension Group Overview

The IviRFSigGenList Extension Group supports signal generators that can set the frequency and power of the RF output signal to values given as a list of values. The user can enable or disable stepping the frequency and power list, specify the name of the list and set its values. The active list can be selected using the list name. Setting single step and dwell time are also included.
This extension group requires the Sweep Extension Group. List stepping is enabled by setting the Sweep Mode to List in the IviRFSigGenSweep Extension Group.

## IviRFSigGenList Attributes

The IviRFSigGenList extension group defines the following attributes:

- List Selected Name

- List Single Step Enabled

- List Dwell

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### List Selected Name

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select List |

.NET Property Name

Sweep.List.SelectedList

COM Property Name

Sweep.List.SelectedName

C Constant Name

IVIRFSIGGEN_ATTR_LIST_SELECTED_NAME

Description

Specifies the name of the selected list to become active. The name shall be one of the lists created.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### List Single Step Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure List Dwell |

.NET Property Name

Sweep.List.SingleStepEnabled

COM Property Name

Sweep.List.SingleStepEnabled

C Constant Name

IVIRFSIGGEN_ATTR_LIST_SINGLE_STEP_ENABLED

Description

If  set to True, the list will advance when the next trigger event occurs.  If set to False, the list will advance immediatly after the dwell time ends.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### List Dwell

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 (C/COM) | R/W | N/A | None | Configure List Dwell |
| PrecisionTimeSpan (.NET) | R/W | N/A | None | Configure List Dwell |

.NET Property Name

Sweep.List.Dwell

COM Property Name

Sweep.List.Dwell

C Constant Name

IVIRFSIGGEN_ATTR_LIST_DWELL

Description

Specifies the duration of one step. This attribute is ignored if the List Single Step Enabled attribute is set to True.

For C and COM, the units are seconds.  For .NET, units are implicit in the definition of PrecisionTimeSpan.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenList Functions

The IviRFSigGenList extension group defines the following functions:

- Create Frequency List

- Create Power List

- Create Frequency Power List

- Select List (IVI-C only)

- Clear All Lists

- Configure List Dwell

- Reset List

This section describes the behavior and requirements of each function.

### Create Frequency List

Description

Creates a named list of frequency values.

.NET Method Prototype

void Sweep.List.CreateFrequency (String name,
                                 Double[] frequency);

COM Method Prototype

HRESULT Sweep.List.CreateFrequency ([in] BSTR Name,
                                    [in] SAFEARRAY(DOUBLE) *Frequency);

C Prototype

ViStatus IviRFSigGen_CreateFrequencyList (ViSession Vi,
ViConstString Name,
ViInt32 Length,
ViReal64 Frequency[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | Specifies the name of the list to be created. |
| Length | Specifies the number of values in the list array |
| Frequency[] | Specifies the array of frequency values to become elements of the list. The array must have at least as many elements as the value in the Length parameter. The units are Hertz. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Create Power List

Description

Creates a named list of power values.

.NET Method Prototype

void Sweep.List.CreatePower (String name,
                             Double[] power);

COM Method Prototype

HRESULT Sweep.List.CreatePower ([in] BSTR Name,
                                [in] SAFEARRAY(DOUBLE) *Power);

C Prototype

ViStatus IviRFSigGen_CreatePowerList (ViSession Vi,
ViConstString Name,
ViInt32 Length,
ViReal64 Power[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | Specifies the name of the list to be created. |
| Length | Specifies the number of values in the list array. |
| Power[] | Specifies the array of power values to become elements of the list. The array must have at least as many elements as the value in the Length parameter. The units are dBm. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Create Frequency Power List

Description

Creates a named list of both frequency and power values.

.NET Method Prototype

void Sweep.List.CreateFrequencyPower (String name,
                                      Double[] frequency,
                                      Double[] power);

COM Method Prototype

HRESULT Sweep.List.CreateFrequencyPower ([in] BSTR Name,
                                   [in] SAFEARRAY(DOUBLE) *Frequency,
                                   [in] SAFEARRAY(DOUBLE) *Power);

C Prototype

ViStatus IviRFSigGen_CreateFrequencyPowerList (ViSession Vi,
ViConstString Name,
ViInt32 Length,
ViReal64 Frequency[],
ViReal64 Power[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | Specifies the name of the list to be created. |
| Length | Specifies the number of values in the list array |
| Frequency[] | Specifies the array of frequency values to become elements of the list. The array must have at least as many elements as the value in the Length parameter. The units are Hertz. |
| Power[] | Specifies the array of power values to become elements of the list. The array must have at least as many elements as the value in the Length parameter. The units are dBm. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Select List (IVI-C only)

Description

Selects one list by name to become active (out of the pool of list).

.NET Method Prototype

N/A

(Use the List Selected Name property.)

COM Method Prototype

N/A

(Use the List Selected Name property.)

C Prototype

ViStatus IviRFSigGen_SelectList (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | Specifies the name of the list to make this list active. The driver uses this value to set the List Selected Name attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

| Completion Codes | Description |
| --- | --- |
| IVIRFSIGGEN_ERROR_LIST_UNKNOWN | The operation cannot be completed because the list is not defined. |

### Clear All Lists

Description

Deletes all lists from the pool of defined lists.

.NET Method Prototype

void Sweep.List.ClearAll ();

COM Method Prototype

HRESULT Sweep.List.ClearAll ();

C Prototype

ViStatus IviRFSigGen_ClearAllLists (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure List Dwell

Description

Configures the attributes that control list stepping.

.NET Method Prototype

void Sweep.List.ConfigureDwell (Boolean singleStepEnabled,
                                Ivi.Driver.PrecisionTimeSpan dwell);

COM Method Prototype

HRESULT Sweep.List.ConfigureDwell ([in] VARIANT_BOOL SingleStepEnabled,
                                   [in] DOUBLE Dwell);

C Prototype

ViStatus IviRFSigGen_ConfigureListDwell (ViSession Vi,
ViBoolean SingleStepEnabled,
ViReal64 Dwell);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| SingleStepEnabled | Specifies whether the trigger initiates transitions to the next list step. The driver uses this value to set the List Single Step Enabled attribute. See the attribute description for more details. |
| Dwell | Specifies the duration of one list step. The driver uses this value to set the List Dwell attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Reset List

Description

Resets the current list to the first entry value.

.NET Method Prototype

void Sweep.List.Reset ();

COM Method Prototype

HRESULT Sweep.List.Reset ();

C Prototype

ViStatus IviRFSigGen_ResetList (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenList Behavior Model

The IviRFSigGenList Extension Group follows the behavior model of the IviRFSigGenSweep capability group.

## IviRFSigGenList Compliance Notes

- If a specific driver implements the IviRFSigGenList Extension Group, it shall also implement the IviRFSigGenSweep Extension Group and support the defined value List for the Sweep Mode attribute.

# IviRFSigGenALC Extension Group

## IviRFSigGenALC Overview

For generators with configurable Automatic Level Control.

## IviRFSigGenALC Attributes

The IviRFSigGenALC extension group defines the following attributes:

- ALC Source

- ALC Bandwidth

The ALC Enabled attribute is defined in the IviRFSigGen Base Capabilities Group.

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### ALC Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure ALC |
| ViString (.NET) | R/W | N/A | None | Configure ALC |

.NET Property Name

Alc.Source

COM Property Name

ALC.Source

COM Enumeration Name

IviRFSigGenALCSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_ALC_SOURCE

Description

Specifies the source of the controlling voltage for the Automatic Level Control. The RF level at the sensor point is held constant.

Defined Values

In IVI.NET the ALC source is a string.  If an IVI driver supports an ALC source and the ALC source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that ALC source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new ALC source strings for ALC inputs that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The ALC is controlled by an internal measurement source. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_ALC_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenALCSourceInternal |
| External | The ALC is controlled by an external voltage. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_ALC_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenALCSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- An instrument driver shall support the defined value Internal.

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ALC_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_ALC_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ALC_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to ALC Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of ALC Source Specific Ext Base, IVIRFSIGGEN_VAL_ALC_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_ALC_SOURCE_CLASS_EXT_BASE.

### ALC Bandwidth

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | Up | Configure ALC |

.NET Property Name

Alc.Bandwidth

COM Property Name

ALC.Bandwidth

C Constant Name

IVIRFSIGGEN_ATTR_ALC_BANDWIDTH

Description

Specifies the bandwidth of the level control. Narrow bandwidth impoves noise and allows AM with modulation frequencies beyond bandwidth frequency. The value is coerced to reflect the ability of the current RF signal generator. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenALC Functions

The IviRFSigGenALC extension group defines the following functions:

- Configure ALC

This section describes the behavior and requirements of this function.

### Configure ALC

Description

Configures the ALC (Automatic Level Control) of the signal generator’s RF output.

.NET Method Prototype

void Alc.Configure (String source,
                    Double bandwidth);

COM Method Prototype

HRESULT ALC.Configure ([in] IviRFSigGenALCSourceEnum Source,
                       [in] DOUBLE Bandwidth);

C Prototype

ViStatus IviRFSigGen_ConfigureALC (ViSession Vi, ViInt32 Source,
ViReal64 Bandwidth);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| Source | The driver uses this value to set the ALC Source attribute. See the attribute description for more details. |
| Bandwidth | The driver uses this value to set the ALC Bandwidth attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenALC Behavior Model

The IviRFSigGenALC Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenALC settings.

# IviRFSigGen ReferenceOscillator Extension Group

## IviRFSigGenReferenceOscillator Overview

The IviRFSigGenReferenceOscillator extension group supports signal generators with a configurable frequency reference.

## IviRFSigGenReferenceOscillator Attributes

The IviRFSigGenReferenceOscillator extension group defines the following attributes:

- Reference Oscillator Source

- Reference Oscillator External Frequency

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Reference Oscillator Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure Reference Oscillator |
| ViString (.NET) | R/W | N/A | None | Configure Reference Oscillator |

.NET Property Name

ReferenceOscillator.Source

COM Property Name

ReferenceOscillator.Source

COM Enumeration Name

IviRFSigGenReferenceOscillatorSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_SOURCE

Description

Specifies the reference frequency source used to generate the exact RF output frequency.

Defined Values

In IVI.NET the reference oscillator source is a string.  If an IVI driver supports a reference oscillator source and the reference oscillator source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that reference oscillator source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new reference oscillator source strings for reference oscillator sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The internal reference oscillator is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenReferenceOScillatorSourceInternal |
| External | An external reference oscillator is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenReferenceOscillatorSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Reference Oscillator Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Reference Oscillator Source Specific Ext Base, IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_CLASS_EXT_BASE.

### Reference Oscillator External Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Reference Oscillator |

.NET Property Name

ReferenceOscillator.ExternalFrequency

COM Property Name

ReferenceOscillator.ExternalFrequency

C Constant Name

IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_EXTERNAL_FREQUENCY

Description

Specifies the frequency of the external signal, which is used as reference for internal RF frequency generation. This value is used only if Reference Oscillator Source is set to External. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenReferenceOscillator Functions

The IviRFSigGenReferenceOscillator extension group defines the following functions:

- Configure Reference Oscillator

This section describes the behavior and requirements of this function.

### Configure Reference Oscillator

Description

Configures the signal generator’s reference oscillator.

.NET Method Prototype

void ReferenceOscillator.Configure (String source,
                                    Double frequency);

COM Method Prototype

HRESULT ReferenceOscillator.Configure ([in] IviRFSigGenReferenceOscillatorSourceEnum Source,
                                       [in] DOUBLE Frequency);

C Prototype

ViStatus IviRFSigGen_ConfigureReferenceOscillator (ViSession Vi,
ViInt32 Source,
ViReal64 Frequency);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |
| Source | Specifies source of reference frequency signal. The driver uses this value to set the Reference Oscillator Source attribute. See the attribute description for more details. |
| Frequency | Specifies the frequency of the external reference oscillator. This parameter is only used if the Source is set to External. The driver uses this value to set the Reference Oscillator Frequency attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenReferenceOscillator Behavior Model

The IviRFSigGenReferenceOscillator Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenReferenceOscillator settings.

# IviRFSigGenSoftwareTrigger Extension Group

## IviRFSigGenSoftwareTrigger Overview

The IviRFSigGenSoftwareTrigger Extension Group supports RF signal generators that can generate output based on a software trigger signal. The user can send a software trigger to cause signal output to occur, to start a sweep (frequency or power), or to initiate the next step (frequency and/or power).

This extension affects instrument behavior when the Trigger Source attribute is set to Software Trigger.

## IviRFSigGenSoftwareTrigger Functions

The IviRFSigGenSoftwareTrigger extension defines the following functions:

- Send Software Trigger

This section describes the behavior and requirements of this function.

### Send Software Trigger

Refer to IVI-3.3: Standard Cross Class Capabilities, Section 2 Software Triggering Capability for the prototype and complete description of this function.

Send Software Trigger is a member of the IIviRFSigGen interface.

## IviRFSigGenSoftwareTrigger Behavior Model

The IviRFSigGenSoftwareTrigger Extension Group follows the behavior model of the IviRFSigGenSweep capability group. The only modification to the behavior model from the IviRFSigGenSweep capability group is the ability to send software triggers.

## IviRFSigGenSoftwareTrigger Compliance Notes

- If a specific driver implements the IviRFSigGenSoftwareTrigger Extension Group, it shall implement at least one of the following extension groups:

- IviRFSigGenSweep Extension Group

- IviRFSigGenArbGenerator Extension Group

- IviRFSigGenCDMABase Extension Group

- IviRFSigGenTDMABase Extension Group

- If a specific driver implements the IviRFSigGenSoftwareTrigger Extension Group together with the IviRFSigGenSweep Extension Group, it shall implement the value Software Trigger for the Sweep Trigger Source attribute.

- If a specific driver implements the IviRFSigGenSoftwareTrigger Extension Group together with the IviRFSigGenArbGenerator Extension Group, it shall implement the value Software Trigger for the Arb Trigger Source attribute.

- If a specific driver implements the IviRFSigGenSoftwareTrigger Extension Group together with the IviRFSigGenCDMABase Extension Group, it shall implement the value Software Trigger for the CDMA Trigger Source attribute.

- If a specific driver implements the IviRFSigGenSoftwareTrigger Extension Group together with the IviRFSigGenTDMABase Extension Group, it shall implement the value Software Trigger for the TDMA Trigger Source attribute.

# IviRFSigGenModulateIQ Extension Group

## IviRFSigGenModulateIQ Extension Group Overview

The IviRFSigGenModulateIQ Extension Group supports signal generators that can apply IQ (vector) modulation to the RF output signal. The user can enable or disable IQ modulation and specify the source of the modulating signal. A calibration is executed with an event function.

## IviRFSigGenModulateIQ Attributes

The IviRFSigGenModulateIQ extension group defines the following attributes:

- IQ Enabled

- IQ Source

- IQ Nominal Voltage

- IQ Swap Enabled

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### IQ Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure IQ Enabled |

.NET Property Name

IQ.Enabled

COM Property Name

IQ.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_IQ_ENABLED

Description

If set to True, the RF signal generator applies IQ modulation to the RF output signal.  If set to False, the RF signal generator does not apply IQ modulation to the RF output signal.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### IQ Nominal Voltage

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | N/A | None | None |

.NET Property Name

IQ.NominalVoltage

COM Property Name

IQ.NominalVoltage

C Constant Name

IVIRFSIGGEN_ATTR_IQ_NOMINAL_VOLTAGE

Description

Returns the voltage at which the instrument achieves full modulation. The value is calculated by SQRT(I2+Q2). The units are Volts.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### IQ Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure IQ |

.NET Property Name

IQ.Source

.NET Enumeration Name

IQSource

COM Property Name

IQ.Source

COM Enumeration Name

IviRFSigGenIQSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_IQ_SOURCE

Description

Specifies the source of the signal that the signal generator uses for IQ modulation.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| DigitalModulationBase | The signal generator uses the internally generated digital modulation signal to apply IQ modulation to the output RF signal. – See chapter IviRFSigGenDigitalModulationBase Extension Group for configuration. |  |  |
|  |  | .NET | IQSource.DigitalModulationBase |
|  |  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_DIGITAL_MODULATION_BASE |
|  |  | COM | IviRFSigGenIQSourceDigitalModulationBase |
| CDMABase | The signal generator uses the internally generated CDMA signal to apply IQ modulation to the output RF signal. – See chapter IviRFSigGenCDMABase Extension Group for configuration. |  |  |
|  |  | .NET | IQSource.CdmaBase |
|  |  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_CDMA_BASE |
|  |  | COM | IviRFSigGenIQSourceCDMABase |
| TDMABase | The signal generator uses the internally generated TDMA signal to apply IQ modulation to the output RF signal. – See chapter IviRFSigGenTDMABase Extension Group for configuration. |  |  |
|  |  | .NET | IQSource.TdmaBase |
|  |  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_TDMA_BASE |
|  |  | COM | IviRFSigGenIQSourceTDMABase |
| ArbGenerator | The signal generator uses the internally generated Arb signal to apply IQ modulation to the output RF signal. – See chapter IviRFSigGenArbGenerator Extension Group for configuration. |  |  |
|  |  | .NET | IQSource.ArbGenerator |
|  |  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_ARB_GENERATOR |
|  |  | COM | IviRFSigGenIQSourceArbGenerator |
| External | The signal generator uses data from an external source for IQ modulation. |  |  |
|  |  | .NET | IQSource.External |
|  |  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenIQSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If a specific driver implements the defined value DigitalModulationBase it shall implement the IviRFSigGenDigitalModulationBase Extension group.

- If a specific driver implements the defined value CDMABase it shall implement the IviRFSigGenCDMABase Extension group.

- If a specific driver implements the defined value TDMABase it shall implement the IviRFSigGenTDMABase Extension group.

- If a specific driver implements the defined value ArbGenerator it shall implement the IviRFSigGenArbGenerator extension group.

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_IQ_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_IQ_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_IQ_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to IQ Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of IQ Source Specific Ext Base, IVIRFSIGGEN_VAL_IQ_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_IQ_SOURCE_CLASS_EXT_BASE.

### IQ Swap Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure IQ |

.NET Property Name

IQ.SwapEnabled

COM Property Name

IQ.SwapEnabled

C Constant Name

IVIRFSIGGEN_ATTR_IQ_SWAP_ENABLED

Description

If set to True, the RF signal generator applies inverse phase rotation of the IQ signal.  If set to False, the RF signal generator applies non-inverse phase rotation of the IQ signal.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the value False.

## IviRFSigGenModulateIQ Functions

The IviRFSigGenModulateIQ extension group defines the following functions:

- Configure IQ Enabled (IVI-C only)

- Configure IQ

- Calibrate IQ

This section describes the behavior and requirements of each function.

### Configure IQ Enabled (IVI-C only)

Description

Configures the signal generator to apply IQ (vector) modulation to the RF output signal.

.NET Method Prototype

N/A

(use the IQ.Enabled property)

COM Method Prototype

N/A

(use the IQ.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureIQEnabled (ViSession Vi, ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | True enables IQ (vector) modulation. The driver uses this value to set the Modulate IQ Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure IQ

Description

Configures the attributes that control the signal generator's IQ modulation. These attributes are the modulation source, impairement state and swap state.

.NET Method Prototype

void IQ.Configure (IQSource source,
                   Boolean swapEnabled);

COM Method Prototype

HRESULT IQ.Configure ([in] IviRFSigGenIQSourceEnum Source,
                      [in] VARIANT_BOOL SwapEnabled);

C Prototype

ViStatus IviRFSigGen_ConfigureIQ (ViSession Vi, ViInt32 Source,
 ViBoolean SwapEnabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | Specifies the modulation source. The driver uses this value to set the Modulate IQ Source attribute. See the attribute description for more details. |
| SwapEnabled | Specifies the swap state. True swaps I and Q inputs, the result is an invers phase rotation. The driver uses this value to set the Modulate IQ Swap Enabled attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Calibrate IQ

Description

This function calibrates the IQ modulator

.NET Method Prototype

void IQ.Calibrate();

COM Method Prototype

HRESULT IQ.Calibrate();

C Prototype

ViStatus IviRFSigGen_CalibrateIQ (ViSession Vi) ;

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenModulateIQ Behavior Model

The IviRFSigGenModulateIQ Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenModulateIQ settings.

# IviRFSigGenIQImpairment Extension Group

## IviRFSigGenIQImpairment Extension Group Overview

With IviRFSigGenIQImpairment Extension Group the user can simulate or correct impairment on IQ (vector) modulation. This group is an extension of the IviRFSigGenModulateIQ Extension Group.

## IviRFSigGenIQImpairment Attributes

The IviRFSigGenIQImpairment extension group defines the following attributes:

- IQ Impairment Enabled

- IQ I-Offset

- IQ Q-Offset

- IQ Ratio

- IQ Skew

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### IQ Impairment Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure IQ Impairment Enabled |

.NET Property Name

IQ.Impairment.Enabled

COM Property Name

IQ.Impairment.Enabled

C Constant Name

IVIRFSIGGEN_ATTR_IQ_IMPAIRMENT_ENABLED

Description

If set to True, the RF signal generator applies all IQ impairment attributes to the IQ modulation.  If set to False, the RF signal generator does not apply any IQ impairment attributes to the IQ modulation.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- Instrument drivers shall support the values True and False.

### IQ I-Offset

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure IQ Impairment |

.NET Property Name

IQ.Impairment.IOffset

COM Property Name

IQ.Impairment.IOffset

C Constant Name

IVIRFSIGGEN_ATTR_IQ_I_OFFSET

Description

Specifies an origin offset voltage to the I signal. The range of values allowed is –100% to +100%. The value is expressed as percentage (%).

Q

I Offset

I

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### IQ Q-Offset

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure IQ Impairment |

.NET Property Name

IQ.Impairment.QOffset

COM Property Name

IQ.Impairment.QOffset

C Constant Name

IVIRFSIGGEN_ATTR_IQ_Q_OFFSET

Description

Specifies an origin offset voltage to the Q signal. The range of values allowed is –100% to +100%. The value is expressed as percentage (%).

Q

Q Offset

I

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### IQ Ratio

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure IQ Impairment |

.NET Property Name

IQ.Impairment.Ratio

COM Property Name

IQ.Impairment.Ratio

C Constant Name

IVIRFSIGGEN_ATTR_IQ_RATIO

Description

Imbalance

Q

I

-   +

-

+

Specifies the gain imbalance between the I and Q channels. For no imbalance this value is set to 0 %. The value is expressed as percentage (%).

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### IQ Skew

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure IQ Impairment |

.NET Property Name

IQ.Impairment.Skew

COM Property Name

IQ.Impairment.Skew

C Constant Name

IVIRFSIGGEN_ATTR_IQ_SKEW

Description

Skew

Q

I

Specifies the adjustment of the phase angle between the I and Q vectors. If this skew is zero, the phase angle is 90 degrees. The units are degrees.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

## IviRFSigGenIQImpairment Functions

The IviRFSigGenIQImpairment extension group defines the following functions:

- Configure IQ Impairment Enabled (IVI-C only)

- Configure IQ Impairment

This section describes the behavior and requirements of each function.

### Configure IQ Impairment Enabled (IVI-C only)

Description

Configures the IQ  modulation to allow controlled impairment for test or external corrections.

.NET Method Prototype

N/A

(use the IQ.Impairment.Enabled property)

COM Method Prototype

N/A

(use the IQ.Impairment.Enabled property)

C Prototype

ViStatus IviRFSigGen_ConfigureIQImpairmentEnabled (ViSession Vi,
ViBoolean Enabled);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Enabled | True enables IQ (vector) modulation impairment. The driver uses this value to set the IQ Impairment Enabled attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure IQ Impairment

Description

Configures the attributes that simulate or correct impairment for the signal generator's IQ  modulation. These attributes are only used, if IQ Impairment Enabled attribute is set to True (see Configure IQ Impairment Enabled function).

.NET Method Prototype

void IQ.Impairment.Configure (Double iOffset,
                              Double qOffset,
                              Double ratio,
                              Double skew);

COM Method Prototype

HRESULT IQ.Impairment.Configure ([in] DOUBLE IOffset,
                                 [in] DOUBLE QOffset,
                                 [in] DOUBLE Ratio,
                                 [in] DOUBLE Skew);

C Prototype

ViStatus IviRFSigGen_ConfigureIQImpairment (ViSession Vi,
 ViReal64 IOffset,
 ViReal64 QOffset,
 ViReal64 Ratio,
 ViReal64 Skew);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| IOffset | Specifies an offset to the I-path of IQ signals. The driver uses this value to set the IQ I Offset attribute. See the attribute description for more details. |
| QOffset | Specifies an offset to the Q-path of IQ signals. The driver uses this value to set the IQ Q Offset attribute. See the attribute description for more details. |
| Ratio | Specifies a gain imbalance to the IQ signals. The driver uses this value to set the IQ Ratio attribute. See the attribute description for more details. |
| Skew | Specifies an angle offset to the IQ signals. The driver uses this value to set the IQ Skew attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenIQImpairment Behavior Model

The IviRFSigGenIQImpairment Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenIQImpairment settings.

## IviRFSigGenIQImpairment Compliance Notes

- If a specific driver implements the IviRFSigGenIQImpairment Extension Group, it shall also implement the IviRFSigGenModulateIQ Extension Group.

# IviRFSigGenArbGenerator Extension Group

## IviRFSigGenArbGenerator Extension Group Overview

The IviRFSigGenArbGenerator Extension Group controls the internal arbitrary waveform generator. The two outputs of the Arb generator are used with the IQ (vector) modulation to simulate digital modulation. So this group is an extension of the IviRFSigGenModulateIQ Extension Group.

## IviRFSigGenArbGenerator Attributes

The IviRFSigGenArbGenerator extension group defines the following attributes:

- Arb Selected Waveform

- Arb Clock Frequency

- Arb Filter Frequency

- Arb Max Number Waveforms

- Arb Waveform Quantum

- Arb Waveform Size Min

- Arb Waveform Size Max

- Arb Trigger Source

- Arb External Trigger Slope

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### Arb Selected Waveform

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select Arb Waveform |

.NET Property Name

DigitalModulation.Arb.SelectedWaveform

COM Property Name

DigitalModulation.Arb.SelectedWaveform

C Constant Name

IVIRFSIGGEN_ATTR_ARB_SELECTED_WAVEFORM

Description

Specifies the selected waveform from the pool of available waveforms.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Clock Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Arb |

.NET Property Name

DigitalModulation.Arb.ClockFrequency

COM Property Name

DigitalModulation.Arb.ClockFrequency

C Constant Name

IVIRFSIGGEN_ATTR_ARB_CLOCK_FREQUENCY

Description

Specifies the sample frequency. The waveform is generated with this clock frequency. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Filter Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | UP | Configure Arb |

.NET Property Name

DigitalModulation.Arb.FilterFrequency

COM Property Name

DigitalModulation.Arb.FilterFrequency

C Constant Name

IVIRFSIGGEN_ATTR_ARB_FILTER_FREQUENCY

Description

Specifies the cut-off frequency of the low pass filter. The waveform is filtered before output with this filter for antialiasing. The filter frequency normally is lower than the clock frequency. The units are Hertz.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Max Number Waveforms

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | Query Arb Waveform Capabilities |

.NET Property Name

DigitalModulation.Arb.MaxNumberWaveforms

COM Property Name

DigitalModulation.Arb.MaxNumberWaveforms

C Constant Name

IVIRFSIGGEN_ATTR_ARB_MAX_NUMBER_WAVEFORMS

Description

Specifies the max number of waveforms the instrument can hold in the memory. The number may depend on the length of the waveform already in the pool of waveforms stored in the instrument.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Waveform Quantum

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | Query Arb Waveform Capabilities |

.NET Property Name

DigitalModulation.Arb.WaveformQuantum

COM Property Name

DigitalModulation.Arb.WaveformQuantum

C Constant Name

IVIRFSIGGEN_ATTR_ARB_WAVEFORM_QUANTUM

Description

The waveform length (the number of samples) shall be a multiple of this quantum. If it is 1, there is no restriction on the length - other than min and max size - for the waveform.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Waveform Size Min

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | Query Arb Waveform Capabilities |

.NET Property Name

DigitalModulation.Arb.WaveformSizeMin

COM Property Name

DigitalModulation.Arb.WaveformSizeMin

C Constant Name

IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MIN

Description

The waveform length (the number of samples) shall be equal or greater than min size. If it is 1, there is no restriction on the length - other than max size and quantum - for the waveform.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Waveform Size Max

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | Query Arb Waveform Capabilities |

.NET Property Name

DigitalModulation.Arb.WaveformSizeMax

COM Property Name

DigitalModulation.Arb.WaveformSizeMax

C Constant Name

IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MAX

Description

The waveform length (the number of samples) shall be equal or less than max size.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### Arb Trigger Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure Arb Trigger Source |
| ViString (.NET) |  |  |  |  |

COM Property Name

DigitalModulation.Arb.TriggerSource

COM Enumeration Name

IviRFSigGenArbTriggerSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_ARB_TRIGGER_SOURCE

Description

Specifies the way to start the Arb waveform or running it continuously.

Defined Values

In IVI.NET the trigger source is a string.  If an IVI driver supports a trigger source and the trigger source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that trigger source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger source strings for trigger sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Immediate | The Arb generator system does not wait for a trigger of any kind, so it is running continuously. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_IMMEDIATE |
|  |  | COM | IviRFSigGenArbTriggerSourceImmediate |
| External | The sweep is started with an external signal. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenArbTriggerSourceExternal |
| Software Trigger | The sweep is started with a software programmable trigger. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SOFTWARE |
|  |  | COM | IviRFSigGenArBbtTriggerSourceSoftware |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Arb Trigger Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Arb Trigger Source Specific Ext Base, IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_CLASS_EXT_BASE.

### Arb External Trigger Slope

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure Arb External Trigger Slope |

.NET Property Name

DigitalModulation.Arb.ExternalTriggerSlope

.NET Enumeration Name

Slope

COM Property Name

DigitalModulation.Arb.ExternalTriggerSlope

COM Enumeration Name

IviRFSigGenArbExternalTriggerSlopeEnum

C Constant Name

IVIRFSIGGEN_ATTR_ARB_EXTERNAL_TRIGGER_SLOPE

Description

Specifies whether the trigger event occurs on the rising or falling edge of the input signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Positive | Enables rising edge triggering. |  |  |
|  |  | .NET | Slope.Positive |
|  |  | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_POSITIVE |
|  |  | COM | IviRFSigGenArbExternalTriggerSlopePositive |
| Negative | Enables falling edge triggering. |  |  |
|  |  | .NET | Slope.Negative |
|  |  | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_NEGATIVE |
|  |  | COM | IviRFSigGenArbExternalTriggerSlopeNegative |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Arb External Trigger Slope Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Arb External Trigger Slope Specific Ext Base, IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE.

## IviRFSigGenArbGenerator Functions

The IviRFSigGenArbGenerator extension group defines the following functions:

- Configure Arb

- Write Arb Waveform

- Select Arb Waveform (IVI-C only)

- Clear All Arb Waveforms

- Query Arb Waveform Capabilities

- Configure Arb Trigger Source (IVI-C only)

- Configure Arb External Trigger Slope (IVI-C only)

This section describes the behavior and requirements of each function.

### Configure Arb

Description

Configures the Arb generator. Specified is the sample frequency the waveform is generated with together with the cut-off frequency of the low pass filter which is used for antialiasing the output waveform. The filter frequency normally is lower than the clock frequency.

.NET Method Prototype

void DigitalModulation.Arb.Configure (Double clockFrequency,
                                      Double filterFrequency);

COM Method Prototype

HRESULT DigitalModulation.Arb.Configure ([in] DOUBLE ClockFrequency,
                                         [in] DOUBLE FilterFrequency);

C Prototype

ViStatus IviRFSigGen_ConfigureArb (ViSession Vi, ViReal64 ClockFrequency,
ViReal64 FilterFrequency);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| ClockFrequency | The driver uses this value to set the Arb Clock Frequency attribute. See the attribute description for more details. |
| FilterFrequency | The driver uses this value to set the Arb Filter Frequency attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return..NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Write Arb Waveform

Description

Stores the transmitted waveform in the drivers’s or instrument’s memory.

.NET Method Prototype

void DigitalModulation.Arb.WriteWaveform (String name,
                                          Double[] iData,
                                          Double[] qData,
                                          Boolean moreDataPending);

COM Method Prototype

HRESULT DigitalModulation.Arb.WriteWaveform ([in] BSTR Name,
                                             [in] SAFEARRAY(DOUBLE) *IData,
                                             [in] SAFEARRAY(DOUBLE) *QData,
                                             [in] VARIANT_BOOL MoreDataPending);

C Prototype

ViStatus IviRFSigGen_WriteArbWaveform (ViSession Vi, ViConstString Name,
ViInt32 NumberOfSamples, ViReal64 IData[],
ViReal64 QData[], ViBoolean MoreDataPending);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| NumberOfSamples | The number of samples in both I and Q array. Not used in COM environment. |
| Name | The driver uses this value to name the stored waveform. |
| IData | The driver uses the values of this array as I part of the waveform . The voltages are normalized to {-1, +1} to the full I range. |
| QData | The driver uses the values of this array as Q part of the waveform . The voltages are normalized to {-1, +1} to the full Q range. |
| MoreDataPending | The value is set to True if more data is coming. To complete the waveform, the next data shall have the same name. False means the waveform has no more data to come. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Select Arb Waveform (IVI-C only)

Description

Selects a waveform from the pool of waveforms to become active.

.NET Method Prototype

N/A

(use the DigitalModulation.Arb.SelectedWaveform property)

COM Method Prototype

N/A

(use the DigitalModulation.Arb.SelectedWaveform property)

C Prototype

ViStatus IviRFSigGen_SelectArbWaveform (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the Arb Selected Waveform attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Clear All Arb Waveforms

Description

Deletes all waveforms from the pool of waveforms currently defined.

.NET Method Prototype

void DigitalModulation.Arb.ClearAllWaveforms ();

COM Method Prototype

HRESULT DigitalModulation.Arb.ClearAllWaveforms ();

C Prototype

ViStatus IviRFSigGen_ClearAllArbWaveforms (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Query Arb Waveform Capabilities (IVI-C and IVI-COM Only)

Description

Returns the capabilities of the Arb generator related to the waveform.

.NET Method Prototype

N/A

(use the DigitalModulation.Arb.MaxNumberWaveforms, DigitalModulation.Arb.WaveformQuantum, DigitalModulation.Arb.MinWaveformSize, and DigitalModulation.Arb.MaxWaveformSize properties.)

COM Method Prototype

HRESULT DigitalModulation.Arb.QueryWaveformCapabilities (
                                       [in, out] LONG *MaxNumberWaveforms,
                                       [in, out] LONG *WaveformQuantum,
                                       [in, out] LONG *MinWaveformSize,
                                       [in, out] LONG *MaxWaveformSize);

C Prototype

ViStatus IviRFSigGen_QueryArbWaveformCapabilities (ViSession Vi,
 ViInt32 *MaxNumberWaveforms,
 ViInt32 *WaveformQuantum,
 ViInt32 *MinWaveformSize,
 ViInt32 *MaxWaveformSize);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

| Outputs | Description |
| --- | --- |
| MaxNumberWaveforms | Returns the Arb Max Number Waveforms attribute. See the attribute description for more details. |
| WaveformQuantum | Returns the Arb Waveform Quantum attribute. See the attribute description for more details. |
| MinWaveformSize | Returns the Arb Min Waveform Size attribute. See the attribute description for more details. |
| MaxWaveformSize | Returns the Arb Max Waveform Size attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Arb Trigger Source (IVI-C only)

Description

Configures the trigger source for waveform generation. The output waveform may be generated continuously or once based on the value of the Arb Trigger Source attribute.

.NET Method Prototype

N/A

(Use the Arb Trigger Source property.)

COM Method Prototype

N/A

(Use the Arb Trigger Source property.)

C Prototype

ViStatus IviRFSigGen_ConfigureArbTriggerSource (ViSession Vi, ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the Arb Trigger Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Arb External Trigger Slope (IVI-C only)

Description

Configures the external trigger slope.

.NET Method Prototype

N/A

(Use the Arb External Trigger Slope property.)

COM Method Prototype

N/A

(Use the Arb External Trigger Slope property.)

C Prototype

ViStatus IviRFSigGen_ConfigureArbExternalTriggerSlope (ViSession Vi,
ViInt32 Slope);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Slope | The driver uses this value to set the Arb External Trigger Slope attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenArbGenerator Behavior Model

The IviRFSigGenArbGenerator Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenArbGenerator settings.

## IviRFSigGenArbGenerator Compliance Notes

If a specific driver implements the IviRFSigGenArbGenerator Extension Group, it shall also implement the IviRFSigGenModulateIQ Extension Group and support the defined value ArbGenerator for the IQ Source attribute.

- If a specific driver does not support the value External for the Arb Trigger Source attribute, it need not support the Arb External Trigger Slope attribute or the Configure Arb External Trigger Slope function.

# IviRFSigGenDigitalModulationBase Extension Group

## IviRFSigGenDigitalModulationBase Extension Group Overview

With IviRFSigGenDigitalModulationBase Extension Group the user can generate signals conforming to wireless communication standards (e.g. mobile cellular standards). The generated signals do not have TDMA framing nor CDMA channel coding.
The functionality covers basic modulation properties such as IQ constellation, symbol mapping, etc. within a specified communication standard.

## IviRFSigGenDigitalModulationBase Attributes

The IviRFSigGenDigitalModulationBaseBase Extension Group defines the following attributes:

- DigitalModulationBase Standard Names (IVI.NET only)

- DigitalModulationBase Standard Name (IVI-COM only)

- DigitalModulationBase Standard Count (IVI-C and IVI-COM only)

- DigitalModulationBase Selected Standard

- DigitalModulationBase Data Source

- DigitalModulationBase PRBS Type

- DigitalModulationBase Selected Bit Sequence

- DigitalModulationBase Clock Source

- DigitalModulationBase External Clock Type

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### DigitalModulationBase Standard Names (.NET only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ReadOnlyCollection<String> | RO | N/A | None | None |

.NET Property Name

ReadOnlyCollection<String> DigitalModulation.Base.StandardNames

COM Property Name

N/A

(Use the DigitalModulationBase Standard Name property.)

C Constant Name

N/A

(Use the Get DigitalModulationBase Standard Name function.)

Description

This property returns a collection of the currently defined Digital Modulation Base Standard identifiers.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### DigitalModulationBase Standard Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A

(Use the DigitalModulationBase Standard Names property.)

COM Property Name

HRESULT DigitalModulation.Base.StandardName ([in] long Index,

[out, retval] BSTR* Name);

C Constant Name

N/A

(Use the Get DigitalModulationBase Standard Name function.)

Description

This property returns the DigitalModulationBase Standard identifier that corresponds to the one-based index that the user specifies. If the value that the user pass for the Index parameter is less than one or greater than the value of the DigitalModulationBase Standard Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the DigitalModulationBase standard name. | ViChar[] |

### DigitalModulationBase Standard Count (IVI-C and IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

N/A

(Use DigitalModulationBase Standard Names Count property.)

COM Property Name

DigitalModulation.Base.StandardCount

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_STANDARD_COUNT

Description

Specifies the number of DigitalModulationBase standards available for a particular instrument.

### DigitalModulationBase Selected Standard

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select DigitalModulationBase Standard |

.NET Property Name

DigitalModulation.Base.SelectedStandard

COM Property Name

DigitalModulation.Base.SelectedStandard

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_SELECTED_STANDARD

Description

Specifies the actual standard used by the instrument. The coding, mapping, symbol rate or bit clock frequency, filter together with the according filter. parameters, FSK deviation or ASK depth (in case of FSK or ASK modulation) are set as defined in the selected standard.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### DigitalModulationBase Data Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure DigitalModulationBase Data Source |

.NET Property Name

DigitalModulation.Base.DataSource

.NET Enumeration Name

DigitalModulationBaseDataSource

COM Property Name

DigitalModulation.Base.DataSource

COM Enumeration Name

IviRFSigGenDigitalModulationBaseDataSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_DATA_SOURCE

Description

Specifies the source of data. The data is used to modulate the RF signal according to the standard selected with the DigitalModulationBase Selected Standard attribute.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| External | The data from an external device connected to the instrument is used. |  |  |
|  |  | .NET | DigitalModulationBaseDataSource.External |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenDigitalModulationBaseDataSourceExternal |
| PRBS | The internal PRBS (Pseudo Random Binary Sequence) generator is used as data source. |  |  |
|  |  | .NET | DigitalModulationBaseDataSource.Prbs |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_PRBS |
|  |  | COM | IviRFSigGenDigitalModulationBaseDataSourcePRBS |
| Bit Sequence | A constant bit sequence is used as data source and repeated continuously. |  |  |
|  |  | .NET | DigitalModulationBaseDataSource.BitSequence |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_BIT_SEQUENCE |
|  |  | COM | IviRFSigGenDigitalModulationBaseDataSourceBitSequence |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Digital Modulation Base Data Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Digital Modulation Base Data Source Specific Ext Base, IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_CLASS_EXT_BASE.

### DigitalModulationBase PRBS Type

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure DigitalModulationBase PRBS Type |

.NET Property Name

DigitalModulation.Base.PrbsType

.NET Enumeration Name

DigitalModulationBasePrbsType

COM Property Name

DigitalModulation.Base.PRBSType

COM Enumeration Name

IviRFSigGenDigitalModulationBasePRBSTypeEnum

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_PRBS_TYPE

Description

Specifies the type of the PRBS as defined in the CCITT-V.52 standard. The PRBS (Pseudo Random Binary Sequence) is used only if DigitalModulationBase Data Source is set to PRBS.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| PRBS9 | Length of PRBS sequence is 29-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs9 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS9 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS9 |
| PRBS11 | Length of PRBS sequence is 211-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs11 |
|  |  | C | IVIRFSIGGEN_VAL_ DIGITAL_MODULATION_BASE PRBS_TYPE_PRBS11 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS11 |
| PRBS15 | Length of PRBS sequence is 215-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs15 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS15 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS15 |
| PRBS16 | Length of PRBS sequence is 216-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs16 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS16 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS16 |
| PRBS20 | Length of PRBS sequence is 220-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs20 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS20 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS20 |
| PRBS21 | Length of PRBS sequence is 2211. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs21 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS21 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS21 |
| PRBS23 | Length of PRBS sequence is 223-1. |  |  |
|  |  | .NET | DigitalModulationBasePrbsType.Prbs23 |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS23 |
|  |  | COM | IviRFSigGenDigitalModulationBasePRBSTypePRBS23 |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Digital Modulation Base PRBS Type Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Digital Modulation Base PRBS Type Specific Ext Base, IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_CLASS_EXT_BASE.

### DigitalModulationBase Selected Bit Sequence

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select DigitalModulationBase Bit Sequence |

.NET Property Name

DigitalModulation.Base.SelectedBitSequence

COM Property Name

DigitalModulation.Base.SelectedBitSequence

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE__BIT_SEQUENCE

Description

Specifies name of the bit sequence (stream) used as data for digital modulation. The sequence is used only if DigitalModulationBase Data Source is set to BitSequence.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### DigitalModulationBase Clock Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure DigitalModulationBase Clock Source |
| ViString (.NET) | R/W | N/A | None | Configure DigitalModulationBase Clock Source |

.NET Property Name

DigitalModulation.Base.ClockSource

COM Property Name

DigitalModulation.Base.ClockSource

COM Enumeration Name

IviRFSigGenDigitalModulationBaseClockSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_CLOCK_SOURCE

Description

Specifies the source of the clock signal used to generate the digital modulation according to the selected standard.

Defined Values

In IVI.NET the clock source is a string.  If an IVI driver supports a clock source and the clock source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that clock source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new clock source strings for clock sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The internal clock generator is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenDigitalModulationBaseClockSourceInternal |
| External | An connected external clock generator (bit or symbol clock frequency) is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenDigitalModulationBaseClockSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Digital Modulation Base Clock Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Digital Modulation Base Clock Source Specific Ext Base, IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_CLASS_EXT_BASE.

### DigitalModulationBase External Clock Type

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure DigitalModulationBase Clock Source |

.NET Property Name

DigitalModulation.Base.ExternalClockType

.NET Enumeration Name

DigitalModulationBaseExternalClockType

COM Property Name

DigitalModulation.Base.ExternalClockType

COM Enumeration Name

IviRFSigGenDigitalModulationBaseExternalClockTypeEnum

C Constant Name

IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE

Description

Specifies the type of the external clock signal used to generate the digital modulation. This value is used only if the DigitalModulationBase Clock Source attribute is set to External.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Bit | The external clock frequency is equal to the bit clock frequency of the digital modulation. |  |  |
|  |  | .NET | DigitalModulationBaseExternalClockType.Bit |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_BIT |
|  |  | COM | IviRFSigGenDigitalModulationBaseExternalClockTypeBit |
| Symbol | The external clock frequency is equal to the symbol clock frequency of the digital modulation. |  |  |
|  |  | .NET | DigitalModulationBaseExternalClockType.Symbol |
|  |  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SYMBOL |
|  |  | COM | IviRFSigGenDigitalModulationBaseExternalClockTypeSymbol |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to Digital Modulation Base External Clock Type Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of Digital Modulation Base External Clock Type Specific Ext Base, IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE.

## IviRFSigGenDigitalModulationBase Functions

The IviRFSigGenDigitalModulationBaseBase Extension Group defines the following functions:

- Get DigitalModulationBaseStandard Name (IVI-C only)

- Select DigitalModulationBase Standard (IVI-C only)

- Configure DigitalModulationBase Clock Source

- Configure DigitalModulationBase Data Source (IVI-C only)

- Configure DigitalModulationBase PRBS Type (IVI-C only)

- Write DigitalModulationBase Bit Sequence

- Select DigitalModulationBase Bit Sequence (IVI-C only)

- Clear All DigitalModulationBase Bit Sequences

This section describes the behavior and requirements of each function.

### Get DigitalModulationBase Standard Name (IVI-C only)

Description

This function returns the specific driver defined DigitalModulationBase standard name that corresponds to the one-based index that the user specifies.

.NET Method Prototype

N/A

(Use DigitalModulation Base Standard Names property.)

COM Method Prototype

N/A

(Use DigitalModulation Base Standard Name property.)

C Prototype

ViStatus IviRFSigGen_GetDigitalModulationBaseStandardName (ViSession Vi,
                                                     ViInt32 Index,
                                                     ViInt32 NameBufferSize,
                                                     ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | A one-based index that defines which name to return. |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name | A user-allocated buffer into which the driver stores the DigitalModulationBase standard name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |

Return Values (C)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

For an instrument with only one DigitalModulationBase standard, that is the DigitalModulationBase Standard Count attribute is one, the driver may return an empty string in the Name parameter.

### Select DigitalModulationBase Standard (IVI-C only)

Description

Selects the actual standard used by the instrument. The standard is selected with one of the names of the list of names queried with the Get DigitalModulationBase Standard Names function. The coding, mapping, symbol rate or bit clock frequency, filter together with the according filter. parameters, FSK deviation or ASK depth (in case of FSK or ASK modulation) are set as defined in the selected standard.

.NET Method Prototype

N/A

(use the DigitalModulation.Base.SelectedStandard property)

COM Method Prototype

N/A

(use the DigitalModulation.Base.SelectedStandard property)

C Prototype

ViStatus IviRFSigGen_SelectDigitalModulationBaseStandard (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the DigitalModulationBase Selected Standard attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

### Configure DigitalModulationBase Clock Source

Description

Configures the DigitalModulationBase clock source.

.NET Method Prototype

void DigitalModulation.Base.ConfigureClockSource(String source,
                          DigitalModulationBaseExternalClockType type);

COM Method Prototype

HRESULT DigitalModulation.Base.ConfigureClockSource ([in] IviRFSigGenDigitalModulationBaseClockSourceEnum Source,
                                   [in] IviRFSigGenDigitalModulationBaseExternalClockTypeEnum Type);

C Prototype

ViStatus IviRFSigGen_ConfigureDigitalModulationBaseClockSource (ViSession Vi, ViInt32 Source, ViInt32 Type);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the DigitalModulationBase Clock Source attribute. See the attribute description for more details. |
| Type | Type is ignored if Source is set to Internal. The driver uses this value to set the DigitalModulationBase External Clock Type attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure DigitalModulationBase Data Source (IVI-C only)

Description

Configures the source of the data for the digital modulation.

.NET Method Prototype

N/A

(use the DigitalModulation.Base.DataSource property)

COM Method Prototype

N/A

(use the DigitalModulation.Base.DataSource property)

C Prototype

ViStatus IviRFSigGen_ConfigureDigitalModulationBaseDataSource (ViSession Vi,
 ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the DigitalModulationBase Data Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

### Configure DigitalModulationBase PRBS Type (IVI-C only)

Description

Configures the type of the PRBS used as data for the digital modulation. The setting is used only if DigitalModulationBase Data Source is set to PRBS.

.NET Method Prototype

N/A

(use the DigitalModulation.Base.PrbsType property)

COM Method Prototype

N/A

(use the DigitalModulation.Base.PRBSType property)

C Prototype

ViStatus IviRFSigGen_ConfigureDigitalModulationBasePRBSType (ViSession Vi, ViInt32 Type);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Type | The driver uses this value to set the DigitalModulationBase PRBS Type attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

### Write DigitalModulationBase Bit Sequence

Description

Writes the bit sequence used as data for the digital modulation. The sequence is repeated continuously. The string consists of binary values (8 bit in 1 char/byte).

.NET Method Prototype

void DigitalModulation.Base.CreateBitSequence (String name,
                                               Long bitCount,
                                               Byte[] sequence,
                                               Boolean moreDataPending);

COM Method Prototype

HRESULT DigitalModulation.Base.WriteBitSequence ([in] BSTR Name,
                                       [in] LONG BitCount,
                                       [in] SAFEARRAY(BYTE) *Sequence,
                                       [in] VARIANT_BOOL MoreDataPending);

C Prototype

ViStatus IviRFSigGen_WriteDigitalModulationBaseBitSequence (ViSession Vi,
ViConstString Name, ViInt32 BitCount,
ViChar Sequence[],
ViBoolean MoreDataPending);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to name the stored bit sequence. |
| BitCount | The number of bits in the sequence. One char/byte has 8 bits, but the stream of bits doesn’t end necessarily on byte boundaries. |
| Sequence | The driver uses this array as stream of bits. The size of the array is bitCount. |
| MoreDataPending | The value is set to True if more data is comming. To complete the sequence, the next data shall have the same name. False means the waveform has no more data to come. |

Return Values (C/COM)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Select DigitalModulationBase Bit Sequence (IVI-C only)

Description

Selects the bit sequence used as data for the digital modulation. The bit sequence is selected with one of the names in the list of names queried with the Get DigitalModulationBase Bit Sequence Names function. The setting is used only if the DigitalModulationBase Data Source attribute is set to BitSequence.

.NET Method Prototype

N/A

(use the DigitalModulation.Base.SelectBitSequence property)

COM Method Prototype

N/A

(use the DigitalModulation.Base.SelectBitSequence property)

C Prototype

ViStatus IviRFSigGen_SelectDigitalModulationBaseBitSequence (ViSession Vi,
 ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the DigitalModulationBase Selected Bit Sequence attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

### Clear All DigitalModulationBase Bit Sequences

Description

Clears (deletes) all named bit sequences.

.NET Method Prototype

void DigitalModulation.Base.ClearAllBitSequences ();

COM Method Prototype

HRESULT DigitalModulation.Base.ClearAllBitSequences ();

C Prototype

ViStatus IviRFSigGen_ClearAllDigitalModulationBaseBitSequences (ViSession Vi);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |

Return Values (C/COM)

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

## IviRFSigGenDigitalModulationBase Behavior Model

The IviRFSigGenDigitalModulationBase Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenDigitalModulationBase settings.

## IviRFSigGenDigitalModulationBase Compliance Notes

If a specific driver implements the IviRFSigGenDigitalModulationBase Extension Group, it shall also implement the IviRFSigGenModulateIQ Extension Group and support the defined value DigitalModulationBase for the IQ Source attribute.

- If a specific driver does not support the value External for the DigitalModulationBase Clock Source, it shall not support the External Clock Type attribute.

- If a specific driver does not support the value PRBS for the DigitalModulationBase Data Source attribute, it shall not implement the DigitalModulationBase PRBS Type attribute and the Configure DigitalModulationBase PRBS Type function.

- If a specific driver does not support the value Bit Sequence for the DigitalModulationBase attribute, it shall not implement the DigitalModulationBase Selected Bit Sequence attribute and the Write DigitalModulationBase Bit Sequence, Select DigitalModulation Bit Sequence and Clear All DigitalModulationBase Bit Sequences functions.

# IviRFSigGenCDMABase Extension Group

## IviRFSigGenCDMABase Extension Group Overview

With IviRFSigGenCDMABase Extension Group the user can simulate mobile (cell) phone modulation based on Code Division Multiple Access (CDMA) standards.

## IviRFSigGenCDMABase Attributes

The IviRFSigGenCDMABase extension group defines the following attributes:

- CDMA Standard Names (IVI.NET only)

- CDMA Standard Name (IVI-COM only)

- CDMA Standard Count (IVI-C and IVI-COM only)

- CDMA Selected Standard

- CDMA Trigger Source

- CDMA External Trigger Slope

- CDMA Test Model Names (IVI.NET only)

- CDMA Test Model Name (IVI-COM only)

- CDMA Test Model Count (IVI-C and IVI-COM only)

- CDMA Selected Test Model

- CDMA Clock Source

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### CDMA Standard Names (IVI.NET only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ReadOnlyCollection<String> | RO | N/A | None | None |

.NET Constant Name

ReadOnlyCollection<String> DigitalModulation.CDMA.StandardNames

COM Property Name

N/A

(Use the CDMA Standard Name property.)

C Constant Name

N/A

(Use the Get CDMA Standard Name function.)

Description

This property returns a collection of the currently defined Digital Modulation CDMA Standard identifiers.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### CDMA Standard Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Constant Name

N/A

(Use the CDMA Standard Names property.)

COM Property Name

HRESULT DigitalModulation.CDMA.StandardName ([in] long Index,

[out, retval] BSTR* Name);

C Constant Name

N/A

(Use the Get CDMA Standard Name function.)

Description

This property returns the CDMA Standard identifier that corresponds to the one-based index that the user specifies. If the value that the user pass for the Index parameter is less than one or greater than the value of the CDMA Standard Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the CDMA standard name. | ViChar[] |

### CDMA Standard Count (IVI-C and IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

N/A

(Use CDMA Standard Names Count property.)

COM Property Name

DigitalModulation.CDMA.StandardCount

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_STANDARD_COUNT

Description

Specifies the number of CDMA standards available for a particular instrument.

### CDMA Selected Standard

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select CDMA Standard |

.NET Property Name

DigitalModulation.Cdma.SelectedStandard

COM Property Name

DigitalModulation.CDMA.SelectedStandard

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_SELECTED_STANDARD

Description

Specifies the actual standard used by the instrument. The modulation type, bit clock frequency and filter together with the according filter. parameters are set as defined in the selected standard.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### CDMA Trigger Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure CDMA Trigger Source |
| ViString (.NET) | R/W | N/A | None | Configure CDMA Trigger Source |

.NET Property Name

DigitalModulation.Cdma.TriggerSource

COM Property Name

DigitalModulation.CDMA.TriggerSource

COM Enumeration Name

IviRFSigGenCDMATriggerSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_TRIGGER_SOURCE

Description

Specifies the source of the trigger signal that starts the channel coding generation.

Defined Values

In IVI.NET the trigger input is a string.  If an IVI driver supports a trigger input and the trigger input is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that trigger input.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger input strings for trigger inputs that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Immediate | The CDMA generator system does not wait for a trigger of any kind, so it is running the channel codings continuously. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_IMMEDIATE |
|  |  | COM | IviRFSigGenCDMATriggerSourceImmediate |
| External | Each channel coding is started with an external signal. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenCDMATriggerSourceExternal |
| Software Trigger | Each channel coding is started with a software programmable trigger. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SOFTWARE |
|  |  | COM | IviRFSigGenCDMATriggerSourceSoftware |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to CDMA Trigger Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of CDMA Trigger Source Specific Ext Base, IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_CLASS_EXT_BASE.

### CDMA External Trigger Slope

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure CDMA External Trigger Slope |

.NET Property Name

DigitalModulation.Cdma.ExternalTriggerSlope

.NET Enumeration Name

Slope

COM Property Name

DigitalModulation.CDMA.ExternalTriggerSlope

COM Enumeration Name

IviRFSigGenCDMAExternalTriggerSlopeEnum

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_EXTERNAL_TRIGGER_SLOPE

Description

Specifies whether the trigger event occurs on the rising or falling edge of the input signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Positive | Enables rising edge triggering. |  |  |
|  |  | .NET | Slope.Positive |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_ EXTERNAL_TRIGGER_SLOPE_POSITIVE |
|  |  | COM | IviRFSigGenCDMAExternalTriggerSlopePositive |
| Negative | Enables falling edge triggering. |  |  |
|  |  | .NET | Slope.Negative |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_ EXTERNAL_TRIGGER_SLOPE_NEGATIVE |
|  |  | COM | IviRFSigGenCDMAExternalTriggerSlopeNegative |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to CDMA Trigger External Slope Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of CDMA Trigger External Slope Specific Ext Base, IVIRFSIGGEN_VAL_CDMA_TRIGGER_EXTERNAL_SLOPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_CDMA_TRIGGER_EXTERNAL_SLOPE_CLASS_EXT_BASE.

### CDMA Test Model Names (IVI.NET only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ReadOnlyCollection<String> | RO | N/A | None | None |

.NET Property Name

ReadOnlyCollection<String> DigitalModulation.CDMA.TestModelNames

COM Property Name

(Use the CDMA Test Model Name property.)

C Constant Name

N/A

(Use the Get CDMA Test Model Name function.)

Description

This property returns a collection of the currently defined Digital Modulation CDMA Test Model identifiers.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### CDMA Test Model Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A

(Use the CDMA Test Model Names property.)

COM Property Name

HRESULT DigitalModulation.CDMA.TestModelName ([in] long Index,

[out, retval] BSTR* Name);

C Constant Name

N/A

(Use the Get CDMA Test Model Name function.)

Description

This property returns the CDMA Test Model identifier that corresponds to the one-based index that the user specifies. If the value that the user pass for the Index parameter is less than one or greater than the value of the CDMA Test Model Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the CDMA test model name. | ViChar[] |

### CDMA Test Model Count (IVI-C and IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

N/A

(Use CDMA Test Model Names Count property.)

COM Property Name

DigitalModulation.CDMA.TestModelCount

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_TEST_MODEL_COUNT

Description

Specifies the number of CDMA test models available for a particular instrument.

### CDMA Selected Test Model

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select CDMA Test Model |

.NET Property Name

DigitalModulation.Cdma.SelectedTestModel

COM Property Name

DigitalModulation.CDMA.SelectedTestModel

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_SELECTED_TEST_MODEL

Description

Specifies the actual CDMA test model used by the instrument.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### CDMA Clock Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure CDMA Clock Source |
| ViString (.NET) | R/W | N/A | None | Configure CDMA Clock Source |

.NET Property Name

DigitalModulation.Cdma.ClockSource

COM Property Name

DigitalModulation.CDMA.ClockSource

COM Enumeration Name

IviRFSigGenCDMAClockSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_CDMA_CLOCK_SOURCE

Description

Specifies the source of the clock signal used to generate the digital modulation according to the selected standard.

Defined Values

In IVI.NET the CDMA clock source is a string.  If an IVI driver supports a CDMA clock source and the CDMA clock source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that CDMA clock source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger input strings for CDMA clock sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The internal clock generator is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenCDMAClockSourceInternal |
| External | A connected external clock generator (bit or symbol clock frequency) is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenCDMAClockSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to CDMA Clock Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of CDMA Clock Source Specific Ext Base, IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_CLASS_EXT_BASE.

## IviRFSigGenCDMABase Functions

The IviRFSigGenCDMABase extension group defines the following functions:

- Get CDMA Standard Name (IVI-C only)

- Select CDMA Standard (IVI-C only)

- Configure CDMA Clock Source (IVI-C only)

- Configure CDMA Trigger Source (IVI-C only)

- Configure CDMA External Trigger Slope (IVI-C only)

- Get CDMA Test Model Name (IVI-C only)

- Select CDMA Test Model (IVI-C only)

This section describes the behavior and requirements of each function.

### Get CDMA Standard Name (IVI-C only)

Description

This function returns the specific driver defined CDMA standard name that corresponds to the one-based index that the user specifies.

.NET Method Prototype

(Use the CDMA Standard Names property.)

COM Method Prototype

N/A

(Use the CDMA Standard Name property.)

C Prototype

ViStatus IviRFSigGen_GetCDMAStandardName (ViSession Vi,

ViInt32 Index,

ViInt32 NameBufferSize,

ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | A one-based index that defines which name to return. |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name | A user-allocated buffer into which the driver stores the CDMA standard name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

For an instrument with only one CDMA standard, that is the CDMA Standard Count attribute is one, the driver may return an empty string in the Name parameter.

### Select CDMA Standard (IVI-C only)

Description

Selects the actual standard used by the instrument. The standard is selected from the list of names queried with the Get CDMA Standard Names function. The modulation type, clock frequency and filter together with the according filter. parameters are set as defined in the selected standard.

.NET Method Prototype

N/A

(use the DigitalModulation.Cdma.SelectedStandard property)

COM Method Prototype

N/A

(use the DigitalModulation.CDMA.SelectedStandard property)

C Prototype

ViStatus IviRFSigGen_SelectCDMAStandard (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the CDMA Selected Standard attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure CDMA Clock Source (IVI-C only)

Description

Configures the CDMA clock source.

.NET Method Prototype

N/A

(use the DigitalModulation.Cdma.ClockSource property)

COM Method Prototype

N/A

(use the DigitalModulation.CDMA.ClockSource property)

C Prototype

ViStatus IviRFSigGen_ConfigureCDMAClockSource (ViSession Vi, ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the CDMA Clock Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure CDMA Trigger Source (IVI-C only)

Description

Configures the CDMA trigger source for starting or synchronising the generation of the channel codings.

.NET Method Prototype

N/A

(use the DigitalModulation.Cdma.TriggerSource property)

COM Method Prototype

N/A

(use the DigitalModulation.CDMA.TriggerSource property)

C Prototype

ViStatus IviRFSigGen_ConfigureCDMATriggerSource (ViSession Vi, ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the CDMA Trigger Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure CDMA External Trigger Slope (IVI-C only)

Description

Configures whether the trigger event occurs on the rising or falling edge of the input signal. This setting is used only if CDMA Trigger Source is set to External.

.NET Method Prototype

N/A

(use the DigitalModulation.Cdma.ExternalTriggerSlope property)

COM Method Prototype

N/A

(use the DigitalModulation.CDMA.ExternalTriggerSlope property)

C Prototype

ViStatus IviRFSigGen_ConfigureCDMAExternalTriggerSlope (ViSession Vi,
 ViInt32 Slope);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Slope | The driver uses this value to set the CDMA External Trigger Slope attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Get CDMA Test Model Name (IVI-C only)

Description

This function returns the specific driver defined CDMA test model name that corresponds to the one-based index that the user specifies.

.NET Method Prototype

N/A

(Use the CDMA Test Model Names property.)

COM Method Prototype

N/A

(Use the CDMA Test Model Name property.)

C Prototype

ViStatus IviRFSigGen_GetCDMATestModelName (ViSession Vi,

ViInt32 Index,

ViInt32 NameBufferSize,

ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | A one-based index that defines which name to return. |
| BufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name | A user-allocated buffer into which the driver stores the CDMA test model name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

For an instrument with only one CDMA test model, that is the CDMA Test Model Count attribute is one, the driver may return an empty string in the Name parameter.

### Select CDMA Test Model (IVI-C only)

Description

Selects the channel coding used for the digital modulation. The channel coding is selected with one of the names in the list of names queried with the Get CDMA Test Model Names function.

.NET Method Prototype

N/A

(use the DigitalModulation.Cdma.SelectedTestModel property)

COM Method Prototype

N/A

(use the DigitalModulation.CDMA.SelectedTestModel property)

C Prototype

ViStatus IviRFSigGen_SelectCDMATestModel (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the CDMA Selected Test Model attribute. See the attribute description for more details. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenCDMABase Behavior Model

The IviRFSigGenCDMABase Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenCDMABase settings.

## IviRFSigGenCDMABase Compliance Notes

For a specific driver to comply with the IviRFSigGenCDMABase extension, it shall also implement the IviRFSigGenModulateIQ Extension Group and support the defined value CDMABase for the IQ Source attribute.

- If a specific driver does not support the defined value External for the CDMA Trigger Source attribute, it shall not support the CDMA External Trigger Slope attribute or the Configure CDMA External Trigger Slope function.

# IviRFSigGenTDMABase Extension Group

## IviRFSigGenTDMABase Extension Group Overview

With IviRFSigGenTDMABase Extension Group the user can simulate mobile (cell) phone modulation based on Time Division Multiple Access (TDMA) standards.

## IviRFSigGenTDMABase Attributes

The IviRFSigGenTDMABase extension group defines the following attributes:

- TDMA Standard Names (IVI.NET only)

- TDMA Standard Name (IVI-COM only)

- TDMA Standard Count  (IVI-C and IVI-COM only)

- TDMA Selected Standard

- TDMA Trigger Source

- TDMA External Trigger Slope

- TDMA Frame Names (IVI.NET only)

- TDMA Frame Name (IVI-COM only)

- TDMA Frame Count  (IVI-C and IVI-COM only)

- TDMA Selected Frame

- TDMA Clock Source

- TDMA External Clock Type

This section describes the behavior and requirements of each attribute.  The actual value for each attribute ID is defined in Section 30, IviRFSigGen Attribute ID Definitions.

### TDMA Standard Names (IVI.NET only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ReadOnlyCollection<String> | RO | N/A | None | None |

.NET Property Name

ReadOnlyCollection<String> DigitalModulation.TDMA.StandardNames

COM Property Name

N/A

(Use the TDMA Standard Name property.)

C Constant Name

N/A

(Use the Get TDMA Standard Name function.)

Description

This property returns a collection of the currently defined Digital Modulation TDMA Standard identifiers.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### TDMA Standard Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A

(Use the TDMA Standard Names property.)

COM Property Name

HRESULT DigitalModulation.TDMA.StandardName ([in] long Index,

[out, retval] BSTR* Name);

C Constant Name

N/A

(Use the Get TDMA Standard Names function.)

Description

This property returns the TDMA Standard identifier that corresponds to the one-based index that the user specifies. If the value that the user pass for the Index parameter is less than one or greater than the value of the TDMA Standard Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the TDMA standard name. | ViChar[] |

### TDMA Standard Count (IVI-C and IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

N/A

(Use TDMA Standard Names Count property.)

COM Property Name

DigitalModulation.TDMA.StandardCount

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_STANDARD_COUNT

Description

Specifies the number of TDMA standards available for a particular instrument.

### TDMA Selected Standard

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select TDMA Standard |

.NET Property Name

DigitalModulation.Tdma.SelectedStandard

COM Property Name

DigitalModulation.TDMA.SelectedStandard

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_SELECTED_STANDARD

Description

Specifies the actual standard used by the instrumen. The coding, mapping, symbol rate or bit clock frequency, filter together with the according filter. parameters, FSK deviation or ASK depth (in case of FSK or ASK modulation) are set as defined in the selected standard.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### TDMA Trigger Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure TDMA Trigger Source |
| ViString (.NET) | R/W | N/A | None | Configure TDMA Trigger Source |

.NET Property Name

DigitalModulation.Tdma.TriggerSource

COM Property Name

DigitalModulation.TDMA. TriggerSource

COM Enumeration Name

IviRFSigGenTDMATriggerSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_TRIGGER_SOURCE

Description

Specifies the source of the trigger signal that starts the frame/slot generation.

Defined Values

In IVI.NET the trigger source is a string.  If an IVI driver supports a trigger source and the trigger source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that trigger source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new trigger source strings for trigger sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Immediate | The TDMA generator system does not wait for a trigger of any kind, so it is running the frames continuously. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_IMMEDIATE |
|  |  | COM | IviRFSigGenTDMATriggerSourceImmediate |
| External | Each frame is started with an external signal. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenTDMATriggerSourceExternal |
| Software Trigger | Each frame is started with a software programmable trigger. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SOFTWARE |
|  |  | COM | IviRFSigGenTDMATriggerSourceSoftware |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to TDMA Trigger Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of TDMA Trigger Source Specific Ext Base, IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_CLASS_EXT_BASE.

### TDMA External Trigger Slope

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure TDMA External Trigger Slope |

.NET Property Name

DigitalModulation.Tdma.ExternalTriggerSlope

.NET Enumeration Name

Slope

COM Property Name

DigitalModulation.TDMA.ExternalTriggerSlope

COM Enumeration Name

IviRFSigGenTDMAExternalTriggerSlopeEnum

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_TRIGGER_SLOPE

Description

Specifies whether the trigger event occurs on the rising or falling edge of the input signal.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Positive | Enables rising edge triggering. |  |  |
|  |  | .NET | Slope.Positive |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_POSITIVE |
|  |  | COM | IviRFSigGenTDMAExternalTriggerSlopePositive |
| Negative | Enables falling edge triggering. |  |  |
|  |  | .NET | Slope.Negative |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_NEGATIVE |
|  |  | COM | IviRFSigGenTDMAExternalTriggerSlopeNegative |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to TDMA External Trigger Slope Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of TDMA External Trigger Slope Specific Ext Base, IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE.

### TDMA Frame Names (IVI.NET only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ReadOnlyCollection<String> | RO | N/A | None | None |

.NET Property Name

ReadOnlyCollection<String> DigitalModulation.TDMA.FrameNames

COM Property Name

N/A

(Use the TDMA Frame Name property.)

C Constant Name

N/A

(Use the Get TDMA Frame Name function.)

Description

This property returns a collection of the currently defined Digital Modulation TDMA Frame identifiers.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### TDMA Frame Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | N/A | None | None |

.NET Property Name

N/A

(Use the TDMA Frame Names property.)

COM Property Name

HRESULT DigitalModulation.TDMA.FrameName ([in] long Index,

[out, retval] BSTR* Name);

C Constant Name

N/A

(Use the Get TDMA Frame Name function.)

Description

This property returns the TDMA Frame identifier that corresponds to the one-based index that the user specifies. If the value that the user pass for the Index parameter is less than one or greater than the value of the TDMA Frame Count attribute, the property returns and empty string in the Name parameter and returns the Invalid Value error.

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Index | A one-based index that defines which name to return. | ViInt32 |
| Output/Return Value | Description | Base Type |
| Name | A driver-allocated buffer into which the driver stores the TDMA frame name. | ViChar[] |

### TDMA Frame Count (IVI-C and IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | N/A |

.NET Property Name

N/A

(Use TDMA Frame Names Count property.)

COM Property Name

DigitalModulation.TDMA.FrameCount

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_FRAME_COUNT

Description

Specifies the number of TDMA frames available for a particular instrument.

### TDMA Selected Frame

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | N/A | None | Select TDMA Frame |

.NET Property Name

DigitalModulation.Tdma.SelectedFrame

COM Property Name

DigitalModulation.TDMA.SelectedFrame

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_SELECTED_FRAME

Description

Specifies the actual frame used by the instrument. It is selected from the list queried with the function Get TDMA Frame Names.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

### TDMA Clock Source

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 (C/COM) | R/W | N/A | None | Configure TDMA Clock Source |
| ViString (.NET) | R/W | N/A | None | Configure TDMA Clock Source |

.NET Property Name

DigitalModulation.Tdma.ClockSource

COM Property Name

DigitalModulation.TDMA.ClockSource

COM Enumeration Name

IviRFSigGenTDMAClockSourceEnum

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_CLOCK_SOURCE

Description

Specifies the source of the clock signal used to generate the digital modulation according to the selected standard.

Defined Values

In IVI.NET the TDMA clock source is a string.  If an IVI driver supports a TDMA clock source and the TDMA clock source is listed in IVI-3.3 Cross Class Capabilities Specification, Section 3 then the IVI driver shall accept the standard string for that TDMA clock source.  This attribute is case insensitive, but case preserving.  That is the setting is case insensitive but when reading it back the programmed case is returned.  IVI specific drivers may define new TDMA clock source strings for TDMA clock sources that are not defined by IVI-3.3 Cross Class Capabilities Specification if needed.

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Internal | The internal clock generator is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_INTERNAL |
|  |  | COM | IviRFSigGenTDMAClockSourceInternal |
| External | A connected external clock generator (bit or symbol clock frequency) is used. |  |  |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_EXTERNAL |
|  |  | COM | IviRFSigGenTDMAClockSourceExternal |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to TDMA Clock Source Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of TDMA Clock Source Specific Ext Base, IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_CLASS_EXT_BASE.

### TDMA External Clock Type

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | N/A | None | Configure TDMA Clock Source |

.NET Property Name

DigitalModulation.Tdma.ExternalClockType

.NET Enumeration Name

TdmaExternalClockType

COM Property Name

DigitalModulation.TDMA.ExternalClockType

COM Enumeration Name

IviRFSigGenTDMAExternalClockTypeEnum

C Constant Name

IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_CLOCK_TYPE

Description

Specifies the type of the external clock signal used to generate the digital modulation. This value is used only if TDMA Clock Source is set to External.

Defined Values

| Name | Description |  |  |
| --- | --- | --- | --- |
|  |  | Language | Identifier |
| Bit | The external clock frequency is equal to the bit clock frequency of the digital modulation. |  |  |
|  |  | .NET | TdmaExternalClockType.Bit |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_BIT |
|  |  | COM | IviRFSigGenTDMAExternalClockTypeBit |
| Symbol | The external clock frequency is equal to the symbol clock frequency of the digital modulation. |  |  |
|  |  | .NET | TdmaExternalClockType.Symbol |
|  |  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SYMBOL |
|  |  | COM | IviRFSigGenTDMAExternalClockTypeSymbol |

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this property.

Compliance Notes

- If an IVI-C class driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE and less than IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE.

- If an IVI-C specific driver defines additional values for this attribute, the actual values shall be greater than or equal to IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE.

- When an IVI-COM specific driver implements this attribute with additional elements in its instrument specific interfaces, the actual values of the additional elements shall be greater than or equal to TDMA External Clock Type Specific Ext Base.

See Section Attribute Value Definitions, for the definitions of TDMA External Clock Type Specific Ext Base, IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE and IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE.

## IviRFSigGenTDMABase Functions

The IviRFSigGenTDMABase extension group defines the following functions:

- Get TDMA Standard Name (IVI-C only)

- Select TDMA Standard (IVI-C only)

- Configure TDMA Clock Source

- Configure TDMA Trigger Source (IVI-C only)

- Configure TDMA External Trigger Slope (IVI-C only)

- Get TDMA Frame Name (IVI-C only)

- Select TDMA Frame (IVI-C only)

This section describes the behavior and requirements of each function.

### Get TDMA Standard Name (IVI-C only)

Description

This function returns the specific driver defined TDMA standard name that corresponds to the one-based index that the user specifies.

.NET Method Prototype

N/A

(Use the TDMA Standard Names property.)

COM Method Prototype

N/A

(Use the TDMA Standard Name property.)

C Prototype

ViStatus IviRFSigGen_GetTDMAStandardName (ViSession Vi,

ViInt32 Index,

ViInt32 NameBufferSize,

ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | A one-based index that defines which name to return. |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name | A user-allocated buffer into which the driver stores the TDMA standard name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

For an instrument with only one TDMA standard, that is the TDMA Standard Count attribute is one, the driver may return an empty string in the Name parameter.

### Select TDMA Standard (IVI-C only)

Description

Selects the actual standard used by the instrument. The standard is selected from the list of names queried with the Get TDMA Standard Names function. The coding, mapping, symbol rate or bit clock frequency, filter together with the according filter. parameters, FSK deviation or ASK depth (in case of FSK or ASK modulation) are set as defined in the selected standard.

.NET Method Prototype

N/A

(use the DigitalModulation.Tdma.SelectedStandard property)

COM Method Prototype

N/A

(use the DigitalModulation.TDMA.SelectedStandard property)

C Prototype

ViStatus IviRFSigGen_SelectTDMAStandard (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the TDMA Selected Standard attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure TDMA Clock Source

Description

Configures the TDMA clock source.

.NET Method Prototype

void DigitalModulation.Tdma.ConfigureClockSource (String source,
                                                  TdmaExternalClockType type);

COM Method Prototype

HRESULT DigitalModulation.TDMA.ConfigureClockSource (
                               [in] IviRFSigGenTDMAClockSourceEnum Source,
                               [in] IviRFSigGenTDMAExternalClockTypeEnum Type);

C Prototype

ViStatus IviRFSigGen_ConfigureTDMAClockSource (ViSession Vi, ViInt32 Source, ViInt32 Type);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the TDMA Clock Source attribute. See the attribute description for more details. |
| Type | Type is ignored if Source is set to Internal. The driver uses this value to set the TDMA External Clock Type attribute. See the attribute description for more details. |

Return Values (C/COM)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

.NET Exceptions

The IVI-3.2: Inherent Capabilities Specification defines general exceptions that may be thrown, and warning events that may be raised, by this method.

### Configure TDMA Trigger Source (IVI-C only)

Description

Configures the TDMA trigger source for starting or synchronising the generation of the frames/slots.

.NET Method Prototype

N/A

(use the DigitalModulation.Tdma.TriggerSource property)

COM Method Prototype

N/A

(use the DigitalModulation.TDMA.TriggerSource property)

C Prototype

ViStatus IviRFSigGen_ConfigureTDMATriggerSource (ViSession Vi,
 ViInt32 Source);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Source | The driver uses this value to set the TDMA Trigger Source attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure TDMA External Trigger Slope (IVI-C only)

Description

Configures whether the trigger event occurs on the rising or falling edge of the input signal. This setting is used only if the TDMA Trigger Source attribute is set to External.

.NET Method Prototype

N/A

(use the DigitalModulation.Tdma.ExternalTriggerSlope property)

COM Method Prototype

N/A

(use the DigitalModulation.TDMA.ExternalTriggerSlope property)

C Prototype

ViStatus IviRFSigGen_ConfigureTDMAExternalTriggerSlope (ViSession Vi,
 ViInt32 Slope);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Slope | The driver uses this value to set the TDMA ExternalTrigger Slope attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Get TDMA Frame Name (IVI-C only)

Description

This function returns the specific driver defined TDMA frame name that corresponds to the one-based index that the user specifies.

.NET Method Prototype

N/A

(Use the TDMA Frame Names property.)

COM Method Prototype

N/A

(Use the TDMA Frame Name property.)

C Prototype

ViStatus IviRFSigGen_GetTDMAFrameName (ViSession Vi,

ViInt32 Index,

ViInt32 NameBufferSize,

ViChar Name[]);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Index | A one-based index that defines which name to return. |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. |

| Outputs | Description |
| --- | --- |
| Name | A user-allocated buffer into which the driver stores the TDMA frame name. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

For an instrument with only one TDMA frame, that is the TDMA Frame Count attribute is one, the driver may return an empty string in the Name parameter.

### Select TDMA Frame (IVI-C only)

Description

Selects the frame used for the framed digital modulation. The frame is selected from the list of names queried with the Get TDMA Framed Names function.

.NET Method Prototype

N/A

(use the DigitalModulation.Tdma.SelectedFrame property)

COM Method Prototype

N/A

(use the DigitalModulation.TDMA.SelectedFrame property)

C Prototype

ViStatus IviRFSigGen_SelectTDMAFrame (ViSession Vi, ViConstString Name);

Parameters

| Inputs | Description |
| --- | --- |
| Vi | Instrument handle |
| Name | The driver uses this value to set the TDMA Selected Frame attribute. See the attribute description for more details. |

Return Values (C)

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviRFSigGenTDMABase Behavior Model

The IviRFSigGenTDMABase Extension Group follows the behavior model of the IviRFSigGenBase capability group. The only modification to the behavior model from the IviRFSigGenBase capability group is the ability to configure IviRFSigGenTDMABase settings.

## IviRFSigGenTDMABase Compliance Notes

For a specific driver to comply with the IviRFSigGenTDMABase extension, it shall also implement the IviRFSigGenModulateIQ Extension Group and support the defined value TDMABase for the IQ Source attribute.

- If a specific driver does not support the defined value External for the TDMA Trigger Source attribute, it shall not support the TDMA External Trigger Slope attribute, the TDMA External Clock Type attribute or the Configure TDMA External Trigger Slope function.

# IviRFSigGen Attribute ID Definitions

Table 30-2 defines the ID values for all IviRFSigGen class attributes.

| Table 30-2 IviRFSigGen Attributes ID Values (C only) |  |
| --- | --- |
| Attribute Name | ID Definition |
| IVIRFSIGGEN_ATTR_FREQUENCY | IVI_CLASS_ATTR_BASE + 1 |
| IVIRFSIGGEN_ATTR_POWER_LEVEL | IVI_CLASS_ATTR_BASE + 2 |
| IVIRFSIGGEN_ATTR_ALC_ENABLED | IVI_CLASS_ATTR_BASE + 3 |
| IVIRFSIGGEN_ATTR_OUTPUT_ENABLED | IVI_CLASS_ATTR_BASE + 4 |
| IVIRFSIGGEN_ATTR_AM_ENABLED | IVI_CLASS_ATTR_BASE + 11 |
| IVIRFSIGGEN_ATTR_AM_SOURCE | IVI_CLASS_ATTR_BASE + 12 |
| IVIRFSIGGEN_ATTR_AM_SCALING | IVI_CLASS_ATTR_BASE + 13 |
| IVIRFSIGGEN_ATTR_AM_EXTERNAL_COUPLING | IVI_CLASS_ATTR_BASE + 14 |
| IVIRFSIGGEN_ATTR_AM_NOMINAL_VOLTAGE | IVI_CLASS_ATTR_BASE + 15 |
| IVIRFSIGGEN_ATTR_AM_DEPTH | IVI_CLASS_ATTR_BASE + 16 |
| IVIRFSIGGEN_ATTR_FM_ENABLED | IVI_CLASS_ATTR_BASE + 21 |
| IVIRFSIGGEN_ATTR_FM_SOURCE | IVI_CLASS_ATTR_BASE + 22 |
| IVIRFSIGGEN_ATTR_FM_EXTERNAL_COUPLING | IVI_CLASS_ATTR_BASE + 23 |
| IVIRFSIGGEN_ATTR_FM_NOMINAL_VOLTAGE | IVI_CLASS_ATTR_BASE + 24 |
| IVIRFSIGGEN_ATTR_FM_DEVIATION | IVI_CLASS_ATTR_BASE + 25 |
| IVIRFSIGGEN_ATTR_PM_ENABLED | IVI_CLASS_ATTR_BASE + 31 |
| IVIRFSIGGEN_ATTR_PM_SOURCE | IVI_CLASS_ATTR_BASE + 32 |
| IVIRFSIGGEN_ATTR_PM_EXTERNAL_COUPLING | IVI_CLASS_ATTR_BASE + 33 |
| IVIRFSIGGEN_ATTR_PM_NOMINAL_VOLTAGE | IVI_CLASS_ATTR_BASE + 34 |
| IVIRFSIGGEN_ATTR_PM_DEVIATION | IVI_CLASS_ATTR_BASE + 35 |
| IVIRFSIGGEN_ATTR_ANALOG_MODULATION_SOURCE_COUNT | IVI_CLASS_ATTR_BASE + 41 |
| IVIRFSIGGEN_ATTR_PULSE_MODULATION_ENABLED | IVI_CLASS_ATTR_BASE + 51 |
| IVIRFSIGGEN_ATTR_PULSE_MODULATION_SOURCE | IVI_CLASS_ATTR_BASE + 52 |
| IVIRFSIGGEN_ATTR_PULSE_MODULATION_EXTERNAL_POLARITY | IVI_CLASS_ATTR_BASE + 53 |
| IVIRFSIGGEN_ATTR_ACTIVE_LF_GENERATOR | IVI_CLASS_ATTR_BASE + 101 |
| IVIRFSIGGEN_ATTR_LF_GENERATOR_COUNT | IVI_CLASS_ATTR_BASE + 102 |
| IVIRFSIGGEN_ATTR_LF_GENERATOR_FREQUENCY | IVI_CLASS_ATTR_BASE + 103 |
| IVIRFSIGGEN_ATTR_LF_GENERATOR_WAVEFORM | IVI_CLASS_ATTR_BASE + 104 |
| IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_AMPLITUDE | IVI_CLASS_ATTR_BASE + 111 |
| IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_ENABLED | IVI_CLASS_ATTR_BASE + 112 |
| IVIRFSIGGEN_ATTR_PULSE_INTERNAL_TRIGGER_PERIOD | IVI_CLASS_ATTR_BASE + 121 |
| IVIRFSIGGEN_ATTR_PULSE_WIDTH | IVI_CLASS_ATTR_BASE + 122 |
| IVIRFSIGGEN_ATTR_PULSE_GATING_ENABLED | IVI_CLASS_ATTR_BASE + 123 |
| IVIRFSIGGEN_ATTR_PULSE_TRIGGER_SOURCE | IVI_CLASS_ATTR_BASE + 124 |
| IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_SLOPE | IVI_CLASS_ATTR_BASE + 125 |
| IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_DELAY | IVI_CLASS_ATTR_BASE + 126 |
| IVIRFSIGGEN_ATTR_PULSE_DOUBLE_ENABLED | IVI_CLASS_ATTR_BASE + 131 |
| IVIRFSIGGEN_ATTR_PULSE_DOUBLE_DELAY | IVI_CLASS_ATTR_BASE + 132 |
| IVIRFSIGGEN_ATTR_PULSE_OUTPUT_POLARITY | IVI_CLASS_ATTR_BASE + 141 |
| IVIRFSIGGEN_ATTR_PULSE_OUTPUT_ENABLED | IVI_CLASS_ATTR_BASE + 142 |
| IVIRFSIGGEN_ATTR_SWEEP_MODE | IVI_CLASS_ATTR_BASE + 201 |
| IVIRFSIGGEN_ATTR_SWEEP_TRIGGER_SOURCE | IVI_CLASS_ATTR_BASE + 202 |
| IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_START | IVI_CLASS_ATTR_BASE + 211 |
| IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_STOP | IVI_CLASS_ATTR_BASE + 212 |
| IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_TIME | IVI_CLASS_ATTR_BASE + 213 |
| IVIRFSIGGEN_ATTR_POWER_SWEEP_START | IVI_CLASS_ATTR_BASE + 221 |
| IVIRFSIGGEN_ATTR_POWER_SWEEP_STOP | IVI_CLASS_ATTR_BASE + 222 |
| IVIRFSIGGEN_ATTR_POWER_SWEEP_TIME | IVI_CLASS_ATTR_BASE + 223 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_START | IVI_CLASS_ATTR_BASE + 241 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_STOP | IVI_CLASS_ATTR_BASE + 242 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SCALING | IVI_CLASS_ATTR_BASE + 243 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SIZE | IVI_CLASS_ATTR_BASE + 244 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SINGLE_STEP_ENABLED | IVI_CLASS_ATTR_BASE + 245 |
| IVIRFSIGGEN_ATTR_FREQUENCY_STEP_DWELL | IVI_CLASS_ATTR_BASE + 246 |
| IVIRFSIGGEN_ATTR_POWER_STEP_START | IVI_CLASS_ATTR_BASE + 261 |
| IVIRFSIGGEN_ATTR_POWER_STEP_STOP | IVI_CLASS_ATTR_BASE + 262 |
| IVIRFSIGGEN_ATTR_POWER_STEP_SIZE | IVI_CLASS_ATTR_BASE + 263 |
| IVIRFSIGGEN_ATTR_POWER_STEP_SINGLE_STEP_ENABLED | IVI_CLASS_ATTR_BASE + 264 |
| IVIRFSIGGEN_ATTR_POWER_STEP_DWELL | IVI_CLASS_ATTR_BASE + 265 |
| IVIRFSIGGEN_ATTR_LIST_SELECTED_NAME | IVI_CLASS_ATTR_BASE + 281 |
| IVIRFSIGGEN_ATTR_LIST_SINGLE_STEP_ENABLED | IVI_CLASS_ATTR_BASE + 282 |
| IVIRFSIGGEN_ATTR_LIST_DWELL | IVI_CLASS_ATTR_BASE + 283 |
| IVIRFSIGGEN_ATTR_ALC_SOURCE | IVI_CLASS_ATTR_BASE + 301 |
| IVIRFSIGGEN_ATTR_ALC_BANDWIDTH | IVI_CLASS_ATTR_BASE + 302 |
| IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_SOURCE | IVI_CLASS_ATTR_BASE + 321 |
| IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_EXTERNAL_FREQUENCY | IVI_CLASS_ATTR_BASE + 322 |
| IVIRFSIGGEN_ATTR_IQ_ENABLED | IVI_CLASS_ATTR_BASE + 401 |
| IVIRFSIGGEN_ATTR_IQ_NOMINAL_VOLTAGE | IVI_CLASS_ATTR_BASE + 402 |
| IVIRFSIGGEN_ATTR_IQ_SOURCE | IVI_CLASS_ATTR_BASE + 403 |
| IVIRFSIGGEN_ATTR_IQ_SWAP_ENABLED | IVI_CLASS_ATTR_BASE + 404 |
| IVIRFSIGGEN_ATTR_IQ_IMPAIRMENT_ENABLED | IVI_CLASS_ATTR_BASE + 421 |
| IVIRFSIGGEN_ATTR_IQ_I_OFFSET | IVI_CLASS_ATTR_BASE + 422 |
| IVIRFSIGGEN_ATTR_IQ_Q_OFFSET | IVI_CLASS_ATTR_BASE + 423 |
| IVIRFSIGGEN_ATTR_IQ_RATIO | IVI_CLASS_ATTR_BASE + 424 |
| IVIRFSIGGEN_ATTR_IQ_SKEW | IVI_CLASS_ATTR_BASE + 425 |
| IVIRFSIGGEN_ATTR_ARB_SELECTED_WAVEFORM | IVI_CLASS_ATTR_BASE + 451 |
| IVIRFSIGGEN_ATTR_ARB_CLOCK_FREQUENCY | IVI_CLASS_ATTR_BASE + 452 |
| IVIRFSIGGEN_ATTR_ARB_FILTER_FREQUENCY | IVI_CLASS_ATTR_BASE + 453 |
| IVIRFSIGGEN_ATTR_ARB_MAX_NUMBER_WAVEFORMS | IVI_CLASS_ATTR_BASE + 454 |
| IVIRFSIGGEN_ATTR_ARB_WAVEFORM_QUANTUM | IVI_CLASS_ATTR_BASE + 455 |
| IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MIN | IVI_CLASS_ATTR_BASE + 456 |
| IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MAX | IVI_CLASS_ATTR_BASE + 457 |
| IVIRFSIGGEN_ATTR_ARB_TRIGGER_SOURCE | IVI_CLASS_ATTR_BASE + 458 |
| IVIRFSIGGEN_ATTR_ARB_EXTERNAL_TRIGGER_SLOPE | IVI_CLASS_ATTR_BASE + 459 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_STANDARD_COUNT | IVI_CLASS_ATTR_BASE + 501 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_SELECTED_STANDARD | IVI_CLASS_ATTR_BASE + 502 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_DATA_SOURCE | IVI_CLASS_ATTR_BASE + 503 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_PRBS_TYPE | IVI_CLASS_ATTR_BASE + 504 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_BIT_SEQUENCE | IVI_CLASS_ATTR_BASE + 505 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_CLOCK_SOURCE | IVI_CLASS_ATTR_BASE + 506 |
| IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE | IVI_CLASS_ATTR_BASE + 507 |
| IVIRFSIGGEN_ATTR_CDMA_STANDARD_COUNT | IVI_CLASS_ATTR_BASE + 601 |
| IVIRFSIGGEN_ATTR_CDMA_SELECTED_STANDARD | IVI_CLASS_ATTR_BASE + 602 |
| IVIRFSIGGEN_ATTR_CDMA_TRIGGER_SOURCE | IVI_CLASS_ATTR_BASE + 603 |
| IVIRFSIGGEN_ATTR_CDMA_EXTERNAL_TRIGGER_SLOPE | IVI_CLASS_ATTR_BASE + 604 |
| IVIRFSIGGEN_ATTR_CDMA_TEST_MODEL_COUNT | IVI_CLASS_ATTR_BASE + 605 |
| IVIRFSIGGEN_ATTR_CDMA_SELECTED_TEST_MODEL | IVI_CLASS_ATTR_BASE + 606 |
| IVIRFSIGGEN_ATTR_CDMA_CLOCK_SOURCE | IVI_CLASS_ATTR_BASE + 607 |
| IVIRFSIGGEN_ATTR_TDMA_STANDARD_COUNT | IVI_CLASS_ATTR_BASE + 701 |
| IVIRFSIGGEN_ATTR_TDMA_SELECTED_STANDARD | IVI_CLASS_ATTR_BASE + 702 |
| IVIRFSIGGEN_ATTR_TDMA_TRIGGER_SOURCE | IVI_CLASS_ATTR_BASE + 703 |
| IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_TRIGGER_SLOPE | IVI_CLASS_ATTR_BASE + 704 |
| IVIRFSIGGEN_ATTR_TDMA_FRAME_COUNT | IVI_CLASS_ATTR_BASE + 705 |
| IVIRFSIGGEN_ATTR_TDMA_SELECTED_FRAME | IVI_CLASS_ATTR_BASE + 706 |
| IVIRFSIGGEN_ATTR_TDMA_CLOCK_SOURCE | IVI_CLASS_ATTR_BASE + 707 |
| IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_CLOCK_TYPE | IVI_CLASS_ATTR_BASE + 708 |

# IviRFSigGen Attribute Value Definitions

This section specifies the actual value for each defined attribute value.

AM Scaling

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| AM Scaling Linear | .NET | AMScaling.Linear | 0 |
|  | C | IVIRFSIGGEN_VAL_AM_SCALING_LINEAR | 0 |
|  | COM | IviRFSigGenAMScalingLinear | 0 |
| AM Scaling Logarithmic | .NET | AMScaling.Logarithmic | 1 |
|  | C | IVIRFSIGGEN_VAL_AM_SCALING_LOGARITHMIC | 1 |
|  | COM | IviRFSigGenAMScalingLogarithmic | 1 |
| AM Scaling Class Ext Base | C | IVIRFSIGGEN_VAL_AM_SCALING_CLASS_EXT_BASE | 500 |
| AM Scaling Specific Ext Base | C | IVIRFSIGGEN_VAL_AM_SCALING_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

AM External Coupling

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| AM External Coupling AC | .NET | ExternalCoupling.AC | 0 |
|  | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_AC | 0 |
|  | COM | IviRFSigGenAMExternalCouplingAC | 0 |
| AM External Coupling DC | .NET | ExternalCoupling.DC | 1 |
|  | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_DC | 1 |
|  | COM | IviRFSigGenAMExternalCouplingDC | 1 |
| AM External Coupling Class Ext Base | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_CLASS_EXT_BASE | 500 |
| AM External Coupling Specific Ext Base | C | IVIRFSIGGEN_VAL_AM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

FM External Coupling

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| FM External Coupling AC | .NET | ExternalCoupling.AC | 0 |
|  | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_AC | 0 |
|  | COM | IviRFSigGenFMExternalCouplingAC | 0 |
| FM External Coupling DC | .NET | ExternalCoupling.DC | 1 |
|  | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_DC | 1 |
|  | COM | IviRFSigGenFMExternalCouplingDC | 1 |
| FM External Coupling Class Ext Base | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_CLASS_EXT_BASE | 500 |
| FM External Coupling Specific Ext Base | C | IVIRFSIGGEN_VAL_FM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

PM External Coupling

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| PM External Coupling AC | .NET | ExternalCoupling.AC | 0 |
|  | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_AC | 0 |
|  | COM | IviRFSigGenPMExternalCouplingAC | 0 |
| PM External Coupling DC | .NET | ExternalCoupling.DC | 1 |
|  | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_DC | 1 |
|  | COM | IviRFSigGenPMExternalCouplingDC | 1 |
| PM External Coupling Class Ext Base | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_CLASS_EXT_BASE | 500 |
| PM External Coupling Specific Ext Base | C | IVIRFSIGGEN_VAL_PM_EXTERNAL_COUPLING_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Pulse Modulation Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Pulse Modulation Source Internal | .NET | PulseModulationSource.Internal | 0 |
|  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenPulseModulationSourceInternal | 0 |
| Pulse Modulation Source External | .NET | PulseModulationSource.External | 1 |
|  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenPulseModulationSourceExternal | 1 |
| Pulse Modulation Source Class Ext Base | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_CLASS_EXT_BASE | 500 |
| Pulse Modulation Source Specific Ext Base | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Pulse Modulation External Polarity

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Pulse Modulation External Polarity Normal | .NET | PulseModulationExternalPolarity.Normal | 0 |
|  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_NORMAL | 0 |
|  | COM | IviRFSigGenPulseModulationExternalPolarityNormal | 0 |
| Pulse Modulation External Polarity Inverse | .NET | PulseModulationExternalPolarity.Inverse | 1 |
|  | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_INVERSE | 1 |
|  | COM | IviRFSigGenPulseModulationExternalPolarityInverse | 1 |
| Pulse Modulation External Polarity Class Ext Base | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_CLASS_EXT_BASE | 500 |
| Pulse Modulation External Polarity Specific Ext Base | C | IVIRFSIGGEN_VAL_PULSE_MODULATION_EXTERNAL_POLARITY_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

LFGenerator Waveform

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| LFGenerator Waveform Sine | .NET | LFGeneratorWaveform.Sine | 0 |
|  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SINE | 0 |
|  | COM | IviRFSigGenLFGeneratorWaveformSine | 0 |
| LFGenerator Waveform Square | .NET | LFGeneratorWaveform.Square | 1 |
|  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SQUARE | 1 |
|  | COM | IviRFSigGenLFGeneratorWaveformSquare | 1 |
| LFGenerator Waveform Triangle | .NET | LFGeneratorWaveform.Triangle | 2 |
|  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_TRANGLE | 2 |
|  | COM | IviRFSigGenLFGeneratorWaveformTriangle | 2 |
| LFGenerator Waveform Ramp Up | .NET | LFGeneratorWaveform.RampUp | 3 |
|  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_RAMP_UP | 3 |
|  | COM | IviRFSigGenLFGeneratorWaveformRampUp | 3 |
| LFGenerator Waveform Ramp Down | .NET | LFGeneratorWaveform.RampDown | 4 |
|  | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_RAMP_DOWN | 4 |
|  | COM | IviRFSigGenLFGeneratorWaveformRampDown | 4 |
| LFGenerator Waveform Class Ext Base | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_CLASS_EXT_BASE | 500 |
| LFGenerator Waveform Specific Ext Base | C | IVIRFSIGGEN_VAL_LF_GENERATOR_WAVEFORM_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Pulse Trigger Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Pulse Trigger Source Internal | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenPulseTriggerSourceInternal | 0 |
| Pulse Trigger Source External | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenPulseTriggerSourceExternal | 1 |
| Pulse Trigger Source Class Ext Base | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_CLASS_EXT_BASE | 500 |
| Pulse Trigger Source Specific Ext Base | C | IVIRFSIGGEN_VAL_PULSE_TRIGGER_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Pulse External Trigger Slope

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Pulse External Trigger Slope Positive | .NET | Slope.Positive | 0 |
|  | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_POSITIVE | 0 |
|  | COM | IviRFSigGenPulseExternalTriggerSlopePositive | 0 |
| Pulse External Trigger Slope Negative | .NET | Slope.Negative | 1 |
|  | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_NEGATIVE | 1 |
|  | COM | IviRFSigGenPulseExternalTriggerSlopeNegative | 1 |
|  | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE | 500 |
| Pulse External Trigger Slope Class Ext Base | C | IVIRFSIGGEN_VAL_PULSE_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE | 1000 |
| Pulse External Trigger Slope Specific Ext Base | COM |  | 1000 |

Pulse Output Polarity

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Pulse Output Polarity Normal | .NET | PulseOutputPolarity.Normal | 0 |
|  | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_NORMAL | 0 |
|  | COM | IviRFSigGenPulseOutputPolarityNormal | 0 |
| Pulse Output Polarity Inverse | .NET | PulseOutputPolarity.Inverse | 1 |
|  | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_INVERSE | 1 |
|  | COM | IviRFSigGenPulseOutputPolarityInverse | 1 |
| Pulse Output Polarity Class Ext Base | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_CLASS_EXT_BASE | 500 |
| Pulse Output Polarity Specific Ext Base | C | IVIRFSIGGEN_VAL_PULSE_OUTPUT_POLARITY_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Sweep Mode

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Sweep Mode None | .NET | SweepMode.None | 0 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_NONE | 0 |
|  | COM | IviRFSigGenSweepModeNone | 0 |
| Sweep Mode Fequency Sweep | .NET | SweepMode.FrequencySweep | 1 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_FREQUENCY_SWEEP | 1 |
|  | COM | IviRFSigGenSweepModeFrequencySweep | 1 |
| Sweep Mode Power Sweep | .NET | SweepMode.PowerSweep | 2 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_POWER_SWEEP | 2 |
|  | COM | IviRFSigGenSweepModePowerSweep | 2 |
| Sweep Mode Frequeny Step | .NET | SweepMode.FrequencyStep | 3 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_FREQUENCY_STEP | 3 |
|  | COM | IviRFSigGenSweepModeFrequencyStep | 3 |
| Sweep Mode Power Step | .NET | SweepMode.PowerStep | 4 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_POWER_STEP | 4 |
|  | COM | IviRFSigGenSweepModePowerStep | 4 |
| Sweep Mode List | .NET | SweepMode.List | 5 |
|  | C | IVIRFSIGGEN_VAL_SWEEP_MODE_LIST | 5 |
|  | COM | IviRFSigGenSweepModeList | 5 |
| Sweep Mode Class Ext Base | C | IVIRFSIGGEN_VAL_SWEEP_MODE_CLASS_EXT_BASE | 500 |
| Sweep Mode Specific Ext Base | C | IVIRFSIGGEN_VAL_SWEEP_MODE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Sweep Trigger Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Sweep Trigger Source Immediate | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_IMMEDIATE | 0 |
|  | COM | IviRFSigGenSweepTriggerSourceImmediate | 0 |
| Sweep Trigger Source External | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenSweepTriggerSourceExternal | 1 |
| Sweep Trigger Source Software | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SOFTWARE | 2 |
|  | COM | IviRFSigGenSweepTriggerSourceSoftware | 2 |
| Sweep Trigger Source Class Ext Base | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_CLASS_EXT_BASE | 500 |
| Sweep Trigger Source Specific Ext Base | C | IVIRFSIGGEN_VAL_SWEEP_TRIGGER_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Frequency Step Scaling

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Frequency Step Scaling Linear | .NET | FrequencyStepScaling.Linear | 0 |
|  | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_LINEAR | 0 |
|  | COM | IviRFSigGenFrequencyStepScalingLinear | 0 |
| Frequency Step Scaling Logarithmic | .NET | FrequencyStepScaling.Logarithmic | 1 |
|  | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_LOGARITHMIC | 1 |
|  | COM | IviRFSigGenFrequencyStepScalingLogarithmic | 1 |
| Frequency Step Scaling Class Ext Base | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_CLASS_EXT_BASE | 500 |
| Frequency Step Scaling Specific Ext Base | C | IVIRFSIGGEN_VAL_FREQUENCY_STEP_SCALING_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

ALC Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| ALC Source Internal | C | IVIRFSIGGEN_VAL_ALC_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenALCSourceInternal | 0 |
| ALC Source External | C | IVIRFSIGGEN_VAL_ALC_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenALCSourceExternal | 1 |
| ALC Source Class Ext Base | C | IVIRFSIGGEN_VAL_ALC_SOURCE_CLASS_EXT_BASE | 500 |
| ALC Source Specific Ext Base | C | IVIRFSIGGEN_VAL_ALC_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Reference Oscillator Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Reference Oscillator Source Internal | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenReferenceOscillatorSourceInternal | 0 |
| Reference Oscillator Source External | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenReferenceOscillatorSourceExternal | 1 |
| Reference Oscillator Source Class Ext Base | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_CLASS_EXT_BASE | 500 |
| Reference Oscillator Source Specific Ext Base | C | IVIRFSIGGEN_VAL_REFERENCE_OSCILLATOR_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

IQ Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| IQ Source DigitalModulation Base | .NET | IQSource.DigitalModulationBase | 0 |
|  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_DIGITAL_MODULATION_BASE | 0 |
|  | COM | IviRFSigGenIQSourceDigitalModulationBase | 0 |
| IQ Source CDMA Base | .NET | IQSource.CdmaBase | 1 |
|  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_CDMA_BASE | 1 |
|  | COM | IviRFSigGenIQSourceCDMABase | 1 |
| IQ Source TDMA Base | .NET | IQSource.TdmaBase | 2 |
|  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_TDMA_BASE | 2 |
|  | COM | IviRFSigGenIQSourceTDMABase | 2 |
| IQ Source Arb Generator | .NET | IQSource.ArbGenerator | 3 |
|  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_ARB_GENERATOR | 3 |
|  | COM | IviRFSigGenIQSourceArbGenerator | 3 |
| IQ Source External | .NET | IQSource.External | 4 |
|  | C | IVIRFSIGGEN_VAL_IQ_SOURCE_EXTERNAL | 4 |
|  | COM | IviRFSigGenIQSourceExternal | 4 |
| IQ Source Class Ext Base | C | IVIRFSIGGEN_VAL_IQ_SOURCE_CLASS_EXT_BASE | 500 |
| IQ Source Specific Ext Base | C | IVIRFSIGGEN_VAL_IQ_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Arb Trigger Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Arb Trigger Source Immediate | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_IMMEDIATE | 0 |
|  | COM | IviRFSigGenArbTriggerSourceImmediate | 0 |
| Arb Trigger Source External | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenArbTriggerSourceExternal | 1 |
| Arb Trigger Source Software | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SOFTWARE | 2 |
|  | COM | IviRFSigGenArbTriggerSourceSoftware | 2 |
| Arb Trigger Source Class Ext Base | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_CLASS_EXT_BASE | 500 |
| Arb Trigger Source Specific Ext Base | C | IVIRFSIGGEN_VAL_ARB_TRIGGER_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Arb External Trigger Slope

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Arb External Trigger Slope Positive | .NET | Slope.Positive | 0 |
|  | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_POSITIVE | 0 |
|  | COM | IviRFSigGenArbExternalTriggerSlopePositive | 0 |
| Arb External Trigger Slope Negative | .NET | Slope.Negative | 1 |
|  | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_NEGATIVE | 1 |
|  | COM | IviRFSigGenArbExternalTriggerSlopeNegative | 1 |
| Arb External Trigger Slope Class Ext Base | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE | 500 |
| Arb External Trigger Slope Specific Ext Base | C | IVIRFSIGGEN_VAL_ARB_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Digital Modulation Base Data Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Digital Modulation Base Data Source External | .NET | DigitalModulationBaseDataSource.External | 0 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_EXTERNAL | 0 |
|  | COM | IviRFSigGenDigitalModulationBaseDataExternal | 0 |
| Digital Modulation Base Data Source PRBS | .NET | DigitalModulationBaseDataSource.Prbs | 1 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_PRBS | 1 |
|  | COM | IviRFSigGenDigitalModulationBaseDataPRBS | 1 |
| Digital Modulation Base Data Source Bit Sequence | .NET | DigitalModulationBaseDataSource.BitSequence | 2 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_BIT_SEQUENCE | 2 |
|  | COM | IviRFSigGenDigitalModulationBaseDataBitSequence | 2 |
| Digital Modulation Base Data Source Class Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_CLASS_EXT_BASE | 500 |
| Digital Modulation Base Data Source Specific Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_DATA_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Digital Modulation Base PRBS Type

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Digital Modulation Base PRBS Type PRBS9 | .NET | DigitalModulationBasePrbsType.Prbs9 | 0 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS9 | 0 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS9 | 0 |
| Digital Modulation Base PRBS Type PRBS11 | .NET | DigitalModulationBasePrbsType.Prbs11 | 1 |
|  | C | IVIRFSIGGEN_VAL_ DIGITAL_MODULATION_BASE PRBS_TYPE_PRBS11 | 1 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS11 | 1 |
| Digital Modulation Base PRBS Type PRBS15 | .NET | DigitalModulationBasePrbsType.Prbs15 | 2 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS15 | 2 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS15 | 2 |
| Digital Modulation Base PRBS Type PRBS16 | .NET | DigitalModulationBasePrbsType.Prbs16 | 3 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS16 | 3 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS16 | 3 |
| Digital Modulation Base PRBS Type PRBS20 | .NET | DigitalModulationBasePrbsType.Prbs20 | 4 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS20 | 4 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS20 | 4 |
| Digital Modulation Base PRBS Type PRBS21 | .NET | DigitalModulationBasePrbsType.Prbs21 | 5 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS21 | 5 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS21 | 5 |
| Digital Modulation Base PRBS Type PRBS23 | .NET | DigitalModulationBasePrbsType.Prbs23 | 6 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_PRBS23 | 6 |
|  | COM | IviRFSigGenDigitalModulationBasePRBS23 | 6 |
| Digital Modulation Base PRBS Type Class Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_CLASS_EXT_BASE | 500 |
| Digital Modulation Base PRBS Type Specific Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_PRBS_TYPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Digital Modulation Base Clock Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Digital Modulation Base Clock Source Internal | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenDigitalModulationBaseClockSourceInternal | 0 |
| Digital Modulation Base Clock Source External | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenDigitalModulationBaseClockSourceExternal | 1 |
| Digital Modulation Base Clock Source Class Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_CLASS_EXT_BASE | 500 |
| Digital Modulation Base Clock Source Specific Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_CLOCK_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

Digital Modulation Base External Clock Type

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| Digital Modulation Base External Clock Type Bit | .NET | DigitalModulationBaseExternalClockType.Bit | 0 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_BIT | 0 |
|  | COM | IviRFSigGenDigitalModulationBaseExternalClockTypeBit | 0 |
| Digital Modulation Base External Clock Type Symbol | .NET | DigitalModulationBaseExternalClockType.Symbol | 1 |
|  | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SYMBOL | 1 |
|  | COM | IviRFSigGenDigitalModulationBaseExternalClockTypeSymbol | 1 |
| Digital Modulation Base External Clock Type Class Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE | 500 |
| Digital Modulation Base External Clock Type Specific Ext Base | C | IVIRFSIGGEN_VAL_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

CDMA Trigger Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| CDMA Trigger Source Immediate | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_IMMEDIATE | 0 |
|  | COM | IviRFSigGenCDMATriggerSourceImmediate | 0 |
| CDMA Trigger Source External | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenCDMATriggerSourceExternal | 1 |
| CDMA Trigger Source Software | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SOFTWARE | 2 |
|  | COM | IviRFSigGenCDMATriggerSourceSoftware | 2 |
| CDMA Trigger Source Class Ext Base | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_CLASS_EXT_BASE | 500 |
| CDMA Trigger Source Specific Ext Base | C | IVIRFSIGGEN_VAL_CDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

CDMA External Trigger Slope

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| CDMA External Trigger Slope Positive | .NET | Slope.Positive | 0 |
|  | C | IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_POSITIVE | 0 |
|  | COM | IviRFSigGenCDMAExternalTriggerSlopePositive | 0 |
| CDMA External Trigger Slope Negative | .NET | Slope.Negative | 1 |
|  | C | IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_NEGATIVE | 1 |
|  | COM | IviRFSigGenCDMAExternalTriggerSlopeNegative | 1 |
| CDMA External Trigger Slope Class Ext Base | C | IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE | 500 |
| CDMA External Trigger Slope Specific Ext Base | C | IVIRFSIGGEN_VAL_CDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

CDMA Clock Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| CDMA Clock Source Internal | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenCDMAClockSourceInternal | 0 |
| CDMA Clock Source External | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenCDMAClockSourceExternal | 1 |
| CDMA Clock Source Class Ext Base | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_CLASS_EXT_BASE | 500 |
| CDMA Clock Source Specific Ext Base | C | IVIRFSIGGEN_VAL_CDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

TDMA Trigger Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| TDMA Trigger Source Immediate | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_IMMEDIATE | 0 |
|  | COM | IviRFSigGenTDMATriggerSourceImmediate | 0 |
| TDMA Trigger Source External | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenTDMATriggerSourceExternal | 1 |
| TDMA Trigger Source Software | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SOFTWARE | 2 |
|  | COM | IviRFSigGenTDMATriggerSourceSoftware | 2 |
| TDMA Trigger Source Class Ext Base | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_CLASS_EXT_BASE | 500 |
| TDMA Trigger Source Specific Ext Base | C | IVIRFSIGGEN_VAL_TDMA_TRIGGER_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

TDMA External Trigger Slope

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| TDMA External Trigger Slope Positive | .NET | Slope.Positive | 0 |
|  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_POSITIVE | 0 |
|  | COM | IviRFSigGenTDMAExternalTriggerSlopePositive | 0 |
| TDMA External Trigger Slope Negative | .NET | Slope.Negative | 1 |
|  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_NEGATIVE | 1 |
|  | COM | IviRFSigGenTDMAExternalTriggerSlopeNegative | 1 |
| TDMA External Trigger Slope Class Ext Base | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_CLASS_EXT_BASE | 500 |
| TDMA External Trigger Slope Specific Ext Base | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_TRIGGER_SLOPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

TDMA Clock Source

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| TDMA Clock Source Internal | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_INTERNAL | 0 |
|  | COM | IviRFSigGenTDMAClockSourceInternal | 0 |
| TDMA Clock Source External | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_EXTERNAL | 1 |
|  | COM | IviRFSigGenTDMAClockSourceExternal | 1 |
| TDMA Clock Source Class Ext Base | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_CLASS_EXT_BASE | 500 |
| TDMA Clock Source Specific Ext Base | C | IVIRFSIGGEN_VAL_TDMA_CLOCK_SOURCE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

TDMA External Clock Type

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| TDMA External Clock Type Bit | .NET | TdmaExternalClockType.Bit | 0 |
|  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_BIT | 0 |
|  | COM | IviRFSigGenTDMAExternalClockTypeBit | 0 |
| TDMA External Clock Type Symbol | .NET | TdmaExternalClockType.Symbol | 1 |
|  | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SYMBOL | 1 |
|  | COM | IviRFSigGenTDMAExternalClockTypeSymbol | 1 |
| TDMA External Clock Type Class Ext Base | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_CLASS_EXT_BASE | 500 |
| TDMA External Clock Type Specific Ext Base | C | IVIRFSIGGEN_VAL_TDMA_EXTERNAL_CLOCK_TYPE_SPECIFIC_EXT_BASE | 1000 |
|  | COM |  | 1000 |

# IviRFSigGen Function Parameter Value Definitions

This section specifies the actual values for each function parameter that defines values.

- Wait Until Settled

| Parameter: MaxTimeMilleseconds |  |  |  |
| --- | --- | --- | --- |
| Value Name | Language | Identifier | Actual Value |
| Max Time Immediate | C | IVIRFSIGGEN_VAL_MAX_TIME_IMMEDIATE | 0x0 |
|  | COM | IviRFSigGenMaxTimeImmediate | 0x0 |
| Max Time Infinite | C | IVIRFSIGGEN_VAL_MAX_TIME_INFINITE | 0xFFFFFFFFUL |
|  | COM | IviRFSigGenMaxTimeInfinite | 0xFFFFFFFFUL |

| Parameter: MaximumTime |  |  |  |
| --- | --- | --- | --- |
| Value Name | Language | Identifier | Actual Value |
| Max Time Immediate | .NET | PrecisionTimeSpan.Zero | PrecisionTimeSpan.Zero |
| Max Time Infinite | .NET | PrecisionTimeSpan.MaxValue | PrecisionTimeSpan.MaxValue |

# IviRFSigGen Error and Completion Code Value Definitions

Table 33-3 specifies the actual value for each status code that the IviRFSigGen class specification defines.

| Table 33-3 IviRFSigGen Completion Codes |  |  |  |
| --- | --- | --- | --- |
| Error Name | Description |  |  |
|  | Language | Identifier | Value(hex) |
| Trigger Not Software | The trigger source is not set to software trigger. |  |  |
|  | .NET | Ivi.Driver.TriggerNotSoftwareException | IVI Defined Exception (See IVI-3.2) |
|  | C | IVIRFSIGGEN_ERROR_TRIGGER_NOT_SOFTWARE | 0xBFFA1001 |
|  | COM | E_IVIRFSIGGEN_TRIGGER_NOT_SOFTWARE | 0x80041001 |
| Max Time Exceeded | Maximum time exceeded before the operation completed. |  |  |
|  | .NET | Ivi.Driver.MaxTimeExceededException | IVI Defined Exception (See IVI-3.2) |
|  | C | IVIRFSIGGEN_ERROR_MAX_TIME_EXCEEDED | 0xBFFA2003 |
|  | COM | E_IVIRFSIGGEN_MAX_TIME_EXCEEDED | 0x80042003 |
| List Unknown | The selected list is not defined. |  |  |
|  | .NET | ListUnknownException |  |
|  | C | IVIRFSIGGEN_ERROR_LIST_UNKNOWN | 0xBFFA200A |
|  | COM | E_IVIRFSIGGEN_LIST_UNKNOWN | 0x8004200A |

Table 33-2 defines the recommended format of the message string associated with the errors. In C, these strings are returned by the Get Error function. In COM, these strings are the description contained in the ErrorInfo object.  For .NET, exception default message strings are defined with the exception.

Note: In the description string table entries listed below, %s is always used to represent the component name.

| Table 33-4. IviRFSigGen Error Message Strings |  |
| --- | --- |
| Name | Message String |
| Trigger Not Software | “%s: Trigger source is not set to software trigger” |
| Max Time Exceeded | “%s: Max Time Exceeded” |
| List Unknown | “%s: List Unknown” |

## IVI.NET IviRFSigGen Exceptions and Warnings

This section defines the list of IVI.NET exceptions and warnings that are specific to the IviRFSigGen class. For general information on IVI.NET exceptions and warnings, refer to IVI-3.1: Driver Architecture Specification and section 12, Common IVI.NET Exceptions and Warnings, of IVI-3.2: Inherent Capabilities Specification.

The IVI.NET exceptions defined in this specification are declared in the Ivi.RFSigGen namespace.

- ListUnknownException

### ListUnknownException

Description

This exception is used when the driver finds that a specified list is not defined.

Constructors

Ivi.RFSigGen.FrequencyListUnknownException(String message,

String listName);

Ivi.RFSigGen.FrequencyListUnknownException();

Ivi.RFSigGen.FrequencyListUnknownException(String message);

Ivi.RFSigGen.FrequencyListUnknownException(String message,

System.Exception innerException);

Message String

The selected list is not defined.

List name: <listName>

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| listName | The name of the list that is undefined. | String |

Usage

If driver developers use constructors that take a message string, they are responsible for message string localization.

# IviRFSigGen Hierarchies

## IviRFSigGen .NET Hierarchy

| Table 34-5. IviRFSigGen .NET Hierarchy |  |  |
| --- | --- | --- |
| .NET Interface Hierarchy | Generic Name | Type |
| SendSoftwareTrigger | Send Software Trigger | M |
| RF |  |  |
| Configure | Configure RF | M |
| IsSettled | Is Settled | M |
| WaitUntilSettled | Wait Until Settled | M |
| DisableAllModulation | Disable All Modulation | M |
| Frequency | Frequency | P |
| Level | Power Level | P |
| OutputEnabled | Output Enabled | P |
| AnalogModulation |  |  |
| Source |  |  |
| GetName | Analog Modulation Source Name | M |
| Count | Analog Modulation Source Count | P |
| AM |  |  |
| Configure | Configure AM | M |
| Enabled | AM Enabled | P |
| Source | AM Source | P |
| Scaling | AM Scaling | P |
| ExternalCoupling | AM External Coupling | P |
| NominalVoltage | AM Nominal Voltage | P |
| Depth | AM Depth | P |
| FM |  |  |
| Configure | Configure FM | M |
| Enabled | FM Enabled | P |
| Source | FM Source | P |
| ExternalCoupling | FM External Coupling | P |
| NominalVoltage | FM Nominal Voltage | P |
| Deviation | FM Deviation | P |
| PM |  |  |
| Configure | Configure PM | M |
| Enabled | PM Enabled | P |
| Source | PM Source | P |
| ExternalCoupling | PM External Coupling | P |
| NominalVoltage | PM Nominal Voltage | P |
| Deviation | PM Deviation | P |
| PulseModulation |  |  |
| Enabled | Pulse Modulation Enabled | P |
| Source | Pulse Modulation Source | P |
| ExternalPolarity | Pulse Modulation External Polarity | P |
| LFGenerator |  |  |
| Configure | Configure LF Generator | M |
| ActiveLFGenerator | Active LFGenerator | P |
| GetName | LFGenerator Name | M |
| Count | LFGenerator Count | P |
| Frequency | LFGenerator Frequency | P |
| Waveform | LFGenerator Waveform | P |
| Output |  |  |
| Configure | Configure LFGenerator Output | M |
| Enabled | LFGenerator Output Enabled | P |
| Amplitude | LFGenerator Output Amplitude | P |
| PulseGenerator |  |  |
| Configure | Configure Pulse | M |
| ConfigureExternalTrigger | Configure Pulse External Trigger | M |
| InternalTriggerPeriod | Pulse Internal Trigger Period | P |
| Width | Pulse Width | P |
| GatingEnabled | Pulse Gating Enabled | P |
| TriggerSource | Pulse Trigger Source | P |
| ExternalTriggerSlope | Pulse External Trigger Slope | P |
| ExternalTriggerDelay | Pulse External Trigger Delay | P |
| DoublePulse |  |  |
| Configure | Configure Pulse Double | M |
| Enabled | Pulse Double Enabled | P |
| Delay | Pulse Double Delay | P |
| Output |  |  |
| Configure | Configure Pulse Output | M |
| Polarity | Pulse Output Polarity | P |
| Enabled | Pulse Output Enabled | P |
| Sweep |  |  |
| Configure | Configure Sweep | M |
| Mode | Sweep Mode | P |
| TriggerSource | Sweep Trigger Source | P |
| FrequencySweep |  |  |
| ConfigureStartStop | Configure Frequency Sweep Start Stop | M |
| ConfigureCenterSpan | Configure Frequency Sweep Center Span | M |
| Start | Frequency Sweep Start | P |
| Stop | Frequency Sweep Stop | P |
| Time | Frequency Sweep Time | P |
| PowerSweep |  |  |
| ConfigureStartStop | Configure Power Sweep Start Stop | M |
| Start | Power Sweep Start | P |
| Stop | Power Sweep Stop | P |
| Time | Power Sweep Time | P |
| FrequencyStep |  |  |
| ConfigureStartStop | Configure Frequency Step Start Stop | M |
| ConfigureDwell | Configure Frequency Step Dwell | M |
| Reset | Frequency Step Reset | M |
| Start | Frequency Step Start | P |
| Stop | Frequency Step Stop | P |
| Scaling | Frequency Step Scaling | P |
| Size | Frequency Step Size | P |
| SingleStepEnabled | Frequency Step Single Step Enabled | P |
| Dwell | Frequency Step Swell | P |
| PowerStep |  |  |
| ConfigureStartStop | Configure Power Step Start Stop | M |
| ConfigureDwell | Configure Power Step Dwell | M |
| Reset | Power Step Reset | M |
| Start | Power Step Start | P |
| Stop | Power Step Stop | P |
| Size | Power Step Size | P |
| SingleStepEnabled | Power Step Single Step Enabled | P |
| Dwell | Power Step Dwell | P |
| List |  |  |
| CreateFrequency | Create Frequency List | M |
| CreatePower | Create Power List | M |
| CreateFrequencyPower | Create Frequency Power List | M |
| ClearAll | Clear All List | M |
| ConfigureDwell | Configure List Dwell | M |
| Reset | List Reset | M |
| SelectedName | List Selected Name | P |
| SingleStepEnabled | List Single Step Enabled | P |
| Dwell | List Dwell | P |
| Alc |  |  |
| Configure | Configure ALC | M |
| Enabled | ALC Enabled | P |
| Source | ALC Source | P |
| Bandwidth | ALC Bandwidth | P |
| ReferenceOscillator |  |  |
| Configure | Configure Reference Oscillator | M |
| Source | Reference Oscillator Source | P |
| ExternalFrequency | Reference Oscillator External Frequency | P |
| IQ |  |  |
| Configure | Configure IQ | M |
| Calibrate | Calibrate IQ | M |
| Enabled | IQ Enabled | P |
| Source | IQ Source | P |
| NominalVoltage | IQ Nominal Voltage | P |
| SwapEnabled | IQ Swap Enabled | P |
| Impairment |  |  |
| Configure | Configure IQ Impairment | M |
| Enabled | IQ Impairment Enabled | P |
| IOffset | IQ I-Offset | P |
| QOffset | IQ Q-Offset | P |
| Ratio | IQ Ratio | P |
| Skew | IQ Skew | P |
| DigitalModulation |  |  |
| Arb |  |  |
| Configure | Configure Arb | M |
| WriteWaveform | Write Arb Waveform | M |
| ClearAllWaveforms | Clear All Arb Waveforms | M |
| SelectedWaveform | Arb Selected Waveform | P |
| ClockFrequency | Arb Clock Frequency | P |
| FilterFrequency | Arb Filter Frequency | P |
| MaxNumberWaveforms | Arb Max Number Waveforms | P |
| WaveformQuantum | Arb Waveform Quantum | P |
| WaveformSizeMin | Arb Waveform Size Min | P |
| WaveformSizeMax | Arb Waveform Size Max | P |
| TriggerSource | Arb Trigger Source | P |
| ExternalTriggerSlope | Arb External Trigger Slope | P |
| Base |  |  |
| StandardNames | DigitalModulationBase Standard Names | M |
| StandardCount | DigitalModulationBase Standard Count | P |
| ConfigureClockSource | Configure DigitalModulationBase Clock Source | M |
| WriteBitSequence | Write DigitalModulationBase Bit Sequence | M |
| ClearAllBitSequences | Clear DigitalModulationBase Bit Sequence | M |
| SelectedStandard | DigitalModulationBase Selected Standard | P |
| DataSource | DigitalModulationBase Data Source | P |
| PrbsType | DigitalModulationBase PRBS TYpe | P |
| SelectedBitSequence | DigitalModulationBase Selected Bit Sequence | P |
| ClockSource | DigitalModulationBase Clock Source | P |
| ExternalClockType | DigitalModulationBase External Clock Type | P |
| Cdma |  |  |
| StandardNames | CDMA Standard Names | M |
| StandardCount | CDMA Standard Count | P |
| TestModelNames | CDMA Test Model Names | M |
| TestModelCount | CDMA Test Model Count | P |
| SelectedStandard | CDMA Selected Standard | P |
| TriggerSource | CDMA Trigger Source | P |
| ExternalTriggerSlope | CDMA External Trigger Slope | P |
| SelectedTestModel | CDMA Selected Test Model | P |
| ClockSource | CDMA Clock Source | P |
| Tdma |  |  |
| StandardNames | TDMA Standard Names | M |
| StandardCount | TDMA Standard Count | P |
| ConfigureClockSource | Configure TDMA Clock Source | M |
| FrameNames | TDMA Frame Names | M |
| FrameCount | TDMA Frame Count | P |
| SelectedStandard | TDMA Selected Standard | P |
| TriggerSource | TDMA Trigger Source | P |
| ExternalTriggerSlope | TDMA External Trigger Slope | P |
| SelectedFrame | TDMA Selected Frame | P |
| ClockSource | TDMA Clock Source | P |
| ExternalClockType | TDMA External Clock Type | P |

### IviRFSigGen COM Interfaces

In addition to implementing IVI inherent capabilities interfaces, IviRFSigGen interfaces contain interface reference properties for accessing the following IviRFSigGen interfaces:

- IIviRFSigGenRF

- IIviRFSigGenAnalogModulation

- IIviRFSigGenPulseModulation

- IIviRFSigGenLFGenerator

- IIviRFSigGenPulseGenerator

- IIviRFSigGenSweep

- IIviRFSigGenAlc

- IIviRFSigGenReferenceOscillator

- IIviRFSigGenIQ

- IiviRFSigGenDigitalModulation

The IIviRFSigGenAnalogModulation interface contains interface reference properties for accessing additional the following IviRFSigGen AnalogModulation interfaces:

- IIviRFSigGenAnalogModulationSource

- IIviRFSigGenAM

- IIviRFSigGenFM

- IIviRFSigGenPM

The IIviRFSigGenLFGenerator interface contains interface reference properties for accessing additional the following IviRFSigGen LFGenerator interfaces:

- IIviRFSigGenLFGeneratorOutput

The IIviRFSigGenPulseGenerator interface contains interface reference properties for accessing additional the following IviRFSigGen PulseGenerator interfaces:

- IIviRFSigGenPulseGeneratorDouble

- IIviRFSigGenPulseGeneratorOutput

The IIviRFSigGenSweep interface contains interface reference properties for accessing additional the following IviRFSigGen Sweep interfaces:

- IIviRFSigGenFrequencySweep

- IIviRFSigGenPowerSweep

- IIviRFSigGenFrequencyStep

- IIviRFSigGenPowerStep

- IIviRFSigGenList

The IIviRFSigGenIQ interface contains interface reference properties for accessing additional the following IviRFSigGen IQ interfaces:

- IIviRFSigGenIQImpairment

The IIviRFSigGenDigitalModulation interface contains interface reference properties for accessing additional the following IviRFSigGen DigitalModulation interfaces:

- IIviRFSigGenArb

- IIviRFSigGenDigitalModulationBase

- IIviRFSigGenCdma

- IIviRFSigGenTdma

### .NET Interface Reference Properties

Interface reference properties are used to navigate the IviRFSigGen .NET hierarchy. This section describes the interface reference properties that the IviRFSigGen interfaces define.  All interface reference properties are read-only.

| Table 34-6. IviRFSigGen Interface Reference Properties |  |
| --- | --- |
| Interface | Reference Property Name |
| IIviRFSigGenRF | RF |
| IIviRFSigGenAlc | Alc |
| IIviRFSigGenReferenceOscillator | ReferenceOscillator |
| IIviRFSigGenLFGenerator | LFGenerator |
| IIviRFSigGenLFGeneratorOutput | LFGenerator.Output |
| IIviRFSigGenPulseGenerator | PulseGenerator |
| IIviRFSigGenPulseGeneratorDouble | PulseGenerator.DoublePulse |
| IIviRFSigGenPulseGeneratorOutput | PulseGenerator.Output |
| IIviRFSigGenAnalogModulation | AnalogModulation |
| IIviRFSigGenAnalogModulationSource | AnalogModulation.Source |
| IIviRFSigGenAM | AnalogModulation.AM |
| IIviRFSigGenFM | AnalogModulation.FM |
| IIviRFSigGenPM | AnalogModulation.PM |
| IIviRFSigGenPulseModulation | PulseModulation |
| IIviRFSigGenSweep | Sweep |
| IIviRFSigGenFrequencySweep | Sweep.FrequencySweep |
| IIviRFSigGenPowerSweep | Sweep.PowerSweep |
| IIviRFSigGenFrequencyStep | Sweep.FrequencyStep |
| IIviRFSigGenPowerStep | Sweep.PowerStep |
| IIviRFSigGenList | Sweep.List |
| IIviRFSigGenIQ | IQ |
| IIviRFSigGenIQImpairment | IQ.Impairment |
| IIviRFSigGenDigitalModulation | DigitalModulation |
| IIviRFSigGenArb | DigitalModulation.Arb |
| IIviRFSigGenDigitalModulationBase | DigitalModulation.Base |
| IIviRFSigGenCdma | DigitalModulation.Cdma |
| IIviRFSigGenTdma | DigitalModulation.Tdma |

## IviRFSigGen COM Hierarchy

| Table 34-7. IviRFSigGen COM Hierarchy |  |  |
| --- | --- | --- |
| COM Interface Hierarchy | Generic Name | Type |
| SendSoftwareTrigger | Send Software Trigger | M |
| RF |  |  |
| Configure | Configure RF | M |
| IsSettled | Is Settled | M |
| WaitUntilSettled | Wait Until Settled | M |
| DisableAllModulation | Disable All Modulation | M |
| Frequency | Frequency | P |
| Level | Power Level | P |
| OutputEnabled | Output Enabled | P |
| AnalogModulation |  |  |
| Source |  |  |
| Name | Analog Modulation Source Name | P |
| Count | Analog Modulation Source Count | P |
| AM |  |  |
| Configure | Configure AM | M |
| Enabled | AM Enabled | P |
| Source | AM Source | P |
| Scaling | AM Scaling | P |
| ExternalCoupling | AM External Coupling | P |
| NominalVoltage | AM Nominal Voltage | P |
| Depth | AM Depth | P |
| FM |  |  |
| Configure | Configure FM | M |
| Enabled | FM Enabled | P |
| Source | FM Source | P |
| ExternalCoupling | FM External Coupling | P |
| NominalVoltage | FM Nominal Voltage | P |
| Deviation | FM Deviation | P |
| PM |  |  |
| Configure | Configure PM | M |
| Enabled | PM Enabled | P |
| Source | PM Source | P |
| ExternalCoupling | PM External Coupling | P |
| NominalVoltage | PM Nominal Voltage | P |
| Deviation | PM Deviation | P |
| PulseModulation |  |  |
| Enabled | Pulse Modulation Enabled | P |
| Source | Pulse Modulation Source | P |
| ExternalPolarity | Pulse Modulation External Polarity | P |
| LFGenerator |  |  |
| Configure | Configure LF Generator | M |
| ActiveLFGenerator | Active LFGenerator | P |
| Name | LFGenerator Name | P |
| Count | LFGenerator Count | P |
| Frequency | LFGenerator Frequency | P |
| Waveform | LFGenerator Waveform | P |
| Output |  |  |
| Configure | Configure LFGenerator Output | M |
| Enabled | LFGenerator Output Enabled | P |
| Amplitude | LFGenerator Output Amplitude | P |
| PulseGenerator |  |  |
| Configure | Configure Pulse | M |
| ConfigureExternalTrigger | Configure Pulse External Trigger | M |
| InternalTriggerPeriod | Pulse Internal Trigger Period | P |
| Width | Pulse Width | P |
| GatingEnabled | Pulse Gating Enabled | P |
| TriggerSource | Pulse Trigger Source | P |
| ExternalTriggerSlope | Pulse External Trigger Slope | P |
| ExternalTriggerDelay | Pulse External Trigger Delay | P |
| Double |  |  |
| Configure | Configure Pulse Double | M |
| Enabled | Pulse Double Enabled | P |
| Delay | Pulse Double Delay | P |
| Output |  |  |
| Configure | Configure Pulse Output | M |
| Polarity | Pulse Output Polarity | P |
| Enabled | Pulse Output Enabled | P |
| Sweep |  |  |
| Configure | Configure Sweep | M |
| Mode | Sweep Mode | P |
| TriggerSource | Sweep Trigger Source | P |
| FrequencySweep |  |  |
| ConfigureStartStop | Configure Frequency Sweep Start Stop | M |
| ConfigureCenterSpan | Configure Frequency Sweep Center Span | M |
| Start | Frequency Sweep Start | P |
| Stop | Frequency Sweep Stop | P |
| Time | Frequency Sweep Time | P |
| PowerSweep |  |  |
| ConfigureStartStop | Configure Power Sweep Start Stop | M |
| Start | Power Sweep Start | P |
| Stop | Power Sweep Stop | P |
| Time | Power Sweep Time | P |
| FrequencyStep |  |  |
| ConfigureStartStop | Configure Frequency Step Start Stop | M |
| ConfigureDwell | Configure Frequency Step Dwell | M |
| Reset | Frequency Step Reset | M |
| Start | Frequency Step Start | P |
| Stop | Frequency Step Stop | P |
| Scaling | Frequency Step Scaling | P |
| Size | Frequency Step Size | P |
| SingleStepEnabled | Frequency Step Single Step Enabled | P |
| Dwell | Frequency Step Swell | P |
| PowerStep |  |  |
| ConfigureStartStop | Configure Power Step Start Stop | M |
| ConfigureDwell | Configure Power Step Dwell | M |
| Reset | Power Step Reset | M |
| Start | Power Step Start | P |
| Stop | Power Step Stop | P |
| Size | Power Step Size | P |
| SingleStepEnabled | Power Step Single Step Enabled | P |
| Dwell | Power Step Dwell | P |
| List |  |  |
| CreateFrequency | Create Frequency List | M |
| CreatePower | Create Power List | M |
| CreateFrequencyPower | Create Frequency Power List | M |
| ClearAll | Clear All List | M |
| ConfigureDwell | Configure List Dwell | M |
| Reset | List Reset | M |
| SelectedName | List Selected Name | P |
| SingleStepEnabled | List Single Step Enabled | P |
| Dwell | List Dwell | P |
| ALC |  |  |
| Configure | Configure ALC | M |
| Enabled | ALC Enabled | P |
| Source | ALC Source | P |
| Bandwidth | ALC Bandwidth | P |
| ReferenceOscillator |  |  |
| Configure | Configure Reference Oscillator | M |
| Source | Reference Oscillator Source | P |
| ExternalFrequency | Reference Oscillator External Frequency | P |
| IQ |  |  |
| Configure | Configure IQ | M |
| Calibrate | Calibrate IQ | M |
| Enabled | IQ Enabled | P |
| Source | IQ Source | P |
| NominalVoltage | IQ Nominal Voltage | P |
| SwapEnabled | IQ Swap Enabled | P |
| Impairment |  |  |
| Configure | Configure IQ Impairment | M |
| Enabled | IQ Impairment Enabled | P |
| IOffset | IQ I-Offset | P |
| QOffset | IQ Q-Offset | P |
| Ratio | IQ Ratio | P |
| Skew | IQ Skew | P |
| DigitalModulation |  |  |
| Arb |  |  |
| Configure | Configure Arb | M |
| WriteWaveform | Write Arb Waveform | M |
| ClearAllWaveforms | Clear All Arb Waveforms | M |
| QueryWaveformCapabilities | Query Arb Waveform Capabilities | M |
| SelectedWaveform | Arb Selected Waveform | P |
| ClockFrequency | Arb Clock Frequency | P |
| FilterFrequency | Arb Filter Frequency | P |
| MaxNumberWaveforms | Arb Max Number Waveforms | P |
| WaveformQuantum | Arb Waveform Quantum | P |
| WaveformSizeMin | Arb Waveform Size Min | P |
| WaveformSizeMax | Arb Waveform Size Max | P |
| TriggerSource | Arb Trigger Source | P |
| ExternalTriggerSlope | Arb External Trigger Slope | P |
| Base |  |  |
| StandardName | DigitalModulationBase Standard Name | P |
| StandardCount | DigitalModulationBase Standard Count | P |
| ConfigureClockSource | Configure DigitalModulationBase Clock Source | M |
| WriteBitSequence | Write DigitalModulationBase Bit Sequence | M |
| ClearAllBitSequences | Clear DigitalModulationBase Bit Sequence | M |
| SelectedStandard | DigitalModulationBase Selected Standard | P |
| DataSource | DigitalModulationBase Data Source | P |
| PRBSType | DigitalModulationBase PRBS TYpe | P |
| SelectedBitSequence | DigitalModulationBase Selected Bit Sequence | P |
| ClockSource | DigitalModulationBase Clock Source | P |
| ExternalClockType | DigitalModulationBase External Clock Type | P |
| CDMA |  |  |
| StandardName | CDMA Standard Name | P |
| StandardCount | CDMA Standard Count | P |
| TestModelName | CDMA Test Model Name | P |
| TestModelCount | CDMA Test Model Count | P |
| SelectedStandard | CDMA Selected Standard | P |
| TriggerSource | CDMA Trigger Source | P |
| ExternalTriggerSlope | CDMA External Trigger Slope | P |
| SelectedTestModel | CDMA Selected Test Model | P |
| ClockSource | CDMA Clock Source | P |
| TDMA |  |  |
| StandardName | TDMA Standard Name | P |
| StandardCount | TDMA Standard Count | P |
| ConfigureClockSource | Configure TDMA Clock Source | M |
| FrameName | TDMA Frame Name | P |
| FrameCount | TDMA Frame Count | P |
| SelectedStandard | TDMA Selected Standard | P |
| TriggerSource | TDMA Trigger Source | P |
| ExternalTriggerSlope | TDMA External Trigger Slope | P |
| SelectedFrame | TDMA Selected Frame | P |
| ClockSource | TDMA Clock Source | P |
| ExternalClockType | TDMA External Clock Type | P |

### IviRFSigGen COM Interfaces

In addition to implementing IVI inherent capabilities interfaces, IviRFSigGen interfaces contain interface reference properties for accessing the following IviRFSigGen interfaces:

- IIviRFSigGenRF

- IIviRFSigGenAnalogModulation

- IIviRFSigGenPulseModulation

- IIviRFSigGenLFGenerator

- IIviRFSigGenPulseGenerator

- IIviRFSigGenSweep

- IIviRFSigGenALC

- IIviRFSigGenReferenceOscillator

- IIviRFSigGenIQ

- IiviRFSigGenDigitalModulation

The IIviRFSigGenAnalogModulation interface contains interface reference properties for accessing additional the following IviRFSigGen AnalogModulation interfaces:

- IIviRFSigGenAnalogModulationSource

- IIviRFSigGenAM

- IIviRFSigGenFM

- IIviRFSigGenPM

The IIviRFSigGenLFGenerator interface contains interface reference properties for accessing additional the following IviRFSigGen LFGenerator interfaces:

- IIviRFSigGenLFGeneratorOutput

The IIviRFSigGenPulseGenerator interface contains interface reference properties for accessing additional the following IviRFSigGen PulseGenerator interfaces:

- IIviRFSigGenPulseGeneratorDouble

- IIviRFSigGenPulseGeneratorOutput

The IIviRFSigGenSweep interface contains interface reference properties for accessing additional the following IviRFSigGen Sweep interfaces:

- IIviRFSigGenFrequencySweep

- IIviRFSigGenPowerSweep

- IIviRFSigGenFrequencyStep

- IIviRFSigGenPowerStep

- IIviRFSigGenList

The IIviRFSigGenIQ interface contains interface reference properties for accessing additional the following IviRFSigGen IQ interfaces:

- IIviRFSigGenIQImpairment

The IIviRFSigGenDigitalModulation interface contains interface reference properties for accessing additional the following IviRFSigGen DigitalModulation interfaces:

- IIviRFSigGenArb

- IIviRFSigGenDigitalModulationBase

- IIviRFSigGenCDMA

- IIviRFSigGenTDMA

Table 34-8. IviRFSigGen Interface GUIDs lists the interfaces that this specification defines and their GUIDs.

| Table 34-8. IviRFSigGen Interface GUIDs |  |
| --- | --- |
| Interface | GUID |
| IIviRFSigGen | {47ed52e2-a398-11d4-ba58-000064657374} |
| IIviRFSigGenRF | {47ed52e3-a398-11d4-ba58-000064657374} |
| IIviRFSigGenALC | {47ed52e4-a398-11d4-ba58-000064657374} |
| IIviRFSigGenReferenceOscillator | {47ed52e5-a398-11d4-ba58-000064657374} |
| IIviRFSigGenLFGenerator | {47ed52e6-a398-11d4-ba58-000064657374} |
| IIviRFSigGenLFGeneratorOutput | {47ed52e7-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPulseGenerator | {47ed52e8-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPulseGeneratorDouble | {47ed52e9-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPulseGeneratorOutput | {47ed52ea-a398-11d4-ba58-000064657374} |
| IIviRFSigGenAnalogModulation | {47ed52eb-a398-11d4-ba58-000064657374} |
| IIviRFSigGenAnalogModulationSource | {47ed52ec-a398-11d4-ba58-000064657374} |
| IIviRFSigGenAM | {47ed52ed-a398-11d4-ba58-000064657374} |
| IIviRFSigGenFM | {47ed52ee-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPM | {47ed52ef-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPulseModulation | {47ed52f0-a398-11d4-ba58-000064657374} |
| IIviRFSigGenSweep | {47ed52f1-a398-11d4-ba58-000064657374} |
| IIviRFSigGenFrequencySweep | {47ed52f2-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPowerSweep | {47ed52f3-a398-11d4-ba58-000064657374} |
| IIviRFSigGenFrequencyStep | {47ed52f4-a398-11d4-ba58-000064657374} |
| IIviRFSigGenPowerStep | {47ed52f5-a398-11d4-ba58-000064657374} |
| IIviRFSigGenList | {47ed52f6-a398-11d4-ba58-000064657374} |
| IIviRFSigGenIQ | {47ed52f8-a398-11d4-ba58-000064657374} |
| IIviRFSigGenIQImpairment | {47ed52f9-a398-11d4-ba58-000064657374} |
| IIviRFSigGenDigitalModulation | {47ed52f7-a398-11d4-ba58-000064657374} |
| IIviRFSigGenArb | {47ed52fa-a398-11d4-ba58-000064657374} |
| IIviRFSigGenDigitalModulationBase | {47ed52fb-a398-11d4-ba58-000064657374} |
| IIviRFSigGenCDMA | {47ed52fc-a398-11d4-ba58-000064657374} |
| IIviRFSigGenTDMA | {47ed52fd-a398-11d4-ba58-000064657374} |

### COM Interface Reference Properties

Interface reference properties are used to navigate the IviRFSigGen .NET hierarchy. This section describes the interface reference properties that the IviRFSigGen interfaces define.  All interface reference properties are read-only.

| Table 34-9. IviRFSigGen Interface Reference Properties |  |  |  |
| --- | --- | --- | --- |
| Interface | Reference Property Name |  |  |
| IIviRFSigGenRF | RF |  |  |
| IIviRFSigGenALC | ALC |  |  |
| IIviRFSigGenReferenceOscillator | ReferenceOscillator |  |  |
| IIviRFSigGenLFGenerator | LFGenerator |  |  |
| IIviRFSigGenLFGeneratorOutput | LFGenerator.Output |  |  |
| IIviRFSigGenPulseGenerator | PulseGenerator |  |  |
| IIviRFSigGenPulseGeneratorDouble | PulseGenerator.Double |  |  |
| IIviRFSigGenPulseGeneratorOutput | PulseGenerator.Output |  |  |
| IIviRFSigGenAnalogModulation | AnalogModulation |  |  |
| IIviRFSigGenAnalogModulationSource | AnalogModulation.Source |  |  |
| IIviRFSigGenAM | AnalogModulation.AM |  |  |
| IIviRFSigGenFM | AnalogModulation.FM |  |  |
| IIviRFSigGenPM | AnalogModulation.PM |  |  |
| IIviRFSigGenPulseModulation | PulseModulation |  |  |
| IIviRFSigGenSweep | Sweep |  |  |
| IIviRFSigGenFrequencySweep | Sweep.FrequencySweep |  |  |
| IIviRFSigGenPowerSweep | Sweep.PowerSweep |  |  |
| IIviRFSigGenFrequencyStep | Sweep.FrequencyStep |  |  |
| IIviRFSigGenPowerStep | Sweep.PowerStep |  |  |
| IIviRFSigGenList | Sweep.List |  |  |
| IIviRFSigGenIQ | IQ |  |  |
| IIviRFSigGenIQImpairment | IQ.Impairment |  |  |
| IIviRFSigGenDigitalModulation | DigitalModulation |  |  |
| IIviRFSigGenArb | DigitalModulation.Arb |  |  |
| IIviRFSigGenDigitalModulationBase | DigitalModulation.Base |  |  |
| IIviRFSigGenCDMA | DigitalModulation.CDMA |  |  |
| IIviRFSigGenTDMA | DigitalModulation.TDMA |  |  |

### IviRFSigGen COM Category

The IviRFSigGen class COM Category shall be “IviRFSigGen”, and the Category ID (CATID) shall be {47ed5159-a398-11d4-ba58-000064657374}.

## IviRFSigGen C Function Hierarchy

The IviRFSigGen class function hierarchy is shown in the following table.

| Table 34-10. IviRFSigGen Function Hierarchy |  |
| --- | --- |
| Name or Class | Function Name |
| Configuration… |  |
| RF… |  |
| Configure RF | IviRFSigGen_ConfigureRF |
| Configure ALC Enabled | IviRFSigGen_ConfigureALCEnabled |
| Configure Output Enabled | IviRFSigGen_ConfigureOutputEnabled |
| AnalogModulation… |  |
| Configure AM Enabled | IviRFSigGen_ConfigureAMEnabled |
| Configure AM External Coupling | IviRFSigGen_ConfigureAMExternalCoupling |
| Configure AM | IviRFSigGen_ConfigureAM |
| Configure FM Enabled | IviRFSigGen_ConfigureFMEnabled |
| Configure FM External Coupling | IviRFSigGen_ConfigureFMExternalCoupling |
| Configure FM | IviRFSigGen_ConfigureFM |
| Configure PM Enabled | IviRFSigGen_ConfigurePMEnabled |
| Configure PM External Coupling | IviRFSigGen_ConfigurePMExternalCoupling |
| Configure PM | IviRFSigGen_ConfigurePM |
| Get Analog Modulation Source Name | IviRFSigGen_GetAnalogModulationSourceName |
| PulseModulation… |  |
| Configure Pulse Modulation Enabled | IviRFSigGen_ConfigurePulseModulationEnabled |
| Configure Pulse Modulation Source | IviRFSigGen_ConfigurePulseModulationSource |
| Configure Pulse Modulation External Polarity | IviRFSigGen_ConfigurePulseModulationExternalPolarity |
| LFGenerator... |  |
| Configure LFGenerator | IviRFSigGen_ConfigureLFGenerator |
| Get LFGenerator Name | IviRFSigGen_GetLFGeneratorName |
| Set Active LFGenerator | IviRFSigGen_SetActiveLFGenerator |
| Configure LFGenerator Output | IviRFSigGen_ConfigureLFGeneratorOutput |
| Pulse Generator... |  |
| Configure Pulse External Trigger | IviRFSigGen_ConfigurePulseExternalTrigger |
| Configure Pulse Internal Trigger | IviRFSigGen_ConfigurePulseInternalTrigger |
| Configure Pulse | IviRFSigGen_ConfigurePulse |
| Configure Pulse Double | IviRFSigGen_ConfigurePulseDouble |
| Configure Pulse Output | IviRFSigGen_ConfigurePulseOutput |
| Sweep… |  |
| Configure Sweep | IviRFSigGen_ConfigureSweep |
| Configure Frequency Sweep Start Stop | IviRFSigGen_ConfigureFrequencySweepStartStop |
| Configure Frequency Sweep Center Span | IviRFSigGen_ConfigureFrequencySweepCenterSpan |
| Configure Frequency Sweep Time | IviRFSigGen_ConfigureFrequencySweepTime |
| Configure Power Sweep Start Stop | IviRFSigGen_ConfigurePowerSweepStartStop |
| Configure Power Sweep Time | IviRFSigGen_ConfigurePowerSweepTime |
| Configure Frequency Step Start Stop | IviRFSigGen_ConfigureFrequencyStepStartStop |
| Configure Frequency Step Dwell | IviRFSigGen_ConfigureFrequencyStepDwell |
| Reset Frequency Step | IviRFSigGen_ResetFrequencyStep |
| Configure Power Step Start Stop | IviRFSigGen_ConfigurePowerStepStartStop |
| Configure Power Step Dwell | IviRFSigGen_ConfigurePowerStepDwell |
| Reset Power Step | IviRFSigGen_ResetPowerStep |
| Create Frequency List | IviRFSigGen_CreateFrequencyList |
| Create Power List | IviRFSigGen_CreatePowerList |
| Create Frequency Power List | IviRFSigGen_CreateFrequencyPowerList |
| Select List | IviRFSigGen_SelectList |
| Clear All Lists | IviRFSigGen_ClearAllLists |
| Configure List Dwell | IviRFSigGen_ConfigureListDwell |
| Reset List | IviRFSigGen_ResetList |
| ALC… |  |
| Configure ALC | IviRFSigGen_ConfigureALC |
| ReferenceOscillator… |  |
| Configure Reference Oscillator | IviRFSigGen_ConfigureReferenceOscillator |
| IQ... |  |
| Configure IQ Enabled | IviRFSigGen_ConfigureIQEnabled |
| Configure IQ | IviRFSigGen_ConfigureIQ |
| Calibrate IQ | IviRFSigGen_CalibrateIQ |
| Configure IQ Impairment Enabled | IviRFSigGen_ConfigureIQImpairmentEnabled |
| Configure IQ Impairment | IviRFSigGen_ConfigureIQImpairment |
| ARB Generator... |  |
| Configure Arb | IviRFSigGen_ConfigureArb |
| Write Arb Waveform | IviRFSigGen_WriteArbWaveform |
| Select Arb Waveform | IviRFSigGen_SelectArbWaveform |
| Clear All Arb Waveforms | IviRFSigGen_ClearAllArbWaveforms |
| Query Arb Waveform Capabilities | IviRFSigGen_QueryArbWaveformCapabilities |
| Configure Arb Trigger Source | IviRFSigGen_ConfigureArbTriggerSource |
| Configure Arb External Trigger Slope | IviRFSigGen_ConfigureArbExternalTriggerSlope |
| Digital Modulation Base... |  |
| Get DigitalModulationBase Standard Name | IviRFSigGen_GetDigitalModulationBaseStandardName |
| Select DigitalModulationBase Standard | IviRFSigGen_SelectDigitalModulationBaseStandard |
| Configure DigitalModulationBase Clock Source | IviRFSigGen_ConfigureDigitalModulationBaseClockSource |
| Configure DigitalModulationBase Data Source | IviRFSigGen_ConfigureDigitalModulationBaseDataSource |
| Configure DigitalModulationBase PRBS Type | IviRFSigGen_ConfigureDigitalModulationBasePRBSType |
| Write DigitalModulationBase Bit Sequence | IviRFSigGen_WriteDigitalModulationBaseBitSequence |
| Select DigitalModulationBase Bit Sequence | IviRFSigGen_SelectDigitalModulationBaseBitSequence |
| Clear All DigitalModulationBase Bit Sequences | IviRFSigGen_ClearAllDigitalModulationBaseBitSequences |
| CDMA... |  |
| Get CDMA Standard Name | IviRFSigGen_GetCDMAStandardName |
| Select CDMA Standard | IviRFSigGen_SelectCDMAStandard |
| Configure CDMA Clock Source | IviRFSigGen_ConfigureCDMAClockSource |
| Configure CDMA Trigger Source | IviRFSigGen_ConfigureCDMATriggerSource |
| Configure CDMA External Trigger Slope | IviRFSigGen_ConfigureCDMAExternalTriggerSlope |
| Get CDMA Test Model Name | IviRFSigGen_GetCDMATestModelName |
| Select CDMA Test Model | IviRFSigGen_SelectCDMATestModel |
| TDMA... |  |
| Get TDMA Standard Name | IviRFSigGen_GetTDMAStandardName |
| Select TDMA Standard | IviRFSigGen_SelectTDMAStandard |
| Configure TDMA Clock Source | IviRFSigGen_ConfigureTDMAClockSource |
| Configure TDMA Trigger Source | IviRFSigGen_ConfigureTDMATriggerSource |
| Configure TDMA External Trigger Slope | IviRFSigGen_ConfigureTDMAExternalTriggerSlope |
| Get TDMA Frame Name | IviRFSigGen_GetTDMAFrameName |
| Select TDMA Frame | IviRFSigGen_SelectTDMAFrame |
| Action… |  |
| Disable All Modulation | IviRFSigGen_DisableAllModulation |
| IsSettled | IviRFSigGen_IsSettled |
| WaitUntilSettled | IviRFSigGen_WaitUntilSettled |
| Send Software Trigger | IviRFSigGen_SendSoftwareTrigger |

## IviRFSigGen C Attribute Hierarchy

The IviRFSigGen C attribute hierarchy is shown in the following table.

|  | Table 34-11. IviRFSigGen C Attributes Hierarchy |  |
| --- | --- | --- |
|  | Category or Generic Attribute Name | C Defined Constant |
|  | RF |  |
|  | Frequency | IVIRFSIGGEN_ATTR_FREQUENCY |
|  | Power Level | IVIRFSIGGEN_ATTR_POWER_LEVEL |
|  | ALC Enabled | IVIRFSIGGEN_ATTR_ALC_ENABLED |
|  | Output Enabled | IVIRFSIGGEN_ATTR_OUTPUT_ENABLED |
|  | Analog Modulation |  |
|  | AM |  |
|  | AM Enabled | IVIRFSIGGEN_ATTR_AM_ENABLED |
|  | AM Source | IVIRFSIGGEN_ATTR_AM_SOURCE |
|  | AM Scaling | IVIRFSIGGEN_ATTR_AM_SCALING |
|  | AM External Coupling | IVIRFSIGGEN_ATTR_AM_EXTERNAL_COUPLING |
|  | AM Nominal Voltage | IVIRFSIGGEN_ATTR_AM_NOMINAL_VOLTAGE |
|  | AM Depth | IVIRFSIGGEN_ATTR_AM_DEPTH |
|  | FM |  |
|  | FM Enabled | IVIRFSIGGEN_ATTR_FM_ENABLED |
|  | FM Source | IVIRFSIGGEN_ATTR_FM_SOURCE |
|  | FM External Coupling | IVIRFSIGGEN_ATTR_FM_EXTERNAL_COUPLING |
|  | FM Nominal Voltage | IVIRFSIGGEN_ATTR_FM_NOMINAL_VOLTAGE |
|  | FM Deviation | IVIRFSIGGEN_ATTR_FM_DEVIATION |
|  | PM |  |
|  | PM Enabled | IVIRFSIGGEN_ATTR_PM_ENABLED |
|  | PM Source | IVIRFSIGGEN_ATTR_PM_SOURCE |
|  | PM External Coupling | IVIRFSIGGEN_ATTR_PM_EXTERNAL_COUPLING |
|  | PM Nominal Voltage | IVIRFSIGGEN_ATTR_PM_NOMINAL_VOLTAGE |
|  | PM Deviation | IVIRFSIGGEN_ATTR_PM_DEVIATION |
|  | Source |  |
|  | Analog Modulation Source Count | IVIRFSIGGEN_ATTR_ANALOG_MODULATION_SOURCE_COUNT |
|  | Pulse Modulation |  |
|  | Pulse Modulation Enabled | IVIRFSIGGEN_ATTR_PULSE_MODULATION_ENABLED |
|  | Pulse Modulation Source | IVIRFSIGGEN_ATTR_PULSE_MODULATION_SOURCE |
|  | Pulse Modulation External Polarity | IVIRFSIGGEN_ATTR_PULSE_MODULATION_EXTERNAL_POLARITY |
|  | LF Generator |  |
|  | Active LFGenerator | IVIRFSIGGEN_ATTR_ACTIVE_LF_GENERATOR |
|  | LFGenerator Count | IVIRFSIGGEN_ATTR_LF_GENERATOR_COUNT |
|  | LFGenerator Frequency | IVIRFSIGGEN_ATTR_LF_GENERATOR_FREQUENCY |
|  | LFGenerator Waveform | IVIRFSIGGEN_ATTR_LF_GENERATOR_WAVEFORM |
|  | LF Generator Outputs |  |
|  | LFGenerator Output Amplitude | IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_AMPLITUDE |
|  | LFGenerator Output Enabled | IVIRFSIGGEN_ATTR_LF_GENERATOR_OUTPUT_ENABLED |
|  | Pulse Generator |  |
|  | Pulse Internal Trigger Period | IVIRFSIGGEN_ATTR_PULSE_INTERNAL_TRIGGER_PERIOD |
|  | Pulse Width | IVIRFSIGGEN_ATTR_PULSE_WIDTH |
|  | Pulse Gating Enabled | IVIRFSIGGEN_ATTR_PULSE_GATING_ENABLED |
|  | Pulse Trigger Source | IVIRFSIGGEN_ATTR_PULSE_TRIGGER_SOURCE |
|  | Pulse External Trigger Slope | IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_SLOPE |
|  | Pulse External Trigger Delay | IVIRFSIGGEN_ATTR_PULSE_EXTERNAL_TRIGGER_DELAY |
|  | Double Pulse Generators |  |
|  | Pulse Double Enabled | IVIRFSIGGEN_ATTR_PULSE_DOUBLE_ENABLED |
|  | Pulse Double Delay | IVIRFSIGGEN_ATTR_PULSE_DOUBLE_DELAY |
|  | Pulse Generator Output |  |
|  | Pulse Output Polarity | IVIRFSIGGEN_ATTR_PULSE_OUTPUT_POLARITY |
|  | Pulse Output Enabled | IVIRFSIGGEN_ATTR_PULSE_OUTPUT_ENABLED |
|  | Sweep |  |
|  | Sweep Mode | IVIRFSIGGEN_ATTR_SWEEP_MODE |
|  | Sweep Trigger Source | IVIRFSIGGEN_ATTR_SWEEP_TRIGGER_SOURCE |
|  | Frequency Sweep |  |
|  | Frequency Sweep Start | IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_START |
|  | Frequency Sweep Stop | IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_STOP |
|  | Frequency Sweep Time | IVIRFSIGGEN_ATTR_FREQUENCY_SWEEP_TIME |
|  | Power Sweep |  |
|  | Power Sweep Start | IVIRFSIGGEN_ATTR_POWER_SWEEP_START |
|  | Power Sweep Stop | IVIRFSIGGEN_ATTR_POWER_SWEEP_STOP |
|  | Power Sweep Time | IVIRFSIGGEN_ATTR_POWER_SWEEP_TIME |
|  | Frequency Step |  |
|  | Frequency Step Start | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_START |
|  | Frequency Step Stop | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_STOP |
|  | Frequency Step Scaling | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SCALING |
|  | Frequency Step Size | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SIZE |
|  | Frequency Step Single Step Enabled | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_SINGLE_STEP_ENABLED |
|  | Frequency Step Dwell | IVIRFSIGGEN_ATTR_FREQUENCY_STEP_DWELL |
|  | Power Step |  |
|  | Power Step Start | IVIRFSIGGEN_ATTR_POWER_STEP_START |
|  | Power Step Stop | IVIRFSIGGEN_ATTR_POWER_STEP_STOP |
|  | Power Step Size | IVIRFSIGGEN_ATTR_POWER_STEP_SIZE |
|  | Power Step Single Step Enabled | IVIRFSIGGEN_ATTR_POWER_STEP_SINGLE_STEP_ENABLED |
|  | Power Step Dwell | IVIRFSIGGEN_ATTR_POWER_STEP_DWELL |
|  | List |  |
|  | List Selected Name | IVIRFSIGGEN_ATTR_LIST_SELECTED_NAME |
|  | List Single Step Enabled | IVIRFSIGGEN_ATTR_LIST_SINGLE_STEP_ENABLED |
|  | List Dwell | IVIRFSIGGEN_ATTR_LIST_DWELL |
|  | ALC |  |
|  | ALC Source | IVIRFSIGGEN_ATTR_ALC_SOURCE |
|  | ALC Bandwidth | IVIRFSIGGEN_ATTR_ALC_BANDWIDTH |
|  | Reference Oscillator |  |
|  | Reference Oscillator Source | IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_SOURCE |
|  | Reference Oscillator External Frequency | IVIRFSIGGEN_ATTR_REFERENCE_OSCILLATOR_EXTERNAL_FREQUENCY |
|  | IQ |  |
|  | IQ Enabled | IVIRFSIGGEN_ATTR_IQ_ENABLED |
|  | IQ Nominal Voltage | IVIRFSIGGEN_ATTR_IQ_NOMINAL_VOLTAGE |
|  | IQ Source | IVIRFSIGGEN_ATTR_IQ_SOURCE |
|  | IQ Swap Enabled | IVIRFSIGGEN_ATTR_IQ_SWAP_ENABLED |
|  | IQ Impairment |  |
|  | IQ Impairment Enabled | IVIRFSIGGEN_ATTR_IQ_IMPAIRMENT_ENABLED |
|  | IQ I-Offset | IVIRFSIGGEN_ATTR_IQ_I_OFFSET |
|  | IQ Q-Offset | IVIRFSIGGEN_ATTR_IQ_Q_OFFSET |
|  | IQ Ratio | IVIRFSIGGEN_ATTR_IQ_RATIO |
|  | IQ Skew | IVIRFSIGGEN_ATTR_IQ_SKEW |
|  | ARB Generator |  |
|  | Arb Clock Frequency | IVIRFSIGGEN_ATTR_ARB_CLOCK_FREQUENCY |
|  | Arb Filter Frequency | IVIRFSIGGEN_ATTR_ARB_FILTER_FREQUENCY |
|  | Waveform |  |
|  | Arb Selected Waveform | IVIRFSIGGEN_ATTR_ARB_SELECTED_WAVEFORM |
|  | Arb Max Number Waveforms | IVIRFSIGGEN_ATTR_ARB_MAX_NUMBER_WAVEFORMS |
|  | Arb Waveform Quantum | IVIRFSIGGEN_ATTR_ARB_WAVEFORM_QUANTUM |
|  | Arb Waveform Size Min | IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MIN |
|  | Arb Waveform Size Max | IVIRFSIGGEN_ATTR_ARB_WAVEFORM_SIZE_MAX |
|  | Trigger |  |
|  | Arb Trigger Source | IVIRFSIGGEN_ATTR_ARB_TRIGGER_SOURCE |
|  | Arb External Trigger Slope | IVIRFSIGGEN_ATTR_ARB_EXTERNAL_TRIGGER_SLOPE |
|  | Digital Modulation |  |
|  | DigitalModulationBase Standard Count | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_STANDARD_COUNT |
|  | DigitalModulationBase Selected Standard | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_SELECTED_STANDARD |
|  | DigitalModulationBase Data Source | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_DATA_SOURCE |
|  | DigitalModulationBase PRBS Type | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_PRBS_TYPE |
|  | DigitalModulationBase Bit Sequence | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_BIT_SEQUENCE |
|  | DigitalModulationBase Clock Source | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_CLOCK_SOURCE |
|  | DigitalModulationBase External Clock Type | IVIRFSIGGEN_ATTR_DIGITAL_MODULATION_BASE_EXTERNAL_CLOCK_TYPE |
|  | CDMA |  |
|  | CDMA Standard Count | IVIRFSIGGEN_ATTR_CDMA_STANDARD_COUNT |
|  | CDMA Selected Standard | IVIRFSIGGEN_ATTR_CDMA_SELECTED_STANDARD |
|  | CDMA Test Model Count | IVIRFSIGGEN_ATTR_CDMA_TEST_MODEL_COUNT |
|  | CDMA Seleted Test Model | IVIRFSIGGEN_ATTR_CDMA_SELECTED_TEST_MODEL |
|  | CDMA Clock Source | IVIRFSIGGEN_ATTR_CDMA_CLOCK_SOURCE |
|  | Trigger |  |
|  | CDMA Trigger Source | IVIRFSIGGEN_ATTR_CDMA_TRIGGER_SOURCE |
|  | CDMA External Trigger Slope | IVIRFSIGGEN_ATTR_CDMA_EXTERNAL_TRIGGER_SLOPE |
|  | TDMA |  |
|  | TDMA Standard Count | IVIRFSIGGEN_ATTR_TDMA_STANDARD_COUNT |
|  | TDMA Selected Standard | IVIRFSIGGEN_ATTR_TDMA_SELECTED_STANDARD |
|  | TDMA Frame Count | IVIRFSIGGEN_ATTR_TDMA_FRAME_COUNT |
|  | TDMA Seleted Frame | IVIRFSIGGEN_ATTR_TDMA_SELECTED_FRAME |
|  | TDMA Clock Source | IVIRFSIGGEN_ATTR_TDMA_CLOCK_SOURCE |
|  | TDMA External Clock Type | IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_CLOCK_TYPE |
|  | Trigger |  |
|  | TDMA Trigger Source | IVIRFSIGGEN_ATTR_TDMA_TRIGGER_SOURCE |
|  | TDMA External Trigger Slope | IVIRFSIGGEN_ATTR_TDMA_EXTERNAL_TRIGGER_SLOPE |

- Specific Driver Development Guidelines

  - Introduction

This section describes situations driver developers should be aware of when developing a specific instrument driver that complies with the IviRFSigGen class.

  - Disabling Unused Extensions

Specific drivers are required to disable extension capability groups that an application program does not explicitly use.  The specific driver can do so by setting the attributes of an extension capability group to the values that this section recommends.  A specific driver can set these values for all extension capability groups when the IViRFSigGen_init, IViRFSigGen_InitWithOptions, or IViRFSigGen reset functions execute.  This assumes that the extension capability groups remain disabled until the application program explicitly uses them.  For the large majority of instruments, this assumption is true.

Under certain conditions, a specific driver might have to implement a more complex approach.  For some instruments, configuring a capability group might affect instrument settings that correspond to an unused extension capability group.  If these instrument settings affect the behavior of the instrument, then this might result in an interchangeability problem.  If this can occur, the specific driver shall take appropriate action so that the instrument settings that correspond to the unused extension capability group do not affect the behavior of the instrument when the application program performs an operation that might be affected by those settings.

The remainder of this section recommends attribute values that effectively disable each extension capability group.

Disabling the IviRFSigGenModulateAM Extension Group

Attribute values that effectively disable the IviRFSigGenModulateAM extension group are shown in the following table. This functionality is also encapsulated in the Disable All Modulation function.

| Table A.1. Values for Disabling the IviRFSigGenModulateAM Extension Group |  |
| --- | --- |
| Attribute | Value |
| AM Enabled | False |

Disabling the IviRFSigGenModulateFM Extension Group

Attribute values that effectively disable the IviRFSigGenModulateFM extension group are shown in the following table. This functionality is also encapsulated in the Disable All Modulation function.

| Table A.2. Values for Disabling the IviRFSigGenModulateFM Extension Group |  |
| --- | --- |
| Attribute | Value |
| FM Enabled | False |

Disabling the IviRFSigGenModulatePM Extension Group

Attribute values that effectively disable the IviRFSigGenModulatePM extension group are shown in the following table. This functionality is also encapsulated in the Disable All Modulation function.

| Table A.3. Values for Disabling the IviRFSigGenModulatePM Extension Group |  |
| --- | --- |
| Attribute | Value |
| PM Enabled | False |

Disabling the IviRFSigGenModulatePulse Extension Group

Attribute values that effectively disable the IviRFSigGenModulatePulse extension group are shown in the following table. This functionality is also encapsulated in the Disable All Modulation function.

| Table A.4. Values for Disabling the IviRFSigGenModulatePulse Extension Group |  |
| --- | --- |
| Attribute | Value |
| Pulse Modulation Enabled | False |

Disabling the IviRFSigGenLFGenerator Extension Group

The IviRFSigGenLFGenerator extension group affects the instrument behavior only when its attributes are used. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenLFGenerator extension group.

Disabling the IviRFSigGenLFGeneratorOutput Extension Group

Attribute values that effectively disable the IviRFSigGenLFGeneratorOutput extension group are shown in the following table.

| Table A.5. Values for Disabling the IviRFSigGenLFGeneratorOutput Extension Group |  |
| --- | --- |
| Attribute | Value |
| LFGenerator Output Enabled | False |

Disabling the IviRFSigGenPulseGenerator Extension Group

The IviRFSigGenPulseGenerator extension group affects the instrument behavior only when its attributes are used. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenPulseGenerator extension group.

Disabling the IviRFSigGenPulseDoubleGenerator Extension Group

Attribute values that effectively disable the IviRFSigGenPulseDoubleGenerator extension group are shown in the following table.

| Table A.6. Values for Disabling the IviRFSigGenPulseDoubleGenerator Extension Group |  |
| --- | --- |
| Attribute | Value |
| Pulse Double Enabled | False |

Disabling the IviRFSigGenPulseGeneratorOutput Extension Group

Attribute values that effectively disable the IviRFSigGenPulseGeneratorOutput extension group are shown in the following table.

| Table A.7. Values for Disabling the IviRFSigGenPulseGeneratorOutput Extension Group |  |
| --- | --- |
| Attribute | Value |
| Pulse Output Enabled | False |

Disabling the IviRFSigGenSweep Extension Group

Attribute values that effectively disable the IviRFSigGenSweep extension group are shown in the following table.

| Table A.8. Values for Disabling the IviRFSigGenSweep Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenFrequencySweep Extension Group

Attribute values that effectively disable the IviRFSigGenFrequencySweep extension group are shown in the following table.

| Table A.9. Values for Disabling the IviRFSigGenFrequencySweep Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenPowerSweep Extension Group

Attribute values that effectively disable the IviRFSigGenPowerSweep extension group are shown in the following table.

| Table A.10. Values for Disabling the IviRFSigGenPowerSweep Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenFrequencyStep Extension Group

Attribute values that effectively disable the IviRFSigGenFrequencyStep extension group are shown in the following table.

| Table A.11. Values for Disabling the IviRFSigGenFrequencyStep Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenPowerStep Extension Group

Attribute values that effectively disable the IviRFSigGenPowerStep extension group are shown in the following table.

| Table A.12. Values for Disabling the IviRFSigGenPowerStep Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenList Extension Group

Attribute values that effectively disable the IviRFSigGenList extension group are shown in the following table.

| Table A.13. Values for Disabling the IviRFSigGenList Extension Group |  |
| --- | --- |
| Attribute | Value |
| Sweep Mode | None |

Disabling the IviRFSigGenALC Extension Group

Attribute values that effectively disable the IviRFSigGenALC extension group are shown in the following table.

| Table A.14. Values for Disabling the IviRFSigGenALC Extension Group |  |
| --- | --- |
| Attribute | Value |
| ALC Enabled | False |

Disabling the IviRFSigGenReferenceOscillator Extension Group

The IviRFSigGenReferenceOscillator extension group affects the instrument behavior only when the Reference Oscillator Source attribute is set to External. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenReferenceOscillator extension group.

Disabling the IviRFSigGenSoftwareTrigger Extension Group

The IviRFSigGenSoftwareTrigger extension group affects the instrument behavior only when the Trigger Source attribute is set to Software Trigger. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenSoftwareTrigger extension group.

Disabling the IviRFSigGenModulateIQ Extension Group

Attribute values that effectively disable the IviRFSigGenModulateIQ extension group are shown in the following table. This functionality is also encapsulated in the Disable All Modulation function.

| Table A.15. Values for Disabling the IviRFSigGenModulateIQ Extension Group |  |
| --- | --- |
| Attribute | Value |
| IQ Enabled | False |

Disabling the IviRFSigGenIQImpairment Extension Group

Attribute values that effectively disable the IviRFSigGenIQImpairment extension group are shown in the following table.

| Table A.16. Values for Disabling the IviRFSigGenModulateIQ Extension Group |  |
| --- | --- |
| Attribute | Value |
| IQ Impairment Enabled | False |

Disabling the IviRFSigGenArbGenerator Extension Group

The IviRFSigGenArbGenerator extension group affects the instrument behavior only when the end user calls any of its functions. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenArbGenerator extension group.

Disabling the IviRFSigGenDigitalModulationBase Extension Group

The IviRFSigGenDigitalModulationBase extension group affects the instrument behavior only when the end user calls any of its functions. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenDigitalModulationBase extension group. This functionality is also encapsulated in the Disable All Modulation function.

Disabling the IviRFSigGenCDMABase Extension Group

The IviRFSigGenCDMABase extension group affects the instrument behavior only when the end user calls any of its functions. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenCDMABase extension group. This functionality is also encapsulated in the Disable All Modulation function.

Disabling the IviRFSigGenTDMABase Extension Group

The IviRFSigGenTDMABase extension group affects the instrument behavior only when the end user calls any of its functions. Therefore, this specification does not recommend attribute values that disable the IviRFSigGenTDMABase extension group. This functionality is also encapsulated in the Disable All Modulation function.

  - Query Instrument Status

Based on the value of Query Instrument Status, the specific driver may check the status of the instrument to see if it has encountered an error.

- Interchangeability Checking Rules

  - Introduction

IVI drivers have a feature called interchangeability checking. Interchangeability checking returns a warning when it encounters a situation were the application program might not produce the same behavior when the user attempts to use a different instrument.

  - When to Perform Interchangeability Checking

Interchangeability checking occurs when all of the following conditions are met:

- The Interchange Check attribute is set to True

- The user calls one of the following functions.

- IsSettled

- WaitUntilSettled

  - Interchangeability Checking Rules

Interchangeability checking is performed on a capability group basis. When enabled, interchangeability checking is always performed on the base capability group.  In addition, interchangeability checking is performed on extension capability groups for which the user has ever set any of the attributes of the group.  If the user has never set any attributes of an extension capability group, interchangeability checking is not performed on that group.

In general interchangeability warnings are generated if the following conditions are encountered:

- An attribute that affects the behavior of the instrument is not in a state that the user specifies.

- The user sets a class driver defined attribute to an instrument-specific value.

- The user configures the value of an attribute that the class defines as read-only.  In a few cases the class drivers define read-only attributes that specific drivers might implement as read/write.

The remainder of this section defines additional rules and exceptions for each capability group.

IviRFSigGenBase Capability Group

- If the Output Enabled attribute is set to False, all other attributes in the IviRFSigGenBase extension group need not be in a user-specified state.

IviRFSigGenModulateAM Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenModulateAM extension group need not be in a user-specified state.

- If the AM Source attribute does not include the ??external modulation source??, the AM External Coupling attribute need not be in a user specified state.

- If the AM Enabled attribute is set to False, all other attributes in the IviRFSigGenModulateAM extension group need not be in a user-specified state.

IviRFSigGenModulateFM Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenModulateFM extension group need not be in a user-specified state.

- If the FM Source attribute does not include the ??external modulation source??, the FM External Coupling attribute need not be in a user specified state.

- If the FM Enabled attribute is set to False, all other attributes in the IviRFSigGenModulateFM extension group need not be in a user-specified state.

IviRFSigGenModulatePM Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenModulatePM extension group need not be in a user-specified state.

- If the PM Source attribute does not include the ??external modulation source??, the PM External Coupling attribute need not be in a user specified state.

- If the PM Enabled attribute is set to False, all other attributes in the IviRFSigGenModulatePM extension group need not be in a user-specified state.

IviRFSigGenAnalogModulationSources Capability Group

No additional interchangeability rules or exceptions are defined for the IviRFSigGenAnalogModulationSources capability group.

IviRFSigGenModulatePulse Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenModulatePulse extension group need not be in a user-specified state.

- If the Pulse Modulation Source attribute is not set to External, the Pulse Modulation External Polarity attribute need not be in a user specified state.

- If the Pulse Modulation Enabled attribute is set to False, all other attributes in the IviRFSigGenModulatePulse extension group need not be in a user-specified state.

IviRFSigGenLFGenerator Capability Group

- If the LFGenerator Output Enabled attribute is set to True 
OR 
If Output Enabled set to True AND one of the following sources includes an internal source
-  AM Source
-  FM Source
-  PM Source
AND any of the following attributes are set to True
-  AM Enabled
-  FM Enabled
-  PM Enabled 
then the attributes in the IviRFSigGenLFGenerator extension group shall be in a user-specified state.

IviRFSigGenLFGeneratorOutput Capability Group

- If the LFGenerator Output Enabled attribute is set to False, the LFGenerator Output Amplitude attribute need not be in a user-specified state.

IviRFSigGenPulseGenerator Capability Group

- If the Pulse Output Enabled attribute is set to True 
OR 
If Output Enabled set to True AND the Pulse Generator Source is set to Internal AND the Pulse Modulation Enabled attribute is set to True, the attributes in the IviRFSigGenPulseGenerator extension group shall be in a user-specified state.

- If the Pulse Trigger Source attribute is not set to Internal, the Pulse Internal Trigger Period attribute and the need not be in a user-specified state.

- If the Pulse Trigger Source attribute is not set to External, the Pulse External Trigger Delay attribute and the Pulse External Trigger Slope attribute need not be in a user-specified state.

IviRFSigGenPulseDoubleGenerator Capability Group

- If the Pulse Double Enabled attribute is set to False, attributes in the IviRFSigGenPulseDoubleGenerator extension group need not be in a user-specified state.

- If the IviRFSigGenPulseGenerator extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenPulseDoubleGenerator extension group need not be in a user-specified state.

IviRFSigGenPulseGeneratorOutput Capability Group

- If the Pulse Output Enabled attribute is set to False, all other attributes in the IviRFSigGenPulseGeneratorOutput extension group need not be in a user-specified state.

IviRFSigGenSweep Capability Group

- If the Sweep Mode attribute is set to None, all other attributes in the IviRFSigGenSweep extension group need not be in a user-specified state.

- If the Output Enabled attribute is set to False, all the attributes in the IviRFSigGenSweep extension group need not be in a user-specified state.

IviRFSigGenFrequencySweep Capability Group

- If the Sweep Mode attribute is not set to Frequency Sweep, attributes in the IviRFSigGenFrequencySweep extension group need not be in a user-specified state.

- If the IviRFSigGenSweep extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenFrequencySweep extension group need not be in a user-specified state.

IviRFSigGenPowerSweep Capability Group

- If the Sweep Mode attribute is not set to Power Sweep, attributes in the IviRFSigGenPowerSweep extension group need not be in a user-specified state.

- If the IviRFSigGenSweep extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenPowerSweep extension group need not be in a user-specified state.

IviRFSigGenFrequencyStep Capability Group

- If the Sweep Mode attribute is not set to Frequency Step, attributes in the IviRFSigGenFrequencyStep extension group need not be in a user-specified state.

- If the IviRFSigGenSweep extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenFrequencyStep extension group need not be in a user-specified state.

IviRFSigGenPowerStep Capability Group

- If the Sweep Mode attribute is not set to Power Step, attributes in the IviRFSigGenPowerStep extension group need not be in a user-specified state.

- If the IviRFSigGenSweep extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenPowerStep extension group need not be in a user-specified state.

IviRFSigGenList Capability Group

- If the Sweep Mode attribute is not set to List, attributes in the IviRFSigGenList extension group need not be in a user-specified state.

- If the IviRFSigGenSweep extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenList extension group need not be in a user-specified state.

IviRFSigGenALC Capability Group

- If the ALC Enabled attribute is set to False, attributes in the IviRFSigGenALC extension group need not be in a user-specified state.

- If the Output Enabled attribute is set to False, all the attributes in the IviRFSigGenALC extension group need not be in a user-specified state.

IviRFSigGenReferenceOscillator Capability Group

- If the Output Enabled attribute is set to False, all the attributes in the IviRFSigGenReferenceOscillator extension group need not be in a user-specified state.

IviRFSigGenSoftwareTrigger Capability Group

No additional interchangeability rules or exceptions are defined for the IviRFSigGenSoftwareTrigger capability group.

IviRFSigGenModulateIQ Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenModulateIQ extension group need not be in a user-specified state.

- If the IQ Enabled attribute is set to False, attributes in the IviRFSigGenModulateIQ extension group need not be in a user-specified state.

IviRFSigGenIQImpairment Capability Group

- If the IQ Impairment Enabled attribute is set to False, attributes in the IviRFSigGenIQImpairment extension group need not be in a user-specified state.

- If the IviRFSigGenModulateIQ extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenIQImpairment extension group need not be in a user-specified state.

IviRFSigGenArbGenerator Capability Group

- If the IQ Source attribute is not set to ArbGenerator, attributes in the IviRFSigGenArbGenerator extension group need not be in a user-specified state.

- If the Arb Trigger Source attribute is not set to External, the Arb External Trigger Slope attribute need not be in a user-specified state.

- If the IviRFSigGenModulateIQ extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenArbGenerator extension group need not be in a user-specified state.

IviRFSigGenDigitalModulationBase Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenDigitalModulationBase extension group need not be in a user-specified state.

- If the IQ Source attribute is not set to DigitalModulationBase, attributes in the IviRFSigGenDigitalModulationBase extension group need not be in a user-specified state.

- If the DigitalModulationBase Clock Source attribute is not set to External, the External Clock Type attribute need not be in a user-specified state.

- If the DigitalModulationBase Data Source attribute is not set to PRBS, the DigitalModulationBase PRBS Type attribute need not be in a user-specified state.

- If the DigitalModulationBase Data Source attribute is not set to Bit Sequence, the DigitalModulationBase Selected Bit Sequence attribute need not be in a user-specified state.

- If the IviRFSigGenModulateIQ extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenDigitalModulationBase extension group need not be in a user-specified state.

IviRFSigGenCDMABase Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenCDMABase extension group need not be in a user-specified state.

- If the IQ Source attribute is not set to CDMABase, attributes in the IviRFSigGenCDMABase extension group need not be in a user-specified state.

- If the CDMA Clock Source is not set to External, the CDMA External Trigger Slope attribute need not be in a user-specified state.

- If the IviRFSigGenModulateIQ extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenCDMABase extension group need not be in a user-specified state.

IviRFSigGenTDMABase Capability Group

- If the Output Enabled attribute is set to False, attributes in the IviRFSigGenTDMABase extension group need not be in a user-specified state.

- If the IQ Source attribute is not set to TDMABase, attributes in the IviRFSigGenTDMABase extension group need not be in a user-specified state.

- If the TDMA Clock Source is not set to External, the TDMA External Trigger Slope attribute need not be in a user-specified state.

- If the IviRFSigGenModulateIQ extension group does not need to be checked for interchangeability, then attributes in the IviRFSigGenTDMABase extension group need not be in a user-specified state.