IVI-4.5: IviACPwr Class Specification

March 11, 2011 Edition

Revision 1.2

- Important Information

IVI-4.5 IviACPwr Class Specification is authored by the IVI Foundation member companies. For a vendor membership roster list, please visit the IVI Foundation web site at www.ivifoundation.org.

The IVI Foundation wants to receive your comments on this specification. You can contact the Foundation through the web site at www.ivifoundation.org.

- Warranty

The IVI Foundation and its member companies make no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. The IVI Foundation and its member companies shall not be liable for errors contained herein or for incidental or consequential damages in connection with the furnishing, performance, or use of this material.

- Trademarks

Product and company names listed are trademarks or trade names of their respective companies.

No investigation has been made of common-law trademark rights in any work.

1	Overview of the IviACPwr Specification	10

1.1	Introduction	10

1.2	IviACPwr Class Overview	10

1.3	References	11

1.4	Definitions of Terms and Acronyms	12

2	IviACPwr Class Capabilities	13

2.1	Introduction	13

2.2	IviACPwr Group Names	13

2.3	Repeated Capability Names	14

2.3.1	OutputPhase	14

2.3.2	Multi Phase Attributes	14

2.3.3	Phase Group	14

2.3.4	Configuring Single/MultiPhase Operation	15

3	General Requirements	16

3.1	Minimum Class Compliance	16

3.1.1	Disable	16

3.2	Capability Group Compliance	16

4	IviACPwrBase Capability Group	17

4.1	Overview	17

4.1.1	User Specified Settings	17

4.1.2	User Queriable Settings	17

4.1.3	Power Source Voltage and Current Scenarios	18

4.2	IviACPwrBase Attributes	19

4.2.1	Current Limit	20

4.2.2	Frequency	21

4.2.3	Number of Phases	22

4.2.4	Output Enabled	23

4.2.5	Voltage Level	25

4.2.6	Waveform	26

4.2.7	Output Phase Item (IVI-COM only)	27

4.2.8	Output Phase Name (IVI-COM only)	28

4.2.9	Output Phase Count	29

4.2.10	Number of Voltage Ranges	30

4.2.11	Voltage Range	31

4.2.12	Number of Frequency Ranges	32

4.2.13	Frequency Range	33

4.3	IviACPwrBase Functions	34

4.3.1	Configure Current Limit (IVI-C only)	35

4.3.2	Configure Output Enabled (IVI-C only)	36

4.3.3	Configure Voltage Level (IVI-C only)	37

4.3.4	Configure Voltage Range (IVI-C only)	38

4.3.5	Configure Frequency (IVI-C only)	39

4.3.6	Configure Frequency Range (IVI-C only)	40

4.3.7	Configure Waveform (IVI-C only)	41

4.3.8	Query Voltage Range Capabilities	42

4.3.9	Query Frequency Range Capabilities	44

4.3.10	Get Output Phase Name (IVI-C only)	45

4.4	IviACPwrBase Behavior Model	46

5	IviACPwrMeasurement Extension Group	47

5.1	IviACPwrMeasurement Overview	47

5.2	IviACPwrMeasurement Functions	47

5.2.1	Initiate Measurement	48

5.2.2	Fetch Measurement	50

5.2.1	Fetch Measurement Array	53

5.3	IviACPwrMeasurement Behavior Model	56

6	IviACPwrPhase Extension Group	57

6.1	IviACPwrPhase Overview	57

6.2	IviACPwrPhase Attributes	58

6.2.1	Phase Angle	59

6.3	IviACPwrPhase Functions	60

6.3.1	Configure Phase Angle (IVI-C only)	61

6.4	IviACPwrPhase Behavior Model	62

7	IviACPwrExternalSync Extension Group	63

7.1	IviACPwrExternalSync Overview	63

7.2	IviACPwrExternalSync Attributes	64

7.2.1	External Synchronization Enabled	65

7.2.2	External Synchronization Phase Offset	66

7.2.3	External Synchronization Locked	67

7.3	IviACPwrExternalSync Functions	68

7.3.1	Configure External Synchronization	69

7.3.2	Query External Synchronization Locked (IVI-C only)	70

7.4	IviACPwrExternalSync Behavior Model	71

8	IviACPwrCurrentProtection Extension Group	72

8.1	IviACPwrCurrentProtection Overview	72

8.2	IviACPwrCurrentProtection Attributes	72

8.2.1	Current Protection Threshold	73

8.2.2	Current Protection Delay	74

8.2.3	Current Protection Enabled	75

8.2.4	Current Protection Tripped	76

8.3	IviACPwrCurrentProtection Functions	77

8.3.1	Query Current Protection Tripped (IVI-C only)	78

8.3.2	Reset Current Protection	80

8.3.3	Configure Current Protection	81

8.4	IviACPwrCurrentProtection Behavior Model	82

9	IviACPwrVoltageProtection Extension Group	83

9.1	IviACPwrVoltageProtection Overview	83

9.2	IviACPwrVoltageProtection Attributes	83

9.2.1	Under Voltage Limit	84

9.2.2	Over Voltage Limit	85

9.2.3	Over Voltage Enabled	86

9.2.4	Under Voltage Enabled	87

9.2.5	Voltage Protection Tripped	88

9.3	IviACPwrVoltageProtection Functions	89

9.3.1	Query Voltage Protection Tripped (IVI-C only)	90

9.3.2	Reset Voltage Protection	92

9.3.3	Configure Voltage Protection	93

9.4	IviACPwrVoltageProtection Behavior Model	94

10	IviACPwrArbWaveform Extension Group	95

10.1	IviACPwrArbWaveform Overview	95

10.2	IviACPwrArbWaveform Attributes	96

10.2.1	Num Waveforms Max	97

10.2.2	Num Optimal Data Points	98

10.2.3	Fixed Waveform Count	99

10.2.4	User Waveform Count	100

10.2.5	Available Waveform Count	101

10.3	IviACPwrArbWaveform Functions	102

10.3.1	Clear Arbitrary Waveform	103

10.3.2	Write Arbitrary Waveform	104

10.3.3	Query Arbitrary Waveform Catalog	106

10.4	IviACPwrArbWaveform Behavior Model	108

11	IviACPwrImpedance Extension Group	109

11.1	IviACPwrImpedance Overview	109

11.2	IviACPwrImpedance Attributes	109

11.2.1	Output Impedance Enabled	110

11.2.2	Output Impedance Resistive	111

11.2.3	Output Impedance Inductive	112

11.3	IviACPwrImpedance Functions	113

11.3.1	Configure Output Impedance	114

11.3.2	Query Output Impedance Capabilities	115

11.4	IviACPwrImpedance Behavior Model	116

12	IviACPwrDCGeneration Extension Group	117

12.1	IviACPwrDCGeneration Overview	117

12.2	IviACPwrDCGeneration Attributes	118

12.2.1	DC Mode	119

12.2.2	DC Voltage Level	120

12.2.3	DC Range Min	121

12.2.4	DC Range Max	122

12.2.5	Number of DC Ranges	123

12.3	IviACPwrDCGeneration Functions	124

12.3.1	Configure DC	125

12.3.2	Configure DC Range	126

12.3.3	Query DC Capabilities	127

12.4	IviACPwrDCGeneration Behavior Model	128

13	IviACPwrVoltageRamp Extension Group	129

13.1	IviACPwrVoltageRamp Overview	129

13.2	IviACPwrVoltageRamp Attributes	129

13.2.1	Voltage Ramp Busy	130

13.3	IviACPwrVoltageRamp Functions	131

13.3.1	Ramp Voltage	132

13.3.2	Query Voltage Ramp Busy (IVI-C only)	133

13.3.3	Abort Voltage Ramp	134

13.4	IviACPwrVoltageRamp Behavior Model	135

14	IviACPwrCurrentRamp Extension Group	136

14.1	IviACPwrCurrentRamp Overview	136

14.2	IviACPwrCurentRamp Attributes	136

14.2.1	Current Ramp Busy	137

14.3	IviACPwrCurrentRamp Functions	138

14.3.1	Ramp Current	139

14.3.2	Query Current Ramp Busy (IVI-C only)	140

14.3.3	Abort Current Ramp	141

14.4	IviACPwrCurrentRamp Behavior Model	142

15	IviACPwrFrequencyRamp Extension Group	143

15.1	IviACPwrFrequencyRamp Overview	143

15.2	IviACPwrFrequencyRamp Attributes	143

15.2.1	Frequency Ramp Busy	144

15.3	IviACPwrFrequencyRamp Functions	145

15.3.1	Ramp Frequency	146

15.3.2	Query Frequency Ramp Busy (IVI-C only)	147

15.3.1	Abort Frequency Ramp	148

15.4	IviACPwrFrequencyRamp Behavior Model	149

16	IviACPwr Attribute ID Definitions	150

17	IviACPwr Attribute Value Definitions	152

18	IviACPwr Function Parameter Value Definitions	153

19	Error and Completion Code Value Definitions	157

20	IviACPwr Hierarchies	158

20.1	IviACPwr COM Hierarchy	158

20.1.1	IviACPwr COM Interfaces	161

20.1.2	IviACPwr COM Interface Reference Properties	162

20.1.3	IviACPwr COM Category	164

20.2	IviACPwr C Function Hierarchy	165

20.3	IviACPwr C Attribute Hierarchy	167

Appendix A.	Specific Driver Development Guidelines	169

A.1	Introduction	169

A.2	Disabling Unused Extensions	169

A.3	Special Repeated Capability Identifier	172

A.4	Current and Voltage Protection	172

A.5	Operations that Require the Output be Disabled	172

A.6	Waveform RMS and Peak Voltage	173

Appendix B.	Interchangeability Checking Rules	174

B.1	Introduction	174

B.2	When to Perform Interchangeability Checking	174

B.3	Interchangeability Checking Rules	174

Index of Tables

Table 1-1.  IviACPwr Class Specification Revisions	9

Table 2-1. IviACPwr Group Names	13

Table 16-1. IviACPwr Attributes ID Values	150

Table 20-1. IviACPwr COM Hierarchy	158

Table 20-2 IviACPwr Interface GUIDs	161

Table 20-3. IviACPwr C Function Hierarchy	165

Table 20-4. IviACPwr C Attributes Hierarchy	167

Index of Figures

Figure 4-1 AC Power Source Operating Point	18

IviACPwr Class Specification

IviACPwr Revision History

This section is an overview of the revision history of the IviACPwr specification.

| Table 1-1. IviACPwr Class Specification Revisions |  |  |
| --- | --- | --- |
| Revision Number | Date of Revision | Revision Notes |
| Revision 0.1 draft | October 2006 | Original draft. |
| Revision 0.2 draft | February 2007 | Original draft revisions. |
| Revision 0.3 draft | May 2007 | Revising Adding Extensions |
| Revision 4.1f draft | December 2007 | Completed adding of all Extensions |
| Revision 4.1M draft | July 18, 2008 | Specification ready for Driver code Prototyping |
| Revision 4.1N draft | August 29, 2008 | Updated as needed to create baseline project file. |
| Revision 4.1P draft | October 27, 2008 | Updated based on addition of Help Sections. |
| Revision 4.1R draft | November 6, 2008 | Updated Ramp descriptions |
| Revision 4.1S draft | December 18, 2008 | Waveform DC Offset, DC Generation |
| Revision 4.1T draft | March 4, 2009 | Updated Current/Voltage Protection |
| Revision 4.1U draft | April 15, 2009 | Changed repeated capability to OutputPhase |
| Revision 4.1V draft | May 23, 2009 | Updated ramp sections, added Fetch Measurement Array function and separated enums. |
| Revision 4.1W draft | May 3, 2010 | Updated DCGeneration extension group |
| Revision 4.1X draft | May 11, 2010 | Added AbortFrequencyRamp, fixed RampCurrent |
| Revision 1.0 | July 14, 2010 | Posted for 45 day review. |
| Revision 1.0 | October 8, 2010 | QueryArbWaveformCatalog parameter order and misc minor cleanup. |
| Revision 1.1 | January 27, 2011 | Editorial changes in the following COM method prototypes: Query Arbitrary Waveform Catalog, Output Impedance Enabled, Output Impedance Resistive, Abort Voltage Ramp, Abort Current Ramp. Updates in the COM and C function hierarchy tables. |
| Revision 1.2 | March 11, 2011 | Misc minor editorial changes |

# Overview of the IviACPwr Specification

## Introduction

This specification defines the IVI class for AC Power Sources. The IviACPwr class is designed to support the typical AC power source as well as common extended functionality found in more complex instruments. This section summarizes the IviACPwr Specification itself and contains general information that the reader may need in order to understand, interpret, and implement aspects of this specification. These aspects include the following:

IviACPwr Class Overview

The definitions of terms and acronyms

References

## IviACPwr Class Overview

This specification describes the IVI class for AC power sources. The IviACPwr class is designed to support the typical single or multi-phase AC power source as well as common extended functionality found in more complex instruments. The IviACPwr class conceptualizes an AC power source as an instrument capable of generating an AC power signal.

The IviACPwrBase capability group provides the ability to configure an AC power source for basic signal output. This includes setting the current limit, voltage level, frequency, and enabling the output. The IviACPwrBase capability group is described in Section 4, IviACPwrBase Capability Group.

The IviACPwrMeasurement extension group provides the ability to measure AC power source output signals such as voltage and current. This extension group is described in Section 5, IviACPwrMeasurement Extension Group.

The IviACPwrPhase extension group provides the ability to set the output phase relationships of a multiphase power source. This extension group is described in Section 6, IviACPwrPhase Extension Group.

The IviACPwrExternalSync extension provides the ability to synchronize the power source output frequency with an external signal. It also allows the querying the synchronization state of the power source. This extension group is described in Section 7, IviACPwrExternalSync Extension GroupIviACPwrPhase Behavior Model.

The IviACPwrCurrentProtection extension group provides programmable current protection to detect an over-current condition and automatically disable the output. It also allows the querying of the current protection state of the power source. This extension group is described in Section 8IviACPwrExternalSync Behavior Model, IviACPwrCurrentProtection Extension Group.

The IviACPwrVoltageProtection extension group provides programmable voltage protection to detect an under/over voltage condition and automatically disable the output. It also allows the querying of the voltage protection state of the power source. This extension group is described in Section 9, IviACPwrVoltageProtection Extension Group.

The IviACPwrArbWaveform extension group provides the ability to create user-defined arbitrary waveforms. The IviACPwrArbWaveform extension group includes functions for Writing and Clearing arbitrary waveforms, and Querying arbitrary waveform catalogs. This extension group is described in Section 10, IviACPwrArbWaveform Extension Group.

The IviACPwrImpedance extension group provides the ability to set and query the power source output impedance. With this extension, the driver can configure the power source output resistance and inductance. This extension group is described in Section 11, IviACPwrImpedance Extension Group.

The IviACPwrDCGeneration extension group provides the ability to configure the output of the AC Power Source with an AC only output, DC only output or an AC signal with DC offset. This extension group is described in Section 12, IviACPwrDCGeneration Extension Group. IviACPwrImpedance Behavior Model

The IviACPwrVoltageRamp extension group provides the ability to configure a linear voltage ramp with starting and ending voltage in volts RMS and ramp duration in seconds. This extension group is described in Section 13, IviACPwrVoltageRamp Extension Group.

The IviACPwrCurrentRamp extension group provides the ability to configure a linear current ramp with starting and ending current in amps and ramp duration in seconds. This extension group is described in Section 14, IviACPwrCurrentRamp Extension GroupIviACPwrVoltageRamp Behavior Model.

The IviACPwrFrequencyRamp extension group provides the ability to configure a linear frequency ramp with starting and ending frequencies in hertz and ramp duration in seconds. This extension group is described in Section 15, IviACPwrFrequencyRamp Extension GroupIviACPwrCurrentRamp Behavior Model.

## References

Several other documents and specifications are related to this specification. These other related documents are the following:

- IVI Charter Document

- IVI-3.1: Driver Architecture Specification

- IVI-3.2: Inherent Capabilities Specification

- IVI-3.3: Standard Cross-Class Capabilities Specification

- VPP-3.x - VXIplug&play Instrument Driver Specifications

- VPP-4.x - Virtual Instrument Software Architecture (VISA) Specifications

- IVI-5.0: Glossary

## Definitions of Terms and Acronyms

This section defines terms acronyms that are specific to the IviACPwr class.

Constant Voltage Mode	The AC power source is said to be in the constant voltage mode when the output voltage reaches the Voltage Level value before the output current reaches the Current Limit value. In the constant voltage mode, the power source output voltage remains constant at the Voltage Level value and the output current can vary based on the load.

Constant Current Mode	The AC power source is said to be in the constant current mode when the output current reaches the Current Limit value before the output voltage reaches the Voltage Level value. In the constant current mode, the power source output current remains constant at the Current Limit value and the output voltage can vary based on the load.

Protection Mode	The AC power source is said to be in a protection mode when the output is disabled by the power source due to a voltage or current protection condition. Protection mode is intended to protect the load from over or under voltage or over current.

# IviACPwr Class Capabilities

## Introduction

The IviACPwr specification divides AC power source capabilities into a base capability group and multiple extension capability groups. Each capability group is discussed in a separate section. This section defines names for each capability group and gives a brief overview for each capability group.

## IviACPwr Group Names

The capability group names for the IviACPwr class are defined in the following table. The group name is used to represent a particular capability group and is returned as one of the possible group names from the Group Capabilities attribute.

| Table 2-2. IviACPwr Group Names |  |
| --- | --- |
| Group Name | Description |
| IviACPwrBase | Base: Complies with the IviACPwr base capabilities. This group supports the ability to generate an AC power signal, to specify output limits, and to control the behavior of the power source when the output is greater than or equal to one of the limits |
| IviACPwrMeasurement | Extension: Supports the ability to query the instrument for the measurement characteristics of the output signal provides the capability to take measurements on the output signal. |
| IviACPwrPhase | Extension: Supports the ability to set and measure output phase relationship. |
| IviACPwrExternalSync | Extension: Supports the ability of synchronizing an AC Power Source’s output frequency with an external signal. |
| IviACPwrCurrentProtection | Extension: Supports the ability to program current protection. |
| IviACPwrVoltageProtection | Extension: Supports the ability to program voltage protection. |
| IviACPwrArbWaveform | Extension: Supports the ability to produce waveforms other than sine waves on the output signal. |
| IviACPwrImpedance | Extension: Supports the ability to set and query the instrument for the output impedance characteristics of the output signal. |
| IviACPwrDCGeneration | Extension: Supports the ability to configure the output of the AC Power Source with DC Content. |
| IviACPwrVoltageRamp | Extension: Supports the ability to configure a linear Voltage ramp with starting and ending voltage in volts (RMS) and ramp duration in seconds. |
| IviACPwrCurrentRamp | Extension: Supports the ability to configure a Current ramp starting and ending Current and ramp duration in seconds. |
| IviACPwrFrequencyRamp | Extension: Supports the ability to configure a linear Frequency ramp with starting and ending Frequencies and ramp duration |

## Repeated Capability Names

The IviACPwr Class Specification defines one repeated capability. Refer to the sections of IVI-3.1, Driver Architecture Specification that deal with repeated capabilities. The relevant sections are Section 2.7, Repeated Capabilities, Section 4.1.9, Repeated Capabilities, Section 4.2.5, Repeated Capabilities, and Section 5.8, Repeated Capability Identifiers and Selectors.

OutputPhase

### OutputPhase

In the configuration store, the name for the Output Phase repeated capability shall be exactly one of “OutputPhase” or “IviACPwrOutputPhase”. Drivers that implement multiple repeated capabilities with the name “OutputPhase” shall use the latter form to disambiguate the names.

Phase names can be vendor unique but the recommended names are “PhaseA” for output phase index 1, “PhaseB” for output phase index 2 and so on. This method of phase naming is assumed throughout this document and is based on industry standard conventions for single and multiphase AC power sources.

In addition to individual phase names, drivers must also support the special phase name ”AllPhases”. When used with attributes or functions, this name sets all phases of that attribute or function to the same value.

### Multi Phase Attributes

Instrument vendors must allow setting all phases of multi-phase attributes and functions to the same value using the ”AllPhases” repeated capability name for the following capability groups:

- IviACPwrBase

- IviACPwrCurrentProtection

- IviACPwrVoltageProtection

- IviACPwrImpedance

- IviACPwrDCGeneration

- IviACPwrVoltageRamp

- IviACPwrCurrentRamp

Instrument vendors may prevent the user from setting the phases for a particular multi-phase attribute independently. In this case, only the “AllPhases” repeated capability name is allowed otherwise it returns the All Phases Required error.

If you attempt to read an attribute using the “AllPhases” repeated capability name and one of the phases contains a different value, the error Phase Values Different is returned. You must then query each phase separately to obtain the attribute value for each phase.

### Phase Group

A driver session must only support one phase group. A phase group is defined as one or more output phases that have the same frequency and are synchronized to each other. If an AC power source supports more than one phase group, then separate driver sessions must be opened for each group.

If an AC power source supports more than one phase group, this specification assumes that each of these phase groups are independent of each other and behave as if they are completely separate devices.  For the case where a device has multiple phase groups, the user initializes a separate instrument driver session to control each phase group.  The Phase Group that is being used must be identified during initialization.  The mechanism for identifying the Phase Group to be used during initialization is instrument specific.  A driver developer can choose to use the configuration store, an initialization option string, or other mechanism.  If there are synchronization or configuration requirements that are common across the multiple phase groups, they will be handled in an instrument specific manner.

### Configuring Single/MultiPhase Operation

AC Power sources that support single or multi-phase operation must only be configured during driver initialization. In other words, once a driver session is opened, the number of output phases remains fixed and must match the number of output phase connections the user connects to their load. To change the number of phases, the user must close the driver session and open a new one. There are two mechanisms available to configure the number of output phases during driver initialization:

- Create a driver session in the configuration store and include the string “NumPhases:n” in the DriverSetup field.

- Pass the token “NumPhases:n” to the InitWithOptions DriverSetup parameter. Note that this overrides the value in the configuration store.

The initialize function will return an error if the AC power source does not support the requested number of phases. If neither of the above methods is used, the driver initializes the number of phases based on the current instrument state or vendor default number of phases . This sets an interchangeability warning.

# General Requirements

This section describes the general requirements a specific driver shall meet in order to be compliant with this specification. In addition, it provides general requirements that specific drivers shall meet in order to comply with a capability group, attribute, or function.

## Minimum Class Compliance

To be compliant with the IviACPwr Class Specification, a specific driver shall implement the inherent capabilities that IVI-3.2: Inherent IVI Capabilities Specification defines, and the IviACPwrBase capability group.

### Disable

Refer to IVI-3.2: Inherent Capabilities Specification for the prototype of this function.

The Disable function shall cause the AC Power Source to apply the minimum amount of power possible at the output terminals. Setting the voltage to a value close to zero, setting the current limit to value close to zero, or physically disconnecting the power source from the output terminals meets this requirement. Other techniques are also allowed.

## Capability Group Compliance

IVI-3.1: Driver Architecture Specification defines the general rules for a specific driver to be compliant with a capability group.

# IviACPwrBase Capability Group

## Overview

The IviACPwrBase capability group supports the basic AC power source capabilities. The user can specify the Voltage Level, Current Limit, Frequency and control the operation of the output relay/contactor.

This specification uses the following terms to describe the power source’s output: Voltage Level, Frequency, Phase Angle (IviACPwrPhase extension group), Waveform, Current limit, DC Voltage Level (IviACPwrDCGeneration extension group) and Impedance (IviACPwrImpedance extension group).

### User Specified Settings

Voltage Level	The RMS AC output voltage of the AC power source. The user configures the voltage level with the Voltage Level attribute.

Frequency	The frequency of the output AC voltage waveform of the AC power source. The user configures the frequency with the Frequency attribute.

Waveform	The waveform the power source uses to generate the output voltage. The user configures the waveform with the Waveform attribute.

Current Limit	Sets the maximum allowable current the AC power source will deliver. The user configures the current limit with the Current Limit attribute.

Output Enabled	Enables/Disables the output contactor/relay of the AC power source. The user configures the output enable with the Output Enabled attribute.

Voltage Range	Specifies the output voltage range. AC power sources may support multiple voltage ranges to allow greater output voltage or current. The user configures the voltage range with the Voltage Range attribute.

Frequency Range	Specifies the output frequency range. AC power sources may support multiple frequency ranges. The user configures the frequency range with the Frequency Range attribute.

### User Queriable Settings

Number of Phases	Returns the number of physical output phases.

Number of Voltage Ranges	Returns the number of supported voltage ranges.

Voltage Range Capabilities	Returns the minimum and maximum voltages supported by a particular voltage range.

Number of Frequency Ranges	Returns the number of supported frequency ranges.

Frequency Range Capabilities	Returns the minimum and maximum frequency supported by a particular frequency range

### Power Source Voltage and Current Scenarios

The signal that the power source produces depends on the values of the voltage level and current limit that the user supplies, and the impedance of the load to which the power source is attached. Therefore, the power source might not produce the exact voltage or current that the user configures. Figure 4-1 shows the possible output scenarios.

Figure 4-1 AC Power Source Operating Point

Point 1	The AC power source output has reached the voltage level before it reached the current limit. This is an example of the AC power source operating in the constant voltage mode. The voltage level is defined by the Voltage Level attribute in the IviACPwrBase capability group described in section 4

Point 2	The AC power source output has reached the current limit before it reached the voltage level. This is an example of the AC power source operating in the constant current mode. The current limit is defined by the Current Limit attribute in the IviACPwrBase capability group described in section 4.

Point 3,	The AC power source output is below the under voltage protection limit. If under voltage protection is enabled, the power source disables the output. This mode is controlled by the IviACPwrExternalSync Behavior Model described in section 9.

Point 4	The AC power source output exceeds the over voltage protection limit. If over voltage protection is enabled, the power source disables the output. This mode is controlled by the IviACPwrExternalSync Behavior Model described in section 9.

Point 5	The AC power source output exceeds the current protection threshold. If current protection is enabled and the AC power source output current has reached or exceeds the current protection threshold for the time specified by the current protection delay, the AC power source disables the output. This mode is controlled by the IviACPwrExternalSync Behavior Model described in section 8.

## IviACPwrBase Attributes

The IviACPwrBase capability group defines the following attributes:

- Current Limit

- Frequency

- Number of Phases

- Output Enabled

- Voltage Level

- Waveform

- Number of Voltage Ranges

- Voltage Range

- Number of Frequency Ranges

- Frequency Range

- Output Phase Item (IVI-COM only)

- Output Phase Name (IVI-COM only)

- Output Phase Count

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 15.4, IviACPwrFrequencyRamp Behavior Model.

### Current Limit

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Current Limit |

COM Property Name

OutputPhases.Item().CurrentLimit

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_LIMIT

Description

Specifies the output current limit. The units are Amps. The RMS behavior of this attribute is vendor specific.

### Frequency

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure Frequency |

COM Property Name

OutputPhases.Frequency

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_FREQUENCY

Description

Specifies the frequency the AC output voltage waveform. The units are Hertz.

### Number of Phases

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

OutputPhases.NumPhases

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_NUM_PHASES

Description

The number of physical output phases of the AC power source the user can connect to their load for this phase group (driver session). The number of phases returned by this attribute does not include the special “AllPhases” output phase name used to set all output phases.

AC power sources that support single or multi phase operation can only be configured when opening a new driver session using the “NumPhases” token in the DriverSetup string or in the configuration store. Otherwise, the present number of phases configured in the AC power source is used.

### Output Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | OutputPhase | None | Configure Output Enabled |

COM Property Name

OutputPhases.Item().Enabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OUTPUT_ENABLED

Description

Enables or disables the output of the AC power source. When Output Enabled is set to False. The output of the AC power source is effectively disconnected from the load.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | The output contactor/relay is closed. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | The output contactor/relay is open. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this attribute can return. The table below specifies additional class-defined status codes for this attribute.

| Completion Codes | Description |
| --- | --- |
| Current Protection Tripped | The power source current protection has tripped. Refer to the ResetCurrentProtection function in the Current Protection extension group. |
| Voltage Protection Tripped | The power source voltage protection has tripped. Refer to the ResetVoltageProtection function in the Voltage Protection extension group. |

Compliance Notes

If current protection or voltage protection has tripped, the driver must not allow setting Output Enabled to True and must return the Current Protection Tripped or Voltage Protection Tripped completion code. Refer to Section  8 : IviACPwrCurrentProtection Extension Group and Section  9 : IviACPwrVoltageProtection Extension Group for more information on resetting a current or voltage protection tripped condition.

### Voltage Level

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Voltage Level |

COM Property Name

OutputPhases.Item().VoltageLevel

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_VOLTAGE_LEVEL

Description

Specifies the Line to Neutral output voltage level of the AC power source when operating in constant voltage mode. The units are Volts RMS. The power source will reduce the actual output voltage to limit the output current to the value specified by the Current Limit attribute.

Use the Query Voltage Range Capabilities function to determine the minimum and maximum allowable values for the Voltage Level attribute for a particular voltage range and waveform.

### Waveform

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | R/W | OutputPhase | None | Configure Waveform |

COM Property Name

OutputPhases.Item().Waveform

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_WAVEFORM

Description

Specifies the waveform name (refer to the IviACPwrArbWaveform capability group) used to generate output voltage.

Compliance Notes

All vendors must support ‘Sine’.

### Output Phase Item (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| IIviACPwrOutputPhase | RO | OutputPhase | None |  |

COM Property Name

OutputPhases.Item(BSTR Name);

COM Enumeration Name

N/A

C Constant Name

N/A

Description

Gets an interface pointer for an interface to control a particular Output Phase.

Return Values

If the IVI-COM driver cannot recognize the Name parameter, it returns an Unknown Capability Name completion code as described in IVI-3.2: Inherent Capabilities Specification, Section 9.3.

### Output Phase Name (IVI-COM only)

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViString | RO | OutputPhase | None |  |

COM Property Name

OutputPhases.Name ([in] LONG Index)

COM Enumeration Name

N/A

C Constant Name

N/A

Description

Returns the physical output phase identifier that corresponds to the one-based index that the user specifies. If the driver defines a qualified output phase name, this property returns the qualified name. If the value that the user passes for the Index parameter is less than one or greater than the value of the Output Phase Count attribute, the property returns an empty string in the Name parameter and returns the Invalid Value error.

### Output Phase Count

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None |  |

COM Property Name

OutputPhases.Count

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OUTPUT_PHASE_COUNT

Description

Returns the number of output phase names supported by the power source.

### Number of Voltage Ranges

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | OutputPhase | None | None |

COM Property Name

OutputPhases.Item().NumVoltageRanges

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_NUM_VOLTAGE_RANGES

Description

The number of output voltage ranges supported by the AC power source. Use the IVIACPWR_ATTR_VOLTAGE_RANGE attribute to read or change the output voltage range.

Compliance Notes

All vendors must support at least 1 voltage range.

### Voltage Range

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | Up | Configure Voltage Range |

COM Property Name

OutputPhases.Item().VoltageRange

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_VOLTAGE_RANGE

Description

Sets or returns the voltage range of the AC power source based on a specified maximum voltage. The driver coerces this value to the upper limit of the lowest range that supports that voltage.

Use the Query Voltage Range Capabilities function to determine the minimum and maximum RMS voltages supported by a particular range and waveform.

Compliance Notes

Changing the voltage range may require that the output be disabled. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

### Number of Frequency Ranges

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

OutputPhases.NumFrequencyRanges

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_NUM_FREQUENCY_RANGES

Description

The number of output frequency ranges supported by the AC power source. Use the IVIACPWR_ATTR_FREQUENCY_RANGE attribute to read or change the output frequency range.

Compliance Notes

All vendors must support at least 1 frequency range.

### Frequency Range

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | Up | Configure Frequency Range |

COM Property Name

OutputPhases.FrequencyRange

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_FREQUENCY_RANGE

Description

Sets or returns the frequency range of the AC power source based on a specified maximum frequency. The driver coerces this value to the upper limit of the lowest range that supports that frequency.

Use the Query Frequency Range Capabilities function to determine the minimum and maximum frequency supported by a particular range.

Compliance Notes

Changing the frequency range may require that output power to be disabled. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

## IviACPwrBase Functions

The IviACPwrBase capability group defines the following functions:

- Configure Current Limit (IVI-C only)

- Configure Output Enabled (IVI-C only)

- Configure Voltage Level (IVI-C only)

- Configure Voltage Range (IVI-C only)

- Configure Frequency (IVI-C only)

- Configure Frequency Range (IVI-C only)

- Configure Waveform (IVI-C only)

- Query Voltage Range Capabilities

- Query Frequency Range Capabilities

- Get Output Phase Name (IVI-C only)

This section describes the behavior and requirements of each function.

### Configure Current Limit (IVI-C only)

Description

Configures the current limit. It specifies the output current limit value for constant current mode.

COM Method Prototype

N/A

(use the OutputPhases.Item().CurrentLimit  property)

C Prototype

ViStatus IviACPwr_ConfigureCurrentLimit (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 Limit);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the current limit. | ViConstString |
| Limit | Specifies the output current limit. The driver uses this value to set the Current Limit attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Output Enabled (IVI-C only)

Description

Enables/Disables the output contactor/relay on the power source output.

COM Method Prototype

N/A

(use the OutputPhases.Item().Enabled  property)

C Prototype

ViStatus IviACPwr_ConfigureOutputEnabled (ViSession Vi,
                                          ViConstString PhaseName,
                                          ViBoolean Enabled);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output to enable or disable. | ViConstString |
| Enabled | Specifies whether the signal the power source produces appears at the output connector. The driver uses this value to set the Output Enabled attribute. See the attribute description for more details. | ViBoolean |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Voltage Level (IVI-C only)

Description

Configures the output Line to Neutral voltage level of the AC power source.

COM Method Prototype

N/A

(use the OutputPhases.Item().VoltageLevel  property)

C Prototype

ViStatus IviACPwr_ConfigureVoltageLevel (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 VoltageLevel);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the voltage level. | ViConstString |
| VoltageLevel | Specifies the output voltage level of the power source. The driver uses this value to set the Voltage Level attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Voltage Range (IVI-C only)

Description

Configures the voltage range of the AC power source.

COM Method Prototype

N/A

(use the OutputPhases.Item().VoltageRange  property)

C Prototype

ViStatus IviACPwr_ConfigureVoltageRange (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 Range);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the voltage range. | ViConstString |
| Range | Specifies the voltage range of the power source. The driver uses this value to set the Voltage Range attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Frequency (IVI-C only)

Description

Configures the frequency of a single phase power source or all phases of a multi-phase power source.

COM Method Prototype

N/A

(use the OutputPhases.Frequency  property)

C Prototype

ViStatus IviACPwr_ConfigureFrequency (ViSession Vi,
                                      ViReal64 Frequency);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| Frequency | Specifies the output frequency of the power source. The driver uses this value to set the Frequency attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Frequency Range (IVI-C only)

Description

Configures the frequency range.

COM Method Prototype

N/A

(use the OutputPhases.FrequencyRange  property)

C Prototype

ViStatus IviACPwr_ConfigureFrequencyRange (ViSession Vi,
                                      	   ViReal64 Range);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| Range | Specifies the frequency range of the power source. The driver uses this value to set the Frequency Range attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Waveform (IVI-C only)

Description

Configures the waveform name used to generate output voltage.

COM Method Prototype

N/A

(use the OutputPhases.Item().Waveform  property)

C Prototype

ViStatus IviACPwr_ConfigureWaveform (ViSession Vi,

ViConstString PhaseName,
                                     ViConstString WaveformName);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the waveform. | ViConstString |
| WaveformName | Specifies the waveform name (refer to the IviACPwrArbWaveform capability group) used to generate output voltage on the specified phase. The driver uses this value to set the Waveform attribute. See the attribute description for more details. | ViConstString |

Return Values

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

| Completion Codes | Description |
| --- | --- |
| Invalid Waveform Name | The waveform name is not currently stored in the AC power source |

Compliance Notes

All vendors shall support a waveform name of ‘Sine’.

### Query Voltage Range Capabilities

# Description

Returns the minimum and maximum RMS voltage allowed for the user specified voltage range and waveform. The value assigned to the Voltage Level attribute must not be outside these minimum and maximum values. The returned voltages assume no load on the output.

Refer to section A.6 Waveform RMS and Peak Voltage for more information on how waveforms can affect the maximum voltage the power source can generate.

COM Method Prototype

HRESULT OutputPhases.Item().QueryVoltageRangeCapabilities([in] LONG Range,

[in] BSTR WaveformName,
											[in, out] double *MinVoltage,

[in, out] double *MaxVoltage);

# C Prototype

ViStatus IviACPwr_QueryVoltageRangeCapabilities (ViSession Vi,

ViConstString PhaseName,

ViInt32 Range,

ViConstString WaveformName,

ViReal64 *MinVoltage,

ViReal64 *MaxVoltage);

# Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to query the voltage range. | ViConstString |
| Range | The voltage range to query. This parameter must be between 1 and the value reported by the IVIACPWR_ATTR_NUM_VOLTAGE_RANGES attribute | ViInt32 |
| WaveformName | The name of the waveform to query the range. If the waveform name parameter is VI_NULL, the range for a ‘Sine’ waveform is returned. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| MinVoltage | Returns the minimum RMS voltage supported by the specified range and waveform. | ViReal64 |
| MaxVoltage | Returns the maximum RMS voltage supported by the specified range and waveform. | ViReal64 |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return. The table below specifies additional class-defined status codes for this function.

| Completion Codes | Description |
| --- | --- |
| Waveform Not Found | The waveform name is not currently stored in the AC Power Source. |

Compliance Notes

All vendors must support a waveform name of ‘Sine’ and VI_NULL.

### Query Frequency Range Capabilities

# Description

Returns the minimum and maximum frequency in hertz allowed for the user specified frequency range. The value assigned to the Frequency attribute must not be outside these minimum and maximum values.

COM Method Prototype

HRESULT OutputPhases.QueryFrequencyRangeCapabilities([in] LONG range,

[in, out] double *MinFrequency,

[in, out] double *MaxFrequency);

# C Prototype

ViStatus IviACPwr_QueryFrequencyRangeCapabilities (ViSession Vi,

ViInt32 range,

ViReal64 *MinFrequency,

ViReal64 *MaxFrequency);

# Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| range | The frequency range to query. This parameter must be between 1 and the value reported by the IVIACPWR_ATTR_NUM_FREQUENCY_RANGES attribute | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| MinFrequency | Returns the minimum frequency supported by the specified range. | ViReal64 |
| MaxFrequency | Returns the maximum frequency supported by the specified range. | ViReal64 |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Get Output Phase Name (IVI-C only)

Description

Returns the specific driver defined output phase name that corresponds to the one-based index that the user specifies. If the driver defines a qualified output phase name, this function returns the qualified name.  If the value that the user passes for the Index parameter is less than one or greater than the value of the Output Phase Count attribute, the function returns an empty string in the Name parameter and returns the Invalid Value error.

COM Method Prototype

N/A

(use the OutputPhases.Name()  property)

C Prototype

ViStatus IviACPwr_GetOutputPhaseName (ViSession Vi,

ViInt32 Index,

ViInt32 NameBufferSize,
                                         ViChar Name[]);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| Index | A one-based index that defines which name to return. | ViInt32 |
| NameBufferSize | The number of bytes in the ViChar array that the user specifies for the Name parameter. | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Name | The buffer into which the function returns the name that corresponds to the index the user specifies. The caller may pass VI_NULL for this parameter if the NameBufferSize parameter is 0. | ViChar[] |

Return Values

The IVI-3.2: Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

- For an instrument with only one Output Phase, that is the Output Phase Count attribute is one, the driver may return an empty string.

- Refer to Section 3.1.2, Additional Compliance Rules for C Functions with ViChar Array Output Parameters, in IVI-3.2  Inherent Capabilities Specification, Section 3.1.2.1 for rules regarding the NameBufferSize and Name parameters.

## IviACPwrBase Behavior Model

After the user calls the Initialize or Reset functions, the driver must immediately disable output power and set the Output Enabled attribute to False for all output phases. The driver must then set all extension capability groups to the disabled state as described in section A.2. It is the responsibility of the user to determine if the attribute values in the IviACPwrBase capability group are appropriate or safe for their load prior to setting the Output Enabled attribute to True.

All changes to the IviACPwrBase functions and attributes take place immediately.

Certain AC power sources may require that the output power be disabled in order to change the voltage or frequency range. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

# IviACPwrMeasurement Extension Group

## IviACPwrMeasurement Overview

The IviACPwrMeasurement extension group defines extensions for AC Power Sources capable of measuring various output signals such as voltage and current.

## IviACPwrMeasurement Functions

The IviACPwrMeasurement extension defines the following functions:

- Initiate Measurement

- Fetch Measurement

- Fetch Measurement Array

The Initiate Measurement function initiates a measurement of one or more measurement groups. To minimize the time to perform measurements and to correlate them to the same output cycle, measurement types are divided into groups. Ideally, for each call to the Initiate Measurement function, all measurement types for all phases for the groups specified are measured on the same cycle of the output, but individual vendors may implement this differently. The exact behavior must be documented by the instrument driver.

The Fetch Measurement function returns the measurement result of a previous call to the Initiate Measurement function. The Fetch Measurement function returns a single value of the specified measurement type. Call Fetch Measurement multiple times to retrieve values for the measurement types for each output phase in that measurement group.

The Fetch Measurement Array function returns an array of measured values like harmonics or waveform of a previous call to the Initiate Measurement function. The size of the user allocated measurement array is passed to the function and the function returns the actual number of values stored in the array. If you pass VI_NULL for the measurement pointer or 0 for ArraySize, the function will not perform the measurement but will return the array size needed (in the ActualSize parameter) to store the results for that measurement type. In cases where there are more values to return than the array size you specified, the function will fill the array and return the additional array size needed in the function result.

### Initiate Measurement

Description

Initiates a measurement of one or more measurement groups for all output phases. You can specify multiple groups by ORing the group identifiers together.

COM Method Prototype

HRESULT OutputPhases.InitiateMeasurement (
[in] long Group);

C Prototype

ViStatus IviACPwr_InitiateMeasurement (ViSession Vi,
                           ViInt32 Group);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| Group | The group(s) to measure. | ViInt32 |

Defined Values for Group Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Base | Measure all items in the Base measurement group |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASUREMENT_GROUP_BASE |  |
|  |  | COM | IviACPwrMeasurementGroupBase |  |
| Harmonic | Measure all items in the Harmonic measurement group |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASUREMENT_GROUP_HARMONIC, |  |
|  |  | COM | IviACPwrMeasurementGroupHarmonic |  |
| Distortion | Measure all items in the Distortion measurement group |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASUREMENT_GROUP_DISTORTION, |  |
|  |  | COM | IviACPwrMeasurementGroupDistortion |  |
| Waveform | Measure all items in the Waveform measurement group |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASUREMENT_GROUP_WAVEFORM, |  |
|  |  | COM | IviACPwrMeasurementGroupWaveform |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

| Completion Codes | Description |
| --- | --- |
| Group Not Supported | The AC power source does not support the requested measurement group(s). |

### Fetch Measurement

Description

Returns the value measured by a previous call to the Initiate Measurement function. The Fetch Measurement function returns a single measured value for the requested measurement type. Call this function multiple times to retrieve the available measurement types and output phases. The available measurement types are divided into measurement groups. Use the Initiate Measurement function to measure fresh values for that group.

COM Method Prototype

HRESULT OutputPhases.Item().FetchMeasurement([in] IviACPwrMeasurementTypeEnum MeasurementType,
[out] double *Measurement);

C Prototype

ViStatus IviACPwr_FetchMeasurement (ViSession Vi,
                           ViConstString PhaseName,
                           ViInt32 MeasurementType,

ViReal64 *Measurement);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output phase to return the measurement. | ViConstString |
| MeasurementType | The type of measurement to return. | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Measurement | Returns the measured value. | ViReal64 |

Defined Values for MeasurementType Parameter – Base Group

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Voltage RMS L-N | Voltage RMS line to neutral is measured. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_RMS_LN |  |
|  |  | COM | IviACPwrMeasureVoltageRMS |  |
| Current RMS | Current RMS is measured. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_RMS |  |
|  |  | COM | IviACPwrMeasureCurrentRMS |  |

| Frequency | Frequency is measured. |  |  |
| --- | --- | --- | --- |
|  |  | C | IVIACPWR_VAL_MEASURE_FREQUENCY |
|  |  | COM | IviACPwrMeasureFrequency |
| Voltage DC | Voltage level is measured when in DC Only Mode. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_DC |
|  |  | COM | IviACPwrMeasureVoltageDC |
| Current DC | Current level is measured when in DC Only Mode. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_DC |
|  |  | COM | IviACPwrMeasureCurrentDC |
| Power Factor | Power Factor is measured. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_POWER_FACTOR |
|  |  | COM | IviACPwrMeasurePowerFactor |
| Crest Factor | Crest Factor is measured. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CREST_FACTOR |
|  |  | COM | IviACPwrMeasureCrestFactor |
| Current Peak | Peak Current is measured. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_PEAK |
|  |  | COM | IviACPwrMeasureCurrentPeak |
| Power VA | Power is measured in Volts-Amps. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_POWER_VA |
|  |  | COM | IviACPwrMeasurePowerVA |
| Real Power | Real Power is measured in Watts. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_POWER_REAL |
|  |  | COM | IviACPwrMeasurePowerReal |
| Power DC | Power is measured in Watts when in DC Only Mode. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_POWER_DC |
|  |  | COM | IviACPwrMeasurePowerDC |
| Phase Angle | Phase Angle is measured in Degrees. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_PHASE_ANGLE |
|  |  | COM | IviACPwrMeasurePhaseAngle |
| Voltage RMS L-L | Voltage RMS (line-to-line) is measured. |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_RMS_LL |
|  |  | COM | IviACPwrMeasureVoltageRMSLineToLine |

Defined Values for MeasurementType Parameter – Distortion Group

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Current OHD | Current Odd Harmonics Distortion is measured. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_OHD |  |
|  |  | COM | IviACPwrMeasureCurrentOHD |  |
| Current EHD | Current Even Harmonics Distortion is measured |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_EHD |  |
|  |  | COM | IviACPwrMeasureCurrentEHD |  |
| Current THD | Current Total (Odd+Even) Harmonic Distortion is measured |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_CURRENT_THD |  |
|  |  | COM | IviACPwrMeasureCurrentTHD |  |
| Voltage OHD | Voltage Odd Harmonics Distortion is measured. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_OHD |  |
|  |  | COM | IviACPwrMeasureVoltageOHD |  |
| Voltage EHD | Voltage Even Harmonics Distortion is measured |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_EHD |  |
|  |  | COM | IviACPwrMeasureVoltageEHD |  |
| Voltage THD | Voltage Total (Odd+Even) Harmonic Distortion is measured |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_VOLTAGE_THD |  |
|  |  | COM | IviACPwrMeasureVoltageTHD |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

| Completion Codes | Description |
| --- | --- |
| Measurement Not Initiated | You must use the Initiate Measurement function for that group prior to calling Fetch Measurement for that measurement type. |
| Measurement Not Supported | The AC Power source does not support the requested measurement type. |

- Compliance Notes

- If an IVI-C class driver defines additional values for the MeasurementType parameter, the actual values shall be greater than or equal to IVIACPWR_VAL_MEASURE_CLASS_EXT_BASE and less than IVIACPWR_VAL_MEASURE_SPECIFIC_EXT_BASE.

- IviACPwr specific drivers that implement the IviACPwrMeasurement Extension Group shall implement at least one of the following measurement types:

- Voltage RMS L-N

- Current RMS

### Fetch Measurement Array

Description

Returns the values measured by a previous call to the Initiate Measurement function. The Fetch Measurement Array function returns an array of measured values of the requested measurement type. Call this function multiple times to retrieve the available measurement types and output phases. The available measurement types are divided into measurement groups. Use the Initiate Measurement function to measure fresh values for that group.

COM Method Prototype

HRESULT OutputPhases.Item().FetchMeasurementArray([in] IviACPwrMeasurementTypeArrayEnum MeasurementType,
[in, out] SAFEARRAY(double) *Measurement);

C Prototype

ViStatus IviACPwr_FetchMeasurementArray (ViSession Vi,
                           ViConstString PhaseName,
                           ViInt32 MeasurementType,

ViInt32 MeasurementBufferSize,

ViReal64 Measurement[],

ViInt32 *MeasurementActualSize);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output phase to return the measurement | ViConstString |
| MeasurementType | The type of measurement to return. | ViInt32 |
| MeasurementBufferSize | The size of the user allocated IVI-C measurement array. | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Measurement | User allocated (IVI-C) or driver allocated (IVI-COM) array to store the measured values. | ViReal64 |
| MeasurementActualSize | The number of values stored in the measurement array. | ViInt32 |

Defined Values for MeasurementType Parameter – Harmonic Group

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Array Current Harmonic Phase | Current Harmonic Phase Angles are measured in degrees |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_HARMONIC_PHASE |  |
|  |  | COM | IviACPwrMeasureArrayCurrentHarmonicPhase |  |
| Array Current Harmonic Magnitude Absolute | Current Harmonic Magnitudes are measured as an absolute value in RMS Amps |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_HARMONIC_ABS |  |
|  |  | COM | IviACPwrMeasureArrayCurrentHarmonicAbs |  |
| Array Current Harmonic Magnitude Percent Fundamental | Current Harmonic Magnitudes are measured as a Percent of fundamental. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT HARMONIC_PCT |  |
|  |  | COM | IviACPwrMeasureArrayCurrentHarmonicPct |  |
| Array Voltage Harmonic Phase | Voltage Harmonic Phase Angles are measured in Degrees. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_PHASE |  |
|  |  | COM | IviACPwrMeasureArrayVoltageHarmonicPhase |  |
| Array Voltage Harmonic Magnitude Absolute | Voltage Harmonic Magnitudes are measured as an absolute value in RMS Volts. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_ABS |  |
|  |  | COM | IviACPwrMeasureArrayVoltageHarmonicAbs |  |
| Array Voltage Harmonic Magnitude Percent Fundamental | Voltage Harmonic Magnitudes are measured as a percent of the fundamental. |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_PCT |  |
|  |  | COM | IviACPwrMeasureArrayVoltageHarmonicPct |  |

Defined Values for MeasurementType Parameter – Waveform Group

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Array Current Cycle | Single cycle of the current waveform in Amps |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_CYCLE |  |
|  |  | COM | IviACPwrMeasureArrayCurrentCycle |  |
| Array Voltage Cycle | Single cycle of the Voltage Waveform in Volts |  |  |  |
|  |  | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_CYCLE |  |
|  |  | COM | IviACPwrMeasureArrayVoltageCycle |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return. For IVI-C, in cases where there are more values to return than the user specified array size, the function will fill the array and return the additional array size needed in the function result.

| Completion Codes | Description |
| --- | --- |
| Measurement Not Initiated | You must use the Initiate function for that group prior to calling Fetch for that measurement type. |
| Measurement Not Supported | The AC Power source does not support the requested measurement type. |

- Compliance Notes

- If an IVI-C class driver defines additional values for the MeasurementType parameter, the actual values shall be greater than or equal to IVIACPWR_VAL_MEASURE_CLASS_EXT_BASE and less than IVIACPWR_VAL_MEASURE_SPECIFIC_EXT_BASE.

- IviACPwr specific drivers that do not support any measurement types for the Fetch Measurement Array function should return the Measurement Not Supported status code.

## IviACPwrMeasurement Behavior Model

The IviACPwrMeasurement Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior model from the IviACPwrBase capability group is the ability to measure output signals.

# IviACPwrPhase Extension Group

## IviACPwrPhase Overview

The IviACPwrPhase extension group defines extensions for AC Power Sources which support multiple phase operation. The Phase angle is given in reference to Phase A (refer to section 2.3.1 for information on how output phases map to phase letters). The functions and attributes in the IviACPwrPhase extension group will allow for positive leading and negative lagging phase angles relative to Phase A. If an instrument only allows positive phase angles or defines positive angles as lagging relative to phase A, the driver must translate the user value to the instrument value.

Example:

Phase B set to negative 120 degrees lagging relative to Phase A translates to positive 240 degrees leading relative to Phase A.

- PHASOR DIAGRAM SHOWING LEADING vs. LAGGING

- PHASE ANGLE FOR B PHASE OUTPUT

The definition of phase rotation or phase sequence is that the correct phase sequence will be A-B-C in all cases. This sequence is shown in oscilloscope capture:

PHASE ROTATION OR PHASE SEQUENCE

AS DISPLAYED ON AN OSCILLOSCOPE

## IviACPwrPhase Attributes

The IviACPwrPhase extension group defines the following Attributes:

- Phase Angle

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 15.4, IviACPwrFrequencyRamp Behavior Model.

### Phase Angle

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Phase Angle |

COM Property Name

OutputPhases.Item().PhaseAngle

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_PHASE_ANGLE

Description

Specifies the Phase Angle of the output phase in relation to Phase A. The units are degrees.

## IviACPwrPhase Functions

The IviACPwrPhase extension defines the following function:

- Configure Phase Angle (IVI-C only)

This section describes the behavior and requirements of the function(s).

### Configure Phase Angle (IVI-C only)

Description

Configures the phase angle of a multi-phase power source. It specifies the phase angle of each phase of the power source as it relates to the Phase A. The functions and attributes will allow for positive leading and negative lagging phase angles relative to Phase A.

COM Method Prototype

N/A

(use the OutputPhases.Item().PhaseAngle property)

C Prototype

ViStatus IviACPwr_ConfigurePhaseAngle (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 PhaseAngle);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the phase angle. | ViConstString |
| PhaseAngle | Sets the phase angle of the specified output phase of a multiphase power source relative to the Phase A. The driver uses this value to set the Phase Angle attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrPhase Behavior Model

The IviACPwrPhase Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior from the IviACPwrBase capability group is the ability to configure IviACPwrPhase capabilities.

# IviACPwrExternalSync Extension Group

## IviACPwrExternalSync Overview

The IviACPwrExternalSync extension group defines extensions for AC Power Sources with the ability to use an external signal to synchronize the power source output frequency. These attributes and functions allow you to enable external synchronization, configure the phase offset and query the synchronization status of the power source with the external signal.

## IviACPwrExternalSync Attributes

The IviACPwrExternalSync extension group defines the following Attributes:

- External Synchronization Enabled

- External Synchronization Phase Offset

- External Synchronization Locked

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 15.4, IviACPwrFrequencyRamp Behavior Model.

### External Synchronization Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | N/A | None | Configure External Sync |

COM Property Name

OutputPhases.ExternalSync.Enabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_EXTERNAL_SYNC_ENABLED

Description

Specifies whether to enable or disable synchronization to an external signal.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Enable synchronization to external signal. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Disable synchronization to external signal. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Compliance Notes

- Instrument drivers shall support the value False.

### External Synchronization Phase Offset

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | N/A | None | Configure External Sync |

COM Property Name

OutputPhases.ExternalSync.PhaseOffset

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_EXTERNAL_SYNC_PHASE_OFFSET

Description

Specifies the phase angle offset (degrees) of the power sources output relative to the external synchronization signal.

A positive external sync phase offset value means the output of a single phase power source, or phase A of a multiphase power source, leads the external sync signal and a negative phase offset value means it lags the external sync signal. For multiphase power sources, the phase relationship between output phases is always exclusively determined by the IviACPwrPhase extension group.

### External Synchronization Locked

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | NA | None | Query External Synchronization Locked (IVI-C only) |

COM Property Name

OutputPhases.ExternalSync.Locked

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_EXTERNAL_SYNC_LOCKED

Description

Indicates if the AC power source is locked to the external synchronization signal.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | The AC power source is locked to the external synchronization signal. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | The AC power source is not locked to the external synchronization signal. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrExternalSync Functions

The IviACPwrExternalSync extension defines the following functions:

- Configure External Synchronization

- Query External Synchronization Locked (IVI-C only)

This section describes the behavior and requirements of the functions.

### Configure External Synchronization

Description

Specifies whether to enable or disable external synchronization and the phase offset relative to the external synchronization signal.

COM Method Prototype

HRESULT OutputPhases.ExternalSync.Configure (
 [in] BOOL Enabled

[in] double PhaseOffset);

C Prototype

ViStatus IviACPwr_ConfigureExternalSync (ViSession Vi,
                                         ViBoolean Enabled,

ViReal64 PhaseOffset);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| Enabled | Specifies if the power source should synchronize its frequency to an external signal. The driver uses this value to set the External Sync Enabled attribute. See the attribute description for more details. | ViBoolean |
| PhaseOffset | Sets the phase angle offset of the specified output phase relative to the external synchronization signal. The driver uses this value to set the External Sync Phase Offset attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Query External Synchronization Locked (IVI-C only)

Description

Returns if the power source is locked to the external synchronization signal.

COM Method Prototype

N/A

(use the OutputPhases.ExternalSync.Locked property)

C Prototype

ViStatus IviACPwr_QueryExternalSyncLocked (ViSession Vi,
                                         ViBoolean *Locked);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Locked | Returns if the power source output is locked with the external synchronization signal. | ViBoolean |

Defined Values for Locked Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | The power source is presently locked to the external synchronization signal. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | The power source is not presently locked to the external synchronization signal. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrExternalSync Behavior Model

The IviACPwrExternalSync Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior from the IviACPwrBase capability group is the ability to configure IviACPwrExternalSync capabilities.

# IviACPwrCurrentProtection Extension Group

## IviACPwrCurrentProtection Overview

The IviACPwrCurrentProtection extension group defines extensions for AC Power Sources which allow programmable current protection. A method for resetting the current protection and querying the current protection state of the power source is also provided.

## IviACPwrCurrentProtection Attributes

The IviACPwrCurrentProtection extension group defines the following attributes:

- Current Protection Threshold Value

- Current Protection Delay Value

- Current Protection Enabled

- Current Protection Tripped

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in Section 15.4, IviACPwrFrequencyRamp Behavior Model.

### Current Protection Threshold

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Current Protection |

COM Property Name

OutputPhases.Item().CurrentProtection.Threshold

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_PROTECTION_THRESHOLD

Description

Specifies the output current threshold that triggers current protection. The units are Amps RMS.

Compliance Notes

When current protection is enabled, changing this attribute may invalidate the Current Limit attribute in IviACPwrBase. In other words, some instruments may share the same value for Current Limit and Current Protection Threshold.

### Current Protection Delay

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Current Protection |

COM Property Name

OutputPhases.Item().CurrentProtection.Delay

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_PROTECTION_DELAY

Description

Specifies the time the output current must exceed the current protection threshold until current protection mode is triggered. The units are seconds.

### Current Protection Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | OutputPhase | None | Configure Current Protection |

COM Property Name

OutputPhases.Item().CurrentProtection.Enabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_PROTECTION_ENABLED

Description

When set to True, current protection mode is triggered when the output current exceeds the current protection threshold for the time specified by the current protection delay.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Current protection enabled. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Current protection disabled. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Compliance Notes

- Instrument drivers shall support the value False.

### Current Protection Tripped

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | OutputPhase | None | Query Current Protection Tripped (IVI-C only) |

COM Property Name

OutputPhases.Item().CurrentProtection.Tripped

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_PROTECTION_TRIPPED

Description

Indicates if current protection has tripped. Current protection trips when the output current exceeds the value of the current protection threshold attribute for the time specified by the current protection delay attribute. Use the Reset Current Protection function to reset the current protection state.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Current protection has tripped. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Current protection has not tripped. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrCurrentProtection Functions

The IviACPwrCurrentProtection extension group defines the following functions:

- Query Current Protection Tripped (IVI-C only)

- Reset Current Protection

- Configure Current Protection

### Query Current Protection Tripped (IVI-C only)

Description

Returns the current protection state of the AC power source.

Current protection trips when the output current exceeds the value of the current protection threshold attribute for the time specified by the current protection delay attribute.

When current protection trips, the power source enters the current protection state and disables the output. The user must call the Reset Current Protection function to reset the current protection state of the AC power source. Once current protection is reset, the user must then set the Output Enabled attribute to True for the AC power source to resume generating output voltage.

COM Method Prototype

N/A

(use the OutputPhases.Item().CurrentProtection.Tripped  property)

C Prototype

ViStatus IviACPwr_QueryCurrentProtectionTripped (ViSession Vi,
                                      ViConstString PhaseName,
                                      ViBoolean *Tripped);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output to return the current protection status. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Tripped | Returns True if current protection has tripped. | ViBoolean |

Defined Values for Tripped Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Current protection has tripped. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Current protection has not tripped. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Reset Current Protection

Description

Resets the AC power source current protection state after a current protection condition has occurred. The user must also set the Output Enabled attribute in IviACPwrBase to True to resume generating output power.

Use the Query Current Protection Tripped function to determine if current protection has tripped.

COM Method Prototype

HRESULT OutputPhases.Item().CurrentProtection.Reset();

C Prototype

ViStatus IviACPwr_ResetCurrentProtection (ViSession Vi,

ViConstString PhaseName);

Parameters

| Inputs |  | Description | Base Type |
| --- | --- | --- | --- |
| Vi |  | Instrument handle | ViSession |
| PhaseName | The name of the output on which to reset the current protection. |  | ViConstString |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Current Protection

Description

Configures current protection. When the Enabled parameter is True, the power source enters the current protection state (tripped) and disables the output when the output current exceeds the current threshold for the current delay time.

When the Enabled parameter is False, the threshold and delay do not affect the instrument’s behavior and current protection is disabled.

COM Method Prototype

HRESULT OutputPhases.Item().CurrentProtection.Configure (

[in] BOOL Enabled,
[in] DOUBLE Threshold,
[in] DOUBLE Delay);

C Prototype

ViStatus IviACPwr_ConfigureCurrentProtection (ViSession Vi,
                             ViConstString PhaseName,
                             ViBoolean Enabled,

ViReal64 Threshold,

ViReal64 Delay);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to set the current protection values. | ViConstString |
| Enabled | Enables/disables current protection. The driver uses this value to set the Current Protection Enabled attribute. See the attribute description for more details. | ViBoolean |
| Threshold | Specifies the current threshold that triggers over current protection. The driver uses this value to set the Current Protection Threshold attribute. See the attribute description for more details. | ViReal64 |
| Delay | Specifies the time the output current must exceed the over current protection threshold until over current protection is triggered. The driver uses this value to set the Current Protection Delay attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrCurrentProtection Behavior Model

The IviACPwrCurrentProtection Extension Group follows the behavior model of the IviACPwrBase capability group. The only addition to the behavior model is the driver must set the Output Enabled attribute in the IviACPwrBase capability group to False when a current protection condition occurs.

In order to resume generating output power after current protection trips, the user must call the Reset Current Protection function then set the Output Enabled attribute in IviACPwrBase to True.

It is the responsibility of the user to determine if the circuit is appropriate or safe for resuming generating output voltage after a current protection trips.

# IviACPwrVoltageProtection Extension Group

## IviACPwrVoltageProtection Overview

The IviACPwrVoltageProtection extension group defines extensions for AC Power Sources which allow programmable voltage protection. A method for resetting the protection and querying the protection state of the power source is also provided. In order for an instrument to be compatible with this extension group, it must support either or both Under Voltage or Over Voltage protection.

## IviACPwrVoltageProtection Attributes

The IviACPwrCurrentProtection extension group defines the following attributes:

- Under Voltage Limit

- Over Voltage Limit

- Over Voltage Enabled

- Under Voltage Enabled

- Voltage Protection Tripped

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in Section 15.4, IviACPwrFrequencyRamp Behavior Model.

### Under Voltage Limit

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Voltage Protection |

COM Property Name

OutputPhases.Item().VoltageProtection.UnderLimit

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_LIMIT

Description

Specifies the output under voltage protection limit value. The units are Volts RMS.

### Over Voltage Limit

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Voltage Protection |

COM Property Name

OutputPhases.Item().VoltageProtection.OverLimit

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_LIMIT

Description

Specifies the output over voltage protection limit value. The units are Volts RMS.

### Over Voltage Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | OutputPhase | None | Configure Voltage Protection |

COM Property Name

OutputPhases.Item().VoltageProtection.OverEnabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_ENABLED

Description

When set to True, voltage protection mode is triggered when the output voltage is above the Over voltage limit.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Over voltage protection enabled. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Over voltage protection disabled. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Compliance Notes

- Instrument drivers shall support the value False.

### Under Voltage Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | OutputPhase | None | Configure Voltage Protection |

COM Property Name

OutputPhases.Item().VoltageProtection.UnderEnabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_ENABLED

Description

When set to True, voltage protection mode is triggered when the output voltage is below the Under Voltage Limit.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Enable under voltage protection. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Disable under voltage protection. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Compliance Notes

- Instrument drivers shall support the value False.

### Voltage Protection Tripped

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | OutputPhase | None | Query Voltage Protection Tripped (IVI-C only) |

COM Property Name

OutputPhases.Item().VoltageProtection.Tripped

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_VOLTAGE_PROTECTION_TRIPPED

Description

Indicates if voltage protection has tripped. Use the Reset Voltage Protection function to reset the voltage protection state.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Voltage protection has tripped. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Voltage protection has not tripped. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrVoltageProtection Functions

The IviACPwrVoltageProtection extension group defines the following functions:

- Query Voltage Protection Tripped (IVI-C only)

- Reset Voltage Protection

- Configure Voltage Protection

### Query Voltage Protection Tripped (IVI-C only)

Description

Returns the voltage protection state of the AC power source.

Voltage protection trips when the output voltage is above the over-voltage protection limit attribute or below the under-voltage protection limit attributes and the corresponding voltage protection enabled attribute is set to True.

When voltage protection trips, the power source enters the voltage protection state and disables the output. The user must call the Reset Voltage Protection function to reset the voltage protection state of the AC power source. Once voltage protection is reset, the user must then set the Output Enabled attribute to True for the AC power source to resume generating output voltage.

COM Method Prototype

N/A

(use the OutputPhases.Item().VoltageProtection.Tripped  property)

C Prototype

ViStatus IviACPwr_QueryVoltageProtectionTripped (ViSession Vi,
                                      ViConstString PhaseName,
                                      ViBoolean *Tripped);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to return the voltage protection status. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Tripped | Returns True if voltage protection has tripped | ViBoolean |

Defined Values for Tripped Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Voltage protection has tripped. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Voltage protection has not tripped. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Reset Voltage Protection

Description

Resets the AC power source voltage protection state after a voltage protection condition has occurred. The user must also set the Output Enabled attribute in IviACPwrBase to True to resume generating output power.

Use the Query Voltage Protection Tripped function to determine if voltage protection has tripped.

COM Method Prototype

HRESULT OutputPhases.Item().VoltageProtection.Reset();

C Prototype

ViStatus IviACPwr_ResetVoltageProtection (ViSession Vi,

ViConstString PhaseName);

Parameters

| Inputs |  | Description | Base Type |
| --- | --- | --- | --- |
| Vi |  | Instrument handle | ViSession |
| PhaseName | The name of the output on which to reset the voltage protection. |  | ViConstString |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure Voltage Protection

Description

Configures voltage protection. When the corresponding enable parameter is True, the AC power source enters the voltage protection state (tripped) and disables the output when the output voltage is below the Under Limit or above the Over Limit.

When the OverEnabled and/or UnderEnabled parameter is False, the corresponding limit does not affect the instrument’s behavior and under and/or over voltage protection is disabled.

COM Method Prototype

HRESULT OutputPhases.Item().VoltageProtection.Configure(

[in} BOOL UnderEnabled,

[in} BOOL OverEnabled,
[in] DOUBLE UnderLimit,
[in] DOUBLE OverLimit);

C Prototype

ViStatus IviACPwr_ConfigureVoltageProtection (ViSession Vi,
                             ViConstString PhaseName,
                             ViBoolean UnderEnabled,

ViBoolean OverEnabled,

ViReal64 UnderLimit,

ViReal64 OverLimit);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to set the voltage protection values. | ViConstString |
| OverEnabled | The driver uses this value to set the Over Voltage Enabled attribute. See the attribute description for more details. | ViBoolean |
| UnderEnabled | The driver uses this value to set the Under Voltage Enabled attribute. See the attribute description for more details. | ViBoolean |
| UnderLimit | Specifies the under voltage protection level. The driver uses this value to set the Under Voltage Limit attribute. See the attribute description for more details. | ViReal64 |
| OverLimit | Specifies the over voltage protection level. The driver uses this value to set the Over Voltage Limit attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrVoltageProtection Behavior Model

The IviACPwrVoltageProtection Extension Group follows the behavior model of the IviACPwrBase capability group. The only addition to the behavior model is the driver must set the Output Enabled attribute in the IviACPwrBase capability group to False when a voltage protection condition occurs.

In order to resume generating output power after voltage protection trips, the user must call the Reset Voltage Protection function then set the Output Enabled attribute in IviACPwrBase to True.

It is the responsibility of the user to determine if the circuit is appropriate or safe for resuming generating output voltage after voltage protection trips.

# IviACPwrArbWaveform Extension Group

## IviACPwrArbWaveform Overview

The IviACPwrArbWaveform Extension Group defines an AC Power Source capable of producing user-defined arbitrary waveforms. The IviACPwrArbWaveform extension group includes functions for Writing, Clearing, and for returning information about arbitrary waveform capabilities and catalogs.

An Arbitrary Waveform is a user-defined series of sequential data points that describe an output waveform for a full cycle (360°). The frequency, phase angle, RMS amplitude and DC offset of the arbitrary waveform at the power source output is defined by the capability group and attribute shown in the following table.

| Waveform Property | Capability Group | Attribute |
| --- | --- | --- |
| Frequency | IviAcPwrBase | Frequency |
| Phase Angle | IviAcPwrPhase | Phase Angle |
| RMS Amplitude | IviAcPwrBase | Voltage Level |
| DC Offset | IviAcPwrDCGeneration | DC Offset |

The amplitude of the waveform data array passed to the Write Waveform function is rescaled to the native waveform amplitude range supported by the AC power source. The number of data points in the waveform data array is automatically resampled to the native waveform data points supported by the AC power source.

The IviACPwrArbWaveform extension group includes read only attributes that define the maximum number of arbitrary waveforms the power source can store, the number of presently stored and available waveforms and the optimal number of data points of an arbitrary waveform. The functions include writing and clearing arbitrary waveforms and querying the arbitrary waveform catalog .

In addition to supporting user-defined arbitrary waveforms, the AC power source contains a base Sine waveform that is always present. Instrument vendors may also define additional fixed waveforms that cannot be cleared.

The driver must not allow user defined or fixed vendor defined waveforms to contain a DC Offset. This is because the DC offset of the power source output voltage must be exclusively controlled by the IviAcPwrDCGeneration extension capability group. The DC Offset can be determining by adding together all data points of the waveform. The absolute value of the sum must be less than 0.001% of the peak to peak amplitude.

Waveforms are referenced using a string name. This name can be used to clear a waveform from the power source memory (allowing other waveforms to be written) or specify which waveform the power source should use when generating the voltage on an output phase (Refer to Waveform attribute in the IviACPwrBase capability group).

The waveform data values may affect the maximum output voltage the power source can generate using that waveform. Refer to section A.6 Waveform RMS and Peak Voltage for more information.

## IviACPwrArbWaveform Attributes

The IviACPwrArbWaveform capability group defines the following attributes:

- Num Waveforms Max

- Num Optimal Data Points

- Fixed Waveform Count

- User Waveform Count

- Available Waveform Count

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in Section 15.4, IviACPwrFrequencyRamp Behavior Model.

### Num Waveforms Max

| Data Type | Access | Applies To | Coercion | High Level Function(s) |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

ArbWaveform.NumWaveformsMax

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_NUM_WAVEFORMS_MAX

Description

Returns the maximum number of arbitrary waveforms the power source can store including user defined waveforms and fixed vendor defined waveforms.

### Num Optimal Data Points

| Data Type | Access | Applies To | Coercion | High Level Function(s) |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

ArbWaveform.NumOptimalDataPoints

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_NUM_OPTIMAL_DATA_POINTS

Description

Returns the optimal (native) number of data points the AC power source uses for arbitrary waveforms.

The driver will automatically resample the waveform data array passed to the Write Arbitrary Waveform function to fit the optimal number of data points supported by the AC power source. To minimize the effects of resampling and to maximize the waveform resolution, allocate user defined waveform data using the optimal number of data points of the AC power source.

### Fixed Waveform Count

| Data Type | Access | Applies To | Coercion | High Level Function(s) |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

ArbWaveform.FixedWaveformCount

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_FIXED_WAVEFORM_COUNT

Description

Returns the number of fixed vendor defined arbitrary waveforms stored in the power source.

### User Waveform Count

| Data Type | Access | Applies To | Coercion | High Level Function(s) |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

ArbWaveform.UserWaveformCount

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_USER_WAVEFORM_COUNT

Description

Returns the number of user defined arbitrary waveforms stored in the power source.

### Available Waveform Count

| Data Type | Access | Applies To | Coercion | High Level Function(s) |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | N/A | None | None |

COM Property Name

ArbWaveform.AvailableWaveformCount

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_AVAILABLE_WAVEFORM_COUNT

Description

Returns the number of additional arbitrary waveforms the user can store in the power source. Use the Clear Arbitrary Waveform function to increase the number user defined arbitrary waveforms the power source can store.

The number of available waveforms can also be calculated from the following attributes:

Available Waveforms = Num Waveforms Max – Fixed Waveform Count – User Waveform Count

## IviACPwrArbWaveform Functions

The IviACPwrArbWaveform extension defines the following functions:

- Clear Arbitrary Waveform

- Write Arbitrary Waveform

- Query Arbitrary Waveform Catalog

This section describes the behavior and requirements of each function.

### Clear Arbitrary Waveform

# Description

Deletes individual user-defined waveforms from the power source's memory.

If the waveform cannot be cleared because it is currently being used to generate output voltage, this function returns the Waveform In Use error. If the waveform name is “Sine” or a fixed vendor defined waveform, this function returns the Waveform Reserved error.

COM Method Prototype

HRESULT ArbWaveform.Clear ([in] BSTR WaveformName );

# C Prototype

ViStatus IviACPwr_ClearArbWaveform (ViSession Vi,
                                   ViConstString WaveformName);

# Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| WaveformName | Specifies the name of the user defined waveform to remove from the power source memory. If the waveform name parameter is VI_NULL, then all user defined waveforms are cleared. | ViConstString |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return. The table below specifies additional class-defined status codes for this function.

| Completion Codes | Description |
| --- | --- |
| Waveform In Use | The power source is currently configured to produce the specified waveform. |
| Waveform Reserved | The waveform name references a vendor defined fixed waveform. |
| Waveform Not Found | The waveform name is not currently stored in the AC Power Source. |

# Compliance Notes

- Specific drivers shall implement VI_NULL for the WaveformName parameter to clear all user defined waveforms.

### Write Arbitrary Waveform

# Description

Writes an individual user-defined waveform to the AC power source's memory. If the power source cannot store any more arbitrary waveforms, this function returns the No Waveforms Available error.

If the waveform name already exists, then the function returns the Duplicate Waveform Name error.

If the absolute value of the sum of all waveform data points exceeds 0.001% of the peak to peak amplitude, the function returns the Waveform DC Offset error.

COM Method Prototype

HRESULT ArbWaveform.Write ([in] BSTR WaveformName,

[in] SAFEARRAY(double) *WaveformData);

# C Prototype

ViStatus IviACPwr_WriteArbWaveform (ViSession Vi,
                                   ViConstString WaveformName,

ViInt32 WaveformDataBufferSize,

ViReal64 WaveformData[] );

# Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| WaveformName | Specifies the waveform name that identifies the arbitrary waveform to write to the AC power source. | ViConstString |
| WaveformDataBufferSize | The size of the IVI-C user allocated waveform data buffer | ViInt32 |
| WaveformData | Specifies the user allocated array of the user-defined waveform data to be stored in the AC power source. | ViReal64[] |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return. The table below specifies additional class-defined status codes for this function.

| Completion Codes | Description |
| --- | --- |
| Duplicate Waveform Name | The specified waveform name is already stored in the AC power source. |
| No Waveforms Available | The waveform memory in the AC power source is full. |
| Invalid Waveform Name | The specified waveform name is invalid. |
| Waveform DC Offset | The waveform contains a DC offset that exceeds 0.001% of the peak to peak amplitude, |

# Compliance Notes

- Waveform names cannot contain a comma character.

2. 	The driver can use the following code to detect DC offset in the user supplied waveform array.

ViReal64 sum=0, min, max, pkpk, y;

ViInt32 i;

// Calculate sum, min and max

min = WaveformData[0];

max=min;

for (i=0;i<WaveformDataBufferSize;i++) {

y = WaveformData[i];

if (y> max) max = y;

if (y< min) min = y ;

sum += y;

}

// Calculate 0.001% of peak to peak amplitude

pkpk = 0.001 * fabs(max - min) / 100;

// Return error if DC offset exceeds limit

if (fabs(sum) > pkpk) return IVIACPWR_ERROR_WAVEFORM_DC_OFFSET;

### Query Arbitrary Waveform Catalog

# Description

Returns a comma delimited string of user defined and/or fixed vendor defined waveform names currently stored in the AC power source.

The number of additional user defined waveforms the power source can store can be determined by the Available Waveform Count attribute.

COM Method Prototype

HRESULT ArbWaveform.QueryCatalog ([in] long CatalogType,

[in, out] BSTR *Catalog);

# C Prototype

ViStatus IviACPwr_QueryArbWaveformCatalog (ViSession Vi,

ViInt32 CatalogType,

ViInt32 CatalogBufferSize,
ViChar Catalog[]);

# Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| CatalogType | Specifies if the returned catalog string contains fixed vendor defined and/or user defined waveform names. | ViInt32 |
| CatalogBufferSize | The size of the IVI-C user allocated buffer. | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Catalog | A user allocated buffer to store a comma delimited string of waveform names currently stored in the AC power source. | ViChar[] |

Defined Values for CatalogType Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| Fixed | Request catalog of fixed vendor defined waveform names. |  |  |  |
|  |  | C | IVIACPWR_VAL_WAVEFORM_CATALOG_FIXED |  |
|  |  | COM | IviACPwrWaveformCatalogFixed |  |
| User | Request catalog of user defined waveform names. |  |  |  |
|  |  | C | IVIACPWR_VAL_WAVEFORM_CATALOG_USER |  |
|  |  | COM | IviACPwrWaveformCatalogUser |  |
| All | Request catalog of all waveform names. |  |  |  |
|  |  | C | IVIACPWR_VAL_WAVEFORM_CATALOG_ALL |  |
|  |  | COM | IviACPwrWaveformCatalogAll |  |

# Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrArbWaveform Behavior Model

The IviACPwrArbWaveform Extension Group follows the behavior model of the IviACPwrBase capability group but modifies the Output Enabled attribute as described below.

Certain AC power sources may require that output power be disabled in order to write a waveform. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

# IviACPwrImpedance Extension Group

## IviACPwrImpedance Overview

The IviACPwrImpedance extension group defines extensions for AC Power Sources with the ability to configure the output impedance. Typically the output impedance is set using firmware emulation of impedance or analog control loop.

## IviACPwrImpedance Attributes

The IviACPwrImpedance extension defines the following attributes:

- Output Impedance Enabled

- Output Impedance Resistive

- Output Impedance Inductive

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 16, IviACPwr Attribute ID Definitions.

### Output Impedance Enabled

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | R/W | OutputPhase | None | Configure Output Impedance |

COM Property Name

OutputPhases.Item().Impedance.Enabled

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OUTPUT_IMPEDANCE_ENABLED

Description

When set to True, this attribute allows the output impedance resistive and output impedance inductive attribute values to affect the power source output.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | The output impedance values affect the AC source output |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | The output impedance feature is disabled |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

Compliance Notes

- Instrument drivers shall support the value False.

### Output Impedance Resistive

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Output Impedance |

COM Property Name

OutputPhases.Item().Impedance.Resistive

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OUTPUT_IMPEDANCE_RESISTIVE

Description

Specifies the resistive output impedance value in ohms. This value affects the power source output when the Output Impedance Enabled attribute is set to True.

Compliance Notes

- Instrument drivers shall support the value 0 and may coerce this to the lowest values supported by the instrument. Use the Query Output Impedance Capabilities function to obtain a range of allowable values.

### Output Impedance Inductive

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | R/W | OutputPhase | None | Configure Output Impedance |

COM Property Name

OutputPhases.Item().Impedance.Inductive

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_OUTPUT_IMPEDANCE_INDUCTIVE

Description

Specifies the inductive output impedance value in henries. This value affects the power source output when the Output Impedance Enabled attribute is set to True.

Compliance Notes

Instrument drivers shall support the value 0 and may coerce this to the lowest values supported by the instrument. Use the Query Output Impedance Capabilities function to obtain a range of allowable values.

## IviACPwrImpedance Functions

The IviACPwrImpedance extension defines the following functions:

- Configure Output Impedance

- Query Output Impedance Capabilities

Configure Output Impedance configures the output impedance of the AC power source by setting the Output Impedance Enabled, Output Impedance Resistive and Output Impedance Inductive attributes.

Query Output Impedance Capabilities returns the minimum and maximum allowable resistive and inductive impedance the AC power source can be programmed for the present configuration.

### Configure Output Impedance

Description

Configures the output impedance of the AC power source. The Enabled parameter must be set to True in order for the Resistive or Inductive values to affect the power source output.

COM Method Prototype

HRESULT OutputPhases.Item().Impedance.Configure (

[in} BOOL Enabled,
[in] DOUBLE ResistiveValue,
[in] DOUBLE InductiveValue );

C Prototype

ViStatus IviACPwr_ConfigureOutputImpedance (ViSession Vi,
                           ViConstString PhaseName,
                           ViBoolean Enabled,
                           ViReal64 ResistiveValue,
                           ViReal64 InductiveValue );

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the output impedance. | ViConstString |
| Enabled | Enables/disables the output impedance capability of the AC power source. The driver uses this value to set the Output Impedance Enabled attribute. See the attribute description for more details. | ViBoolean |
| ResistiveValue | Sets the resistive part of the output impedance of the AC power source in Ohms. The driver uses this value to set the Output Impedance Resistive attribute. See the attribute description for more details. | ViReal64 |
| InductiveValue | Sets the inductive part of the output impedance of the AC power source in Henries. The driver uses this value to set the Output Impedance Inductive attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Query Output Impedance Capabilities

Description

Queries the output impedance capabilities of the AC power source based on the present configuration.

COM Method Prototype

HRESULT OutputPhases.Item().Impedance.QueryCapabilities (

[in, out} DOUBLE *ResistiveMin,
[in, out] DOUBLE *ResistiveMax,
[in, out] DOUBLE *InductiveMin,

[in, out] DOUBLE *InductiveMax);

C Prototype

ViStatus IviACPwr_QueryOutputImpedanceCapabilities (ViSession Vi,
                           ViConstString PhaseName,
                           ViReal64 *ResistiveMin,
                           ViReal64 *ResistiveMax,

ViReal64 *InductiveMin,
       ViReal64 *InductiveMax);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to return the output impedance capabilities. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| ResistiveMin | Returns the minimum resistive impedance value the power source can be programmed. | ViReal64 |
| ResistiveMax | Returns the maximum resistive impedance value the power source can be programmed. | ViReal64 |
| InductiveMin | Returns the minimum inductive impedance value the power source can be programmed. | ViReal64 |
| InductiveMax | Returns the maximum inductive impedance value the power source can be programmed. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrImpedance Behavior Model

The IviACPwrImpedance Extension Group follows the behavior model of the IviACPwrBase capability group but modifies the Output Enabled attribute as described below.

Certain AC power sources may require that the output power be disabled in order to change the Output Impedance Enabled, Output Impedance Resistive or Output Impedance Inductive values. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

# IviACPwrDCGeneration Extension Group

## IviACPwrDCGeneration Overview

The IviACPwrDCGeneration extension group defines extensions for AC Power Sources capable of producing a DC output.

There are three modes of operation; AC Only, DC Only and AC Plus DC. The mode of operation determines when the Voltage Level attribute in the IviACPwrBase capability group and the DC Voltage Level attribute in the IviACPwrDCGeneration are used as shown in the following table.

| DC Mode | Voltage Level (IviACPwrBase) | DC Voltage Level (IviACPwrDCGeneration) |
| --- | --- | --- |
| AC Only | AC RMS voltage | Not Used |
| DC Only | Not Used | DC Voltage |
| AC Plus DC | AC RMS voltage | DC Offset |

The number of DC ranges the AC power supply supports are specified by the Number DC Ranges attribute. Each DC range supports a minimum and maximum DC voltage. Use the Query DC Capabilities function to determine the minimum and maximum DC voltage supported by each range.

Ranges may support unipolar or bipolar voltages. Ranges that support bipolar voltages may not be symmetrical. The DC Voltage Level attribute must not exceed the minimum and maximum DC voltages supported by the present range.

## IviACPwrDCGeneration Attributes

The IviACPwrDCGeneration extension group defines the following attributes:

- DC Mode

- DC Voltage Level

- DC Range Min

- DC Range Max

- Number of DC Ranges

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 15.4, IviACPwrFrequencyRamp Behavior Model.

### DC Mode

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | R/W | OutputPhase | None | Configure DC |

COM Property Name

OutputPhases.Item().DCGeneration.Mode

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_DC_MODE

Description

Configures AC Only, DC Only or a combination of AC Plus DC output voltage generation. When using AC Only or AC Plus DC generation mode, the Voltage Level attribute in IviACPwrBase sets the RMS value of the AC voltage.

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| AC Only | AC Only voltage generation |  |  |  |
|  |  | C | IVIACPWR_VAL_MODE_AC_ONLY |  |
|  |  | COM | IviACPwrDCGenerationModeACOnly |  |
| DC Only | DC Only voltage generation |  |  |  |
|  |  | C | IVIACPWR_VAL_MODE_DC_ONLY |  |
|  |  | COM | IviACPwrDCGenerationModeDCOnly |  |
| AC Plus DC | AC Plus DC voltage generation |  |  |  |
|  |  | C | IVIACPWR_VAL_MODE_AC_DC |  |
|  |  | COM | IviACPwrDCGenerationModeACPlusDC |  |

### DC Voltage Level

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| Vireal64 | R/W | OutputPhase | None | Configure DC |

COM Property Name

OutputPhases.Item().DCGeneration.DCVoltageLevel

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_DC_VOLTAGE_LEVEL

Description

The DC Voltage when the DC Mode attribute is set to DC Only or the DC Offset when the DC Mode attribute is set to AC Plus DC. Refer to the table in section 12.1 for more information on how this attribute affects the power source output.

### DC Range Min

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | OutputPhase | None | Configure DC Range |

COM Property Name

OutputPhases.Item().DCGeneration.RangeMin

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_DC_RANGE_MIN

Description

Returns the minimum DC voltage supported by the power source for the present DC voltage range. Use the Configure DC Range function to set the DC voltage range.

### DC Range Max

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViReal64 | RO | OutputPhase | None | Configure DC Range |

COM Property Name

OutputPhases.Item().DCGeneration.RangeMax

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_DC_RANGE_MAX

Description

Returns the maximum DC voltage supported by the power source for the present DC voltage range. Use the Configure DC Range function to set the DC voltage range.

### Number of DC Ranges

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViInt32 | RO | OutputPhase | None | None |

COM Property Name

OutputPhases.Item().DCGeneration.NumRanges

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_DC_NUM_RANGES

Description

The number of DC ranges supported by the power source.

Defined Values

N/A

Compliance Notes

All vendors that support the DC Generation extension group must support at least 1 DC voltage range

## IviACPwrDCGeneration Functions

The IviACPwrDCGeneration extension group defines the following functions:

- Configure DC

- Configure DC Range

- Query DC Capabilities

This section describes the behavior and requirements of the functions.

### Configure DC

Description

Configures the DC output generation.

COM Method Prototype

HRESULT OutputPhases.Item().DCGeneration.Configure(
[in] long Mode,

[in] double DCVoltageLevel);

C Prototype

ViStatus IviACPwr_ConfigureDC (ViSession Vi,

ViConstString PhaseName,
                                 ViInt32 Mode,

ViReal64 DCVoltageLevel);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure DC generation. | ViConstString |
| Mode | Configures AC Only, DC Only or AC Plus DC output generation. The driver uses this value to set the DC Mode attribute. See the attribute description for more details. | ViInt32 |
| DCVoltageLevel | Specifies the power source DC output voltage when using the DC Only mode or the DC offset voltage when using the AC Plus DC mode. The driver uses this value to set the DC Voltage Level attribute. See the attribute description for more details. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Configure DC Range

Description

Configures the DC voltage range of the AC power source. The driver sets the power source to the lowest range that supports the minimum and maximum voltages specified by the Minimum and Maximum parameters. It then updates the DC Range Min and DC Range Max attributes.

COM Method Prototype

HRESULT OutputPhases.Item().DCGeneration.ConfigureRange(

[in] double Minimum

[in] double Maximum);

C Prototype

ViStatus IviACPwr_ConfigureDCRange (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 Minimum,

ViReal64 Maximum);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the DC Generation range. | ViConstString |
| Minimum | Specifies the minimum DC voltage. The driver coerces this value to the minimum voltage of the lowest range that supports this value. | ViReal64 |
| Maximum | Specifies the maximum DC voltage. The driver coerces this value to the maximum voltage of the lowest range that supports this value. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

### Query DC Capabilities

Description

Returns the minimum and maximum DC voltages the power source can generate for the specified range. Typically, the user calls the ConfigureDCRange function with the min and max voltages they want to use and it automatically selects the best range. The only time the Query DC Capabilities function would be useful is to eliminate unnecessary range changes by allowing the user to know in advance the range boundaries.

The DC Range Min and DC Range Max attributes return the min and max voltage of the present range.

COM Method Prototype

HRESULT OutputPhases.Item().DCGeneration.QueryCapabilities(

[in] long Range

[in, out] double *Minimum

[in, out] double *Maximum);

C Prototype

ViStatus IviACPwr_QueryDCCapabilities(ViSession Vi,

ViConstString PhaseName,
                                 ViInt32 Range,

ViReal64 *Minimum,

ViReal64 *Maximum);

Parameters

| Inputs |  | Description | Base Type |
| --- | --- | --- | --- |
| Vi |  | Instrument handle | ViSession |
| PhaseName | The name of the output on which to query the DC generation capabilities. |  | ViConstString |
| Range |  | The DC range to query. This number must be from 1 to the values returned by the IVIACPWR_ATTR_DC_ NUM_RANGES attribute | ViInt32 |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Minimum | Returns the minimum DC voltage the power source can generate | ViReal64 |
| Maximum | Returns the maximum DC voltage the power source can generate | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrDCGeneration Behavior Model

The IviACPwrDCGeneration Extension Group follows the behavior model of the IviACPwrBase capability group but modifies the Output Enabled attribute as described below.

Certain AC power sources may require that the output power be disabled in order to change the DC Mode, DC Offset or DC Range. Refer to Appendix A.5: Operations that Require the Output be Disabled for more information.

# IviACPwrVoltageRamp Extension Group

## IviACPwrVoltageRamp Overview

The IviACPwrVoltageRamp extension group defines extensions for AC Power Sources with the capability to configure a linear voltage ramp with a starting and ending voltage in volts RMS and duration in seconds.

The Start Voltage value takes effect immediately then the voltage ramps linearly to the End Voltage value for the duration specified.

This figure above shows 2 voltage ramps. The Start Voltage of the first ramp was set to the same value as the Voltage Level attribute in IviACPwr base. The Start Voltage of the second ramp was set to a value higher than the present Voltage Level attribute, causing an immediate change in output voltage before linearly ramping to the End Voltage value.

## IviACPwrVoltageRamp Attributes

The IviACPwrVoltageRamp extension group defines the following Attributes:

- Voltage Ramp Busy

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 16, IviACPwrFrequencyRamp Behavior Model.

### Voltage Ramp Busy

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | OutputPhase | None | Query Voltage Ramp Busy (IVI-C only) |

COM Property Name

OutputPhases.Item().VoltageRampBusy

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_VOLTAGE_RAMP_BUSY

Description

Indicates if a voltage ramp is in progress (busy).

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Voltage ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Voltage ramp not in progress. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrVoltageRamp Functions

The IviACPwrVoltageRamp extension defines the following functions:

- Ramp Voltage

- Query Voltage Ramp Busy (IVI-C only)

- Abort Voltage Ramp

This section describes the behavior and requirements of these functions.

### Ramp Voltage

Description

Starts a linear voltage ramp from a starting voltage to an ending voltage in volts RMS for a duration in seconds. Call the Query Voltage Ramp Busy function to determine when the ramp completes or call the Abort Voltage Ramp function to stop the voltage ramp.

The starting and ending ramp voltages must not exceed the values of the present voltage range. Refer to the Voltage Range attribute in the IviACPwrBase capability group.

COM Method Prototype

HRESULT OutputPhases.Item().RampVoltage(
 	[in] double StartVoltage);

[in] double EndVoltage);

[in] double Duration);

C Prototype

ViStatus IviACPwr_RampVoltage (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 StartVoltage);

ViReal64 EndVoltage);

ViReal64 Duration);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the voltage ramp. | ViConstString |
| StartVoltage | Specifies the power source’s starting output voltage in volts rms. | ViReal64 |
| EndVoltage | Specifies the power source’s ending output voltage in volts rms. | ViReal64 |
| Duration | Duration of the voltage ramp in seconds. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

Instruments that must complete the voltage ramp before returning are allowed to be compliant with the IviACPwrVoltageRamp extension group. For these instruments, the Ramp Voltage function waits for the ramp to complete before returning and the Query Voltage Ramp Busy function always returns False.

### Query Voltage Ramp Busy (IVI-C only)

Description

Returns if a voltage ramp is in progress. Call this function after calling the Ramp Voltage function to determine when the ramp completes.

COM Method Prototype

N/A

(use the OutputPhases.Item().VoltageRampBusy  property)

C Prototype

ViStatus IviACPwr_QueryVoltageRampBusy (ViSession Vi,
                                      ViConstString PhaseName,
                                      ViBoolean *Busy);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to query if the voltage ramp is busy. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Busy | Returns True if a voltage ramp is in progress. Returns False when a Voltage Ramp completes or is not in progress. | ViBoolean |

Defined Values for Busy Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Voltage ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Voltage ramp not in progress |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

### Abort Voltage Ramp

Description

Aborts a  voltage ramp. If a voltage ramp is not in progress this function does nothing.

COM Method Prototype

HRESULT OutputPhases.Item().AbortVoltageRamp();

C Prototype

ViStatus IviACPwr_AbortVoltageRamp (ViSession Vi,
                                         ViConstString PhaseName);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to abort the voltage ramp. | ViConstString |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrVoltageRamp Behavior Model

The IviACPwrVoltageRamp Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior model from the IviACPwrBase capability group is the ability to ramp the output voltage.

When the  Ramp Voltage function returns immediately after the ramp has started, the user should only call the Query Voltage Ramp Busy or Abort Voltage Ramp functions or read the Voltage Ramp Busy attribute while the ramp is in progress. Calling any other function or reading or writing any other attribute may abort the ramp or cause other vendor specific instrument behavior. After the voltage ramp completes or is aborted, the driver must invalidate the Voltage Level attribute in the IviACPwrBase capability group.

The actual voltage that appears at the power source output during a voltage ramp assumes the power source is operating in constant voltage mode. In other words, the output current during the voltage ramp is below the Current Limit attribute value in the IviACPwrBase capability group. If the power source operates in constant current mode during any portion of the ramp, the actual output voltage is reduced to maintain the current limit value but the actual ramp duration is not affected.

# IviACPwrCurrentRamp Extension Group

## IviACPwrCurrentRamp Overview

The IviACPwrCurrentRamp extension group defines extensions for AC Power Sources with the capability to configure a linear current ramp with a starting and ending current in amps and duration in seconds.

The Start Current value takes effect immediately then the current ramps linearly to the End Current value for the duration specified.

This figure above shows 2 current ramps. The Start Current of the first ramp was set to the same value as the Current Limit attribute in IviACPwr base. The Start Current of the second ramp was set to a value higher than the present Current Limit attribute, causing an immediate change in current limit before linearly ramping to the End Current value.

## IviACPwrCurentRamp Attributes

The IviACPwrCurrentRamp extension group defines the following Attributes:

- Current Ramp Busy

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 16, IviACPwrFrequencyRamp Behavior Model.

### Current Ramp Busy

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | OutputPhase | None | Query Current Ramp Busy (IVI-C only) |

COM Property Name

OutputPhases.Item().CurrentRampBusy

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_CURRENT_RAMP_BUSY

Description

Indicates if a current ramp is in progress (busy).

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Current ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Current ramp not in progress. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrCurrentRamp Functions

The IviACPwrCurrentRamp extension defines the following functions:

- Ramp Current

- Query Current Ramp Busy (IVI-C only)

- Abort Current Ramp

This section describes the behavior and requirements of these functions.

### Ramp Current

Description

Starts a linear current ramp from a starting and ending current in amps for a duration in seconds. Call the Query Current Ramp Busy function to determine when the ramp completes or call the Abort Current Ramp function to stop the current ramp.

COM Method Prototype

HRESULT OutputPhases.Item().RampCurrent(
 	[in] double StartCurrent);

[in] double EndCurrent);

[in] double Duration);

C Prototype

ViStatus IviACPwr_RampCurrent (ViSession Vi,
                                         ViConstString PhaseName,
                                         ViReal64 StartCurrent);

ViReal64 EndCurrent);

ViReal64 Duration);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to configure the current ramp. | ViConstString |
| StartCurrent | Specifies the power source’s starting output current in Amps. | ViReal64 |
| EndCurrent | Specifies the power source’s ending output current in Amps. | ViReal64 |
| Duration | Duration of the current ramp in seconds | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

Instruments that must complete the current ramp before returning are allowed to be compliant with the IviACPwrCurrentRamp extension group. For these instruments, the Ramp Current function waits for the ramp to complete before returning and the Query Current Ramp Busy function always returns False.

### Query Current Ramp Busy (IVI-C only)

Description

Returns if a current ramp is in progress. Call this function after calling the Ramp Current function to determine when the ramp completes.

COM Method Prototype

N/A

(use the OutputPhases.Item().CurrentRampBusy  property)

C Prototype

ViStatus IviACPwr_QueryCurrentRampBusy (ViSession Vi,
                                      ViConstString PhaseName,
                                      ViBoolean *Busy);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to query if the current ramp is busy. | ViConstString |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Busy | Returns True if a current ramp is in progress. Returns False when a Current Ramp completes or is not in progress. | ViBoolean |

Defined Values for Busy Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Current ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Current ramp not in progress |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

### Abort Current Ramp

Description

Aborts a  current ramp. If a current ramp is not in progress this function does nothing.

COM Method Prototype

HRESULT OutputPhases.Item().AbortCurrentRamp();

C Prototype

ViStatus IviACPwr_AbortCurrentRamp (ViSession Vi,
                                         ViConstString PhaseName);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| PhaseName | The name of the output on which to abort the current ramp. | ViConstString |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrCurrentRamp Behavior Model

The IviACPwrCurrentRamp Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior model from the IviACPwrBase capability group is the ability to ramp the output current.

When the Ramp Current function returns immediately after the ramp has started, the user should only call the Query Current Ramp Busy or Abort Current Ramp functions or read the Current Ramp Busy attribute while the ramp is in progress. Calling any other function or reading or writing any other attribute may abort the ramp or cause other vendor specific instrument behavior. After the current ramp completes or is aborted, the driver must invalidate the Current Limit attribute in the IviACPwr capability group

The actual current that appears at the power source output during a current ramp assumes the power source is operating in constant current mode. In other words, the output voltage during the current ramp is below the Voltage Level attribute value in the IviACPwrBase capability group. If the power source operates in constant voltage mode during any portion of the ramp, the output current is reduced to maintain the voltage level value but the actual ramp duration is not affected.

# IviACPwrFrequencyRamp Extension Group

## IviACPwrFrequencyRamp Overview

The IviACPwrFrequencyRamp extension group defines extensions for AC Power Sources with the capability to configure a linear Frequency ramp starting and ending Frequency in Hertz and duration in seconds.

The Start Frequency value takes effect immediately then the frequency ramps linearly to the End Frequency value for the duration specified.

This figure above shows 2 frequency ramps. The Start Frequency of the first ramp was set to the same value as the Frequency attribute in IviACPwr base. The Start Frequency of the second ramp was set to a value higher than the present Frequency attribute, causing an immediate change in output Frequency before linearly ramping to the End Frequency value.

## IviACPwrFrequencyRamp Attributes

The IviACPwrFrequencyRamp extension group defines the following Attributes:

- Frequency Ramp Busy

This section describes the behavior and requirements of each attribute. The actual value for each attribute ID is defined in section 16, IviACPwrFrequencyRamp Behavior Model.

### Frequency Ramp Busy

| Data Type | Access | Applies to | Coercion | High Level Functions |
| --- | --- | --- | --- | --- |
| ViBoolean | RO | N/A | None | Query Frequency Ramp Busy (IVI-C only) |

COM Property Name

OutputPhases.FrequencyRampBusy

COM Enumeration Name

N/A

C Constant Name

IVIACPWR_ATTR_FREQUENCY_RAMP_BUSY

Description

Indicates if a frequency ramp is in progress (busy).

Defined Values

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Frequency ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Frequency ramp not in progress. |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

## IviACPwrFrequencyRamp Functions

The IviACPwrFrequencyRamp extension defines the following functions:

- Ramp Frequency

- Query Frequency Ramp Busy (IVI-C only)

- Abort Frequency Ramp

This section describes the behavior and requirements of the functions.

### Ramp Frequency

Description

Starts a the linear Frequency ramp then waits for it to complete. It specifies the starting and ending frequency in Hertz and duration in seconds. Call the Query Frequency Ramp Busy function to determine when the ramp completes or call the Abort Frequency Ramp function to stop the frequency ramp.

The starting and ending ramp frequencies must not exceed the values of the present frequency range. Refer to the Frequency Range attribute in the IviACPwrBase capability group.

COM Method Prototype

HRESULT OutputPhases.RampFrequency (
 	[in] double StartFrequency);

[in] double EndFrequency);

[in] double Duration);

C Prototype

ViStatus IviACPwr_RampFrequency (ViSession Vi,
                                         ViReal64 StartFrequency);

ViReal64 EndFrequency);

ViReal64 Duration);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |
| StartFrequency | Specifies the power source’s starting output frequency in Hertz. | ViReal64 |
| EndFrequency | Specifies the power source’s ending output frequency in Hertz. | ViReal64 |
| Duration | Duration of the Frequency ramp in seconds. | ViReal64 |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

Compliance Notes

Instruments that must complete the frequency ramp before returning are allowed to be compliant with the IviACPwrFrequencyRamp extension group. For these instruments, the Ramp Frequency function waits for the ramp to complete before returning and the Query Frequency Ramp Busy function always returns False.

### Query Frequency Ramp Busy (IVI-C only)

Description

Returns if a frequency ramp is in progress. Call this function after calling the Ramp Frequency function to determine when the ramp completes.

COM Method Prototype

N/A

(use the OutputPhases.FrequencyRampBusy  property)

C Prototype

ViStatus IviACPwr_QueryFrequencyRampBusy (ViSession Vi,
                                      ViBoolean *Busy);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |

| Outputs | Description | Base Type |
| --- | --- | --- |
| Busy | Returns True if a frequency ramp is in progress. Returns False when a Frequency Ramp completes or is not in progress. | ViBoolean |

Defined Values for Busy Parameter

| Name | Description |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Language |  | Identifier |
| True | Frequency ramp in progress. |  |  |  |
|  |  | C | VI_TRUE |  |
|  |  | COM | True |  |
| False | Frequency ramp not in progress |  |  |  |
|  |  | C | VI_FALSE |  |
|  |  | COM | False |  |

### Abort Frequency Ramp

Description

Aborts a  frequency  ramp. If a frequency  ramp is not in progress this function does nothing.

COM Method Prototype

HRESULT OutputPhases.AbortFrequencyRamp();

C Prototype

ViStatus IviACPwr_AbortFrequencyRamp (ViSession Vi);

Parameters

| Inputs | Description | Base Type |
| --- | --- | --- |
| Vi | Instrument handle | ViSession |

Return Values

The IVI-3.2:  Inherent Capabilities Specification defines general status codes that this function can return.

## IviACPwrFrequencyRamp Behavior Model

The IviACPwrFrequencyRamp Extension Group follows the behavior model of the IviACPwrBase capability group. The only modification to the behavior model from the IviACPwrBase capability group is the ability to configure a frequency ramp.

When the Ramp Frequency function returns immediately after the ramp has started, the user should only call the Query Frequency Ramp Busy or Abort Frequency Ramp functions or read the Frequency Ramp Busy attribute while the ramp is in progress. Calling any other function or reading or writing any other attribute may abort the ramp or cause other vendor specific instrument behavior. After the frequency ramp completes or is aborted, the driver must invalidate the Frequency attribute in the IviACPwr capability group.

# IviACPwr Attribute ID Definitions

The following table defines the ID value for all IviACPwr class attributes.

| Table 16-3. IviACPwr Attributes ID Values |  |
| --- | --- |
| Attribute Name | ID Definition |
| IVIACPWR_ATTR_FREQUENCY | IVI_CLASS_ATTR_BASE+ 1 |
| IVIACPWR_ATTR_NUM_PHASES | IVI_CLASS_ATTR_BASE+ 2 |
| IVIACPWR_ATTR_NUM_FREQUENCY_RANGES | IVI_CLASS_ATTR_BASE+ 3 |
| IVIACPWR_ATTR_FREQUENCY_RANGE | IVI_CLASS_ATTR_BASE+ 4 |
| IVIACPWR_ATTR_NUM_WAVEFORMS_MAX | IVI_CLASS_ATTR_BASE+ 5 |
| IVIACPWR_ATTR_NUM_OPTIMAL_DATA_POINTS | IVI_CLASS_ATTR_BASE+ 6 |
| IVIACPWR_ATTR_FIXED_WAVEFORM_COUNT | IVI_CLASS_ATTR_BASE+ 7 |
| IVIACPWR_ATTR_USER_WAVEFORM_COUNT | IVI_CLASS_ATTR_BASE+ 8 |
| IVIACPWR_ATTR_AVAILABLE_WAVEFORM_COUNT | IVI_CLASS_ATTR_BASE+ 9 |
| IVIACPWR_ATTR_EXTERNAL_SYNC_ENABLED | IVI_CLASS_ATTR_BASE+ 10 |
| IVIACPWR_ATTR_EXTERNAL_SYNC_PHASE_OFFSET | IVI_CLASS_ATTR_BASE+ 11 |
| IVIACPWR_ATTR_EXTERNAL_SYNC_LOCKED | IVI_CLASS_ATTR_BASE+ 12 |
| IVIACPWR_ATTR_CURRENT_LIMIT | IVI_CLASS_ATTR_BASE+ 13 |
| IVIACPWR_ATTR_OUTPUT_ENABLED | IVI_CLASS_ATTR_BASE+ 14 |
| IVIACPWR_ATTR_NUM_VOLTAGE_RANGES | IVI_CLASS_ATTR_BASE+ 15 |
| IVIACPWR_ATTR_VOLTAGE_LEVEL | IVI_CLASS_ATTR_BASE+ 16 |
| IVIACPWR_ATTR_WAVEFORM | IVI_CLASS_ATTR_BASE+ 17 |
| IVIACPWR_ATTR_VOLTAGE_RANGE | IVI_CLASS_ATTR_BASE+ 18 |
| IVIACPWR_ATTR_PHASE_ANGLE | IVI_CLASS_ATTR_BASE+ 19 |
| IVIACPWR_ATTR_OUTPUT_IMPEDANCE_INDUCTIVE | IVI_CLASS_ATTR_BASE+ 20 |
| IVIACPWR_ATTR_OUTPUT_IMPEDANCE_ENABLED | IVI_CLASS_ATTR_BASE+ 21 |
| IVIACPWR_ATTR_OUTPUT_IMPEDANCE_RESISTIVE | IVI_CLASS_ATTR_BASE+ 22 |
| IVIACPWR_ATTR_CURRENT_PROTECTION_DELAY | IVI_CLASS_ATTR_BASE+ 23 |
| IVIACPWR_ATTR_CURRENT_PROTECTION_ENABLED | IVI_CLASS_ATTR_BASE+ 24 |
| IVIACPWR_ATTR_CURRENT_PROTECTION_THRESHOLD | IVI_CLASS_ATTR_BASE+ 25 |
| IVIACPWR_ATTR_CURRENT_PROTECTION_TRIPPED | IVI_CLASS_ATTR_BASE+ 26 |
| IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_ENABLED | IVI_CLASS_ATTR_BASE+ 27 |
| IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_LIMIT | IVI_CLASS_ATTR_BASE+ 28 |
| IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_ENABLED | IVI_CLASS_ATTR_BASE+ 29 |
| IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_LIMIT | IVI_CLASS_ATTR_BASE+ 30 |
| IVIACPWR_ATTR_VOLTAGE_PROTECTION_TRIPPED | IVI_CLASS_ATTR_BASE+ 31 |
| IVIACPWR_ATTR_DC_MODE | IVI_CLASS_ATTR_BASE+ 32 |
| IVIACPWR_ATTR_DC_VOLTAGE_LEVEL | IVI_CLASS_ATTR_BASE+ 33 |
| IVIACPWR_ATTR_DC_NUM_RANGES | IVI_CLASS_ATTR_BASE+ 35 |
| IVIACPWR_ATTR_OUTPUT_PHASE_COUNT | IVI_CLASS_ATTR_BASE+ 36 |
| IVIACPWR_ATTR_VOLTAGE_RAMP_BUSY | IVI_CLASS_ATTR_BASE+ 37 |
| IVIACPWR_ATTR_CURRENT_RAMP_BUSY | IVI_CLASS_ATTR_BASE+ 38 |
| IVIACPWR_ATTR_FREQUENCY_RAMP_BUSY | IVI_CLASS_ATTR_BASE+ 39 |
| IVIACPWR_ATTR_DC_RANGE_MIN | IVI_CLASS_ATTR_BASE+ 40 |
| IVIACPWR_ATTR_DC_RANGE_MAX | IVI_CLASS_ATTR_BASE+ 41 |

# IviACPwr Attribute Value Definitions

This section specifies the actual value for each defined attribute value.

DC Mode

| Value Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| AC Only | C | IVIACPWR_VAL_MODE_AC_ONLY | 0 |
|  | COM | IviACPwrDCGenerationModeACOnly | 0 |
| DC Only | C | IVIACPWR_VAL_MODE_DC_ONLY | 1 |
|  | COM | IviACPwrDCGenerationModeDCOnly | 1 |
| AC Plus DC | C | IVIACPWR_VAL_MODE_AC_DC | 2 |
|  | COM | IviACPwrDCGenerationModeACPlusDC | 2 |
| DC Mode Class Extension Base | C | IVIACPWR_VAL_DC_MODE_CLASS_EXT_BASE | 500 |
| DC Mode Specific Extension Base | C | IVIACPWR_VAL_DC_MODE_SPECIFIC_EXT_BASE | 1000 |

# IviACPwr Function Parameter Value Definitions

This section specifies the actual values for each function parameter that defines values.

Fetch Measurement

| Parameter: MeasurementType |  |  |  |  |
| --- | --- | --- | --- | --- |
| COM Enumeration Name: IviACPwrMeasurementTypeEnum |  |  |  |  |
| Value Name | Language | Identifier | Actual Value |  |
| Voltage RMS L-N | C | IVIACPWR_VAL_MEASURE_VOLTAGE_RMS_LN | 0 |  |
|  | COM | IviACPwrMeasureVoltageRMS | 0 |  |
| Current RMS | C | IVIACPWR_VAL_MEASURE_CURRENT_RMS | 1 |  |
|  | COM | IviACPwrMeasureCurrentRMS | 1 |  |
| Frequency | C | IVIACPWR_VAL_MEASURE_FREQUENCY | 2 |  |
|  | COM | IviACPwrMeasureFrequency | 2 |  |
| Voltage DC | C | IVIACPWR_VAL_MEASURE_VOLTAGE_DC | 3 |  |
|  | COM | IviACPwrMeasureVoltageDC | 3 |  |
| Current DC | C | IVIACPWR_VAL_MEASURE_CURRENT_DC | 4 |  |
|  | COM | IviACPwrMeasureCurrentDC | 4 |  |
| Power Factor | C | IVIACPWR_VAL_MEASURE_POWER_FACTOR | 5 |  |
|  | COM | IviACPwrMeasurePowerFactor | 5 |  |
| Crest Factor | C | IVIACPWR_VAL_MEASURE_CREST_FACTOR | 6 |  |
|  | COM | IviACPwrMeasureCrestFactor | 6 |  |
| Current Peak | C | IVIACPWR_VAL_MEASURE_CURRENT_PEAK | 7 |  |
|  | COM | IviACPwrMeasureCurrentPeak | 7 |  |
| Power VA | C | IVIACPWR_VAL_MEASURE_POWER_VA | 8 |  |
|  | COM | IviACPwrMeasurePowerVA | 8 |  |
| Real Power | C | IVIACPWR_VAL_MEASURE_POWER_REAL | 9 |  |
|  | COM | IviACPwrMeasurePowerReal | 9 |  |
| Power DC | C | IVIACPWR_VAL_MEASURE_POWER_DC | 10 |  |
|  | COM | IviACPwrMeasurePowerDC | 10 |  |
| Phase Angle | C | IVIACPWR_VAL_MEASURE_PHASE_ANGLE | 11 |  |
|  | COM | IviACPwrMeasurePhaseAngle | 11 |  |
| Voltage RMS L-L | C | IVIACPWR_VAL_MEASURE_VOLTAGE_RMS_LL | 12 |  |
|  | COM | IviACPwrMeasureVoltageRMSLineToLine | 12 |  |

| Current OHD | C | IVIACPWR_VAL_MEASURE_CURRENT_OHD | 13 |
| --- | --- | --- | --- |
|  | COM | IviACPwrMeasureCurrentOHD | 13 |
| Current EHD | C | IVIACPWR_VAL_MEASURE_CURRENT_EHD | 14 |
|  | COM | IviACPwrMeasureCurrentEHD | 14 |
| Current THD | C | IVIACPWR_VAL_MEASURE_CURRENT_THD | 15 |
|  | COM | IviACPwrMeasureCurrentTHD | 15 |
| Voltage OHD | C | IVIACPWR_VAL_MEASURE_VOLTAGE_OHD | 16 |
|  | COM | IviACPwrMeasureVoltageOHD | 16 |
| Voltage EHD | C | IVIACPWR_VAL_MEASURE_VOLTAGE_EHD | 17 |
|  | COM | IviACPwrMeasureVoltageEHD | 17 |
| Voltage THD | C | IVIACPWR_VAL_MEASURE_VOLTAGE_THD | 18 |
|  | COM | IviACPwrMeasureVoltageTHD | 18 |

Fetch Measurement Array

| Parameter: MeasurementType |  |  |  |
| --- | --- | --- | --- |
| COM Enumeration Name: IviACPwrMeasurementTypeArrayEnum |  |  |  |
| Value Name | Language | Identifier | Actual Value |
| Array Current Harmonic Phase | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_HARMONIC_PHASE | 0 |
|  | COM | IviACPwrMeasureArrayCurrentHarmonicPhase | 0 |
| Array Current Harmonic Magnitude Absolute | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_HARMONIC_ABS | 1 |
|  | COM | IviACPwrMeasureArrayCurrentHarmonicAbs | 1 |
| Array Current Harmonic Magnitude Percent Fundamental | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_HARMONIC_PCT | 2 |
|  | COM | IviACPwrMeasureArrayCurrentHarmonicPct | 2 |
| Array Voltage Harmonic Phase | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_PHASE | 3 |
|  | COM | IviACPwrMeasureArrayVoltageHarmonicPhase | 3 |
| Array Voltage Harmonic Magnitude Absolute | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_ABS | 4 |
|  | COM | IviACPwrMeasureArrayVoltageHarmonicAbs | 4 |
| Array Voltage Harmonic Magnitude Percent Fundamental | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_HARMONIC_PCT | 5 |
|  | COM | IviACPwrMeasureArrayVoltageHarmonicPct | 5 |
| Array Current Cycle | C | IVIACPWR_VAL_MEASURE_ARRAY_CURRENT_CYCLE | 6 |
|  | COM | IviACPwrMeasureArrayCurrentCycle | 6 |
| Array Voltage Cycle | C | IVIACPWR_VAL_MEASURE_ARRAY_VOLTAGE_CYCLE | 7 |
|  | COM | IviACPwrMeasureArrayVoltageCycle | 7 |

DC Generation

| Parameter: Mode |  |  |  |
| --- | --- | --- | --- |
| COM Enumeration Name: IviACPwrDCGenerationModeEnum |  |  |  |
| Value Name | Language | Identifier | Actual Value |
| AC Only | C | IVIACPWR_VAL_MODE_AC_ONLY | 0 |
|  | COM | IviACPwrModeACOnly | 0 |
| DC Only | C | IVIACPWR_VAL_MODE_DC_ONLY | 1 |
|  | COM | IviACPwrModeDCOnly | 1 |
| AC Plus DC | C | IVIACPWR_VAL_MODE_AC_DC | 2 |
|  | COM | IviACPwrModeACPlusDC | 2 |

Initiate Measurement

| Parameter: Group |  |  |  |
| --- | --- | --- | --- |
| COM Enumeration Name: IviACPwrMeasurementGroupEnum |  |  |  |
| Value Name | Language | Identifier | Actual Value |
| Base | C | IVIACPWR_VAL_MEASUREMENT_GROUP_BASE | 1 |
|  | COM | IviACPwrMeasurementGroupBase | 1 |
| Harmonic | C | IVIACPWR_VAL_MEASUREMENT_GROUP_HARMONIC | 2 |
|  | COM | IviACPwrMeasurementGroupHarmonic | 2 |
| Distortion | C | IVIACPWR_VAL_MEASUREMENT_GROUP_DISTORTION | 4 |
|  | COM | IviACPwrMeasurementGroupDistortion | 4 |
| Waveform | C | IVIACPWR_VAL_MEASUREMENT_GROUP_WAVEFORM | 8 |
|  | COM | IviACPwrMeasurementGroupWaveform | 8 |

Query Arbitrary Waveform Catalog

| Parameter: CatalogType |  |  |  |
| --- | --- | --- | --- |
| COM Enumeration Name: IviACPwrDCGenerationModeEnum |  |  |  |
| Value Name | Language | Identifier | Actual Value |
| Catalog Fixed | C | IVIACPWR_VAL_WAVEFORM_CATALOG_FIXED | 0 |
|  | COM | IviACPwrWaveformCatalogFixed | 0 |
| Catalog User | C | IVIACPWR_VAL_WAVEFORM_CATALOG_USER | 1 |
|  | COM | IviACPwrWaveformCatalogUser | 1 |
| Catalog All | C | IVIACPWR_VAL_WAVEFORM_CATALOG_ALL | 2 |
|  | COM | IviACPwrWaveformCatalogAll | 2 |

# Error and Completion Code Value Definitions

The IviACPwr class specification defines the additional status codes.

| Error Name | Language | Identifier | Actual Value |
| --- | --- | --- | --- |
| All Phases Required | C | IVIACPWR_ERROR_ALL_PHASES_REQUIRED | 0xBFFA2002 |
|  | COM | E_IVIACPWR_ALL_PHASES_REQUIRED | 0x80042002 |
| Current Protection Tripped | C | IVIACPWR_ERROR_CURRENT_PROTECTION | 0xBFFA2003 |
|  | COM | E_IVIACPWR_CURRENT_PROTECTION | 0x80042003 |
| Duplicate Waveform Name | C | IVIACPWR_ERROR_DUPLICATE_WAVEFORM_NAME | 0xBFFA2004 |
|  | COM | E_IVIACPWR_DUPLICATE_WAVEFORM_NAME | 0x80042004 |
| Invalid Waveform Name | C | IVIACPWR_ERROR_INVALID_WAVEFORM_NAME | 0xBFFA2005 |
|  | COM | E_IVIACPWR_INVALID_WAVEFORM_NAME | 0x80042005 |
| Measurement Not Initiated | C | IVIACPWR_ERROR_MEASUREMENT_NOT_INITIATED | 0xBFFA2006 |
|  | COM | E_IVIACPWR_MEASUREMENT_NOT_INITIATED | 0x80042006 |
| Measurement Not Supported | C | IVIACPWR_ERROR_MEASUREMENT_NOT_SUPPORTED | 0xBFFA2007 |
|  | COM | E_IVIACPWR_MEASUREMENT_NOT_SUPPORTED | 0x80042007 |
| No Waveforms Available | C | IVIACPWR_ERROR_NO_WAVEFORMS_AVAILABLE | 0xBFFA2008 |
|  | COM | E_IVIACPWR_NO_WAVEFORMS_AVAILABLE | 0x80042008 |
| Phase Values Different | C | IVIACPWR_ERROR_PHASE_VALUES_DIFFERENT | 0xBFFA200A |
|  | COM | E_IVIACPWR_PHASE_VALUES_DIFFERENT | 0x8004200A |
| Unsupported Measurement Group | C | IVIACPWR_ERROR_UNSUPPORTED_MEASUREMENT_GROUP | 0xBFFA200B |
|  | COM | E_IVIACPWR_UNSUPPORTED_MEASUREMENT_GROUP | 0x8004200B |
| Voltage Protection Tripped | C | IVIACPWR_ERROR_VOLTAGE_PROTECTION | 0xBFFA200C |
|  | COM | E_IVIACPWR_VOLTAGE_PROTECTION | 0x8004200C |
| Waveform In Use | C | IVIACPWR_ERROR_WAVEFORM_IN_USE | 0xBFFA200E |
|  | COM | E_IVIACPWR_WAVEFORM_IN_USE | 0x8004200E |
| Waveform Not Found | C | IVIACPWR_ERROR_WAVEFORM_NOT_FOUND | 0xBFFA200F |
|  | COM | E_IVIACPWR_WAVEFORM_NOT_FOUND | 0x8004200F |
| Waveform Reserved | C | IVIACPWR_ERROR_WAVEFORM_RESERVED | 0xBFFA2010 |
|  | COM | E_IVIACPWR_WAVEFORM_RESERVED | 0x80042010 |
| Waveform DC Offset | C | IVIACPWR_ERROR_WAVEFORM_DC_OFFSET | 0xBFFA200D |
|  | COM | E_IVIACPWR_WAVEFORM_DC_OFFSET | 0x8004200D |
| All Phases Not Supported | C | IVIACPWR_ERROR_ALL_PHASES_NOT_SUPPORTED | 0xBFFA2001 |
|  | COM | E_IVIACPWR_ALL_PHASES_NOT_SUPPORTED | 0x80042001 |
| Output Enabled | C | IVIACPWR_ERROR_OUTPUT_ENABLED | 0xBFFA2009 |
|  | COM | E_IVIACPWR_OUTPUT_ENABLED | 0x80042009 |

# IviACPwr Hierarchies

## IviACPwr COM Hierarchy

The full IviACPwr COM Hierarchy includes the Inherent Capabilities Hierarchy as defined in Section 4.1, COM Inherent Capabilities of IVI-3.2: Inherent Capabilities Specification. To avoid redundancy, the Inherent Capabilities are omitted here.

| Table 20-4. IviACPwr COM Hierarchy |  |  |
| --- | --- | --- |
| COM Interface Hierarchy | Generic Name | Type |
| OutputPhases |  |  |
| Count | Output Phase Count | P |
| Name | Output Phase Name | P |
| Frequency | Frequency | P |
| FrequencyRange | Frequency Range | P |
| NumFrequencyRanges | Number of Frequency Ranges | P |
| NumPhases | Number of Phases | P |
| FrequencyRampBusy | Frequency Ramp Busy | P |
| QueryFrequencyRangeCapabilities | Query Frequency Range Capabilities | M |
| InitiateMeasurement | Initiate Measurement | M |
| RampFrequency | Ramp Frequency | M |
| AbortFrequencyRamp | Abort Frequency Ramp | M |
| Item |  |  |
| Enabled | Output Enabled | P |
| VoltageLevel | Voltage Level | P |
| Waveform | Waveform | P |
| PhaseAngle | Phase Angle | P |
| VoltageRange | Voltage Range | P |
| NumVoltageRanges | Number of Voltage Ranges | P |
| CurrentLimit | Current Limit | P |
| VoltageRampBusy | Voltage Ramp Busy | P |
| CurrentRampBusy | Current Ramp Busy | P |
| FetchMeasurement | Fetch Measurement | M |
| FetchMeasurementArray | Fetch Measurement Array | M |
| QueryVoltageRangeCapabilities | Query Voltage Range Capabilities | M |

| RampVoltage | Ramp Voltage | M |
| --- | --- | --- |
| AbortVoltageRamp | Abort Voltage Ramp | M |
| RampCurrent | Ramp Current | M |
| AbortCurrentRamp | Abort Current Ramp | M |
| Impedance |  |  |
| Resistive | Output Impedance Resistive | P |
| Inductive | Output Impedance Inductive | P |
| Enabled | Output Impedance Enabled | P |
| Configure | Configure Output Impedance | M |
| QueryCapabilities | Query Output Impedance Capabilities | M |
| Current Protection |  |  |
| Enabled | Current Protection Enabled | P |
| Delay | Current Protection Delay | P |
| Threshold | Current Protection Threshold | P |
| Tripped | Current Protection Tripped | P |
| Configure | Configure Current Protection | M |
| Reset | Reset Current Protection | M |
| Voltage Protection |  |  |
| OverEnabled | Over Voltage Enabled | P |
| OverLimit | Over Voltage Limit | P |
| UnderEnabled | Under Voltage Enabled | P |
| UnderLimit | Under Voltage Limit | P |
| Tripped | Voltage Protection Tripped | P |
| Configure | Configure Voltage Protection | M |
| Reset | Reset Voltage Protection | M |
| DC Generation |  |  |
| Mode | DC Mode | P |
| DCVoltageLevel | DC Voltage Level | P |
| RangeMin | DC Range Min | P |
| RangeMax | DC Range Max | P |
| NumRanges | Number of DC Ranges | P |
| Configure | Configure DC | M |
| ConfigureRange | Configure DC Range | M |
| QueryCapabilities | Query DC Capabilities | M |
| External Sync |  |  |
| Enabled | Enabled | P |
| PhaseOffset | Phase Offset | P |
| Locked | Locked | P |
| Configure | Configure | M |
| Arb Waveform |  |  |
| NumWaveformsMax | Num Waveforms Max | P |
| NumOpimalDataPoints | Num Optimal Data Points | P |
| FixedWaveformCount | Fixed Waveform Count | P |
| UserWaveformCount | User Waveform Count | P |
| AvailableWaveformCount | Available Waveform Count | P |
| Clear | Clear Waveform | M |
| Write | Write Waveform | M |
| QueryCatalog | Query Waveform Catalog | M |

### IviACPwr COM Interfaces

In addition to implementing IVI inherent capabilities interfaces, IviACPwr interfaces contain interface reference properties for accessing the following IviACPwr interfaces:

- IIviACPwrOutputPhases

- IIviACPwrArbWaveform

The IIviACPwrOutputPhases interface contains methods and properties for accessing a collection of objects that implement the IIviACPwrOutputPhase interface. The IIviACPwrOutputPhases interface contains interface reference properties for accessing the following IviACPwr interfaces:

- IIviACPwrExternalSync

The IIviACPwrOutputPhase interface contains interface reference properties for accessing the following IviACPwr interfaces:

- IIviACPwrCurrentProtection

- IIviACPwrVoltageProtection

- IIviACPwrImpedance

- IIviACPwrDCGeneration

Table 20-5 IviACPwr Interface GUIDs lists the interfaces that this specification defines and their GUIDs.

| Table 20-5 IviACPwr Interface GUIDs |  |
| --- | --- |
| Interface | GUID |
| IIviACPwr | { 47ed5440-a398-11d4-ba58-000064657374} |
| IIviACPwrOutputPhases | { 47ed5441-a398-11d4-ba58-000064657374} |
| IIviACPwrOutputPhase | { 47ed5442-a398-11d4-ba58-000064657374} |
| IIviACPwrArbWaveform | { 47ed5443-a398-11d4-ba58-000064657374} |
| IIviACPwrExternalSync | { 47ed5444-a398-11d4-ba58-000064657374} |
| IIviACPwrCurrentProtection | { 47ed5445-a398-11d4-ba58-000064657374} |
| IIviACPwrVoltageProtection | { 47ed5446-a398-11d4-ba58-000064657374} |
| IIviACPwrImpedance | { 47ed5447-a398-11d4-ba58-000064657374} |
| IIviACPwrDCGeneration | { 47ed5448-a398-11d4-ba58-000064657374} |

### IviACPwr COM Interface Reference Properties

Interface reference properties are used to navigate the IviACPwr COM hierarchy. This section describes the interface reference properties that the IIviACPwr interface defines.

#### OutputPhases

| Data Type | Access |
| --- | --- |
| IIviACPwrOutputPhases* | RO |

COM Property Name

OutputPhases

Description

Returns a pointer to the IIviACPwrOutputPhases interface.

#### OutputPhase

| Data Type | Access |
| --- | --- |
| IIviACPwrOutputPhase* | RO |

COM Property Name

OutputPhase

Description

Returns a pointer to the IIviACPwrOutputPhase interface.

#### Arb Waveform

| Data Type | Access |
| --- | --- |
| IIviACPwrArbWaveform* | RO |

COM Property Name

ArbWaveform

Description

Returns a pointer to the IIviACPwrArbWaveform interface.

#### External Sync

| Data Type | Access |
| --- | --- |
| IIviACPwrExternalSync* | RO |

COM Property Name

ExternalSync

Description

Returns a pointer to the IIviACPwrExternalSync interface.

#### Current Protection

| Data Type | Access |
| --- | --- |
| IIviACPwrCurrentProtection* | RO |

COM Property Name

CurrentProtection

Description

Returns a pointer to the IIviACPwrCurrentProtection interface.

#### Voltage Protection

| Data Type | Access |
| --- | --- |
| IIviACPwrVoltageProtection* | RO |

COM Property Name

VoltageProtection

Description

Returns a pointer to the IIviACPwrVoltageProtection interface.

#### Impedance

| Data Type | Access |
| --- | --- |
| IIviACPwrImpedance* | RO |

COM Property Name

Impedance

Description

Returns a pointer to the IIviACPwrImpedance interface.

#### DC Generation

| Data Type | Access |
| --- | --- |
| IIviACPwrDCGeneration* | RO |

COM Property Name

DCGeneration

Description

Returns a pointer to the IIviACPwrDCGeneration interface.

### IviACPwr COM Category

The IviACPwr class COM Category shall be “IviACPwr”, and the Category ID (CATID) shall be {47ed5161-a398-11d4-ba58-000064657374}.

## IviACPwr C Function Hierarchy

The IviACPwr class function hierarchy is shown in the following table. The full IviACPwr C Function Hierarchy includes the Inherent Capabilities Hierarchy as defined in Section 4.2, C Inherent Capabilities of IVI-3.2: Inherent Capabilities Specification. To avoid redundancy, the Inherent Capabilities are omitted here.

Table 20-6. IviACPwr C Function Hierarchy

| Name or Class | Function Name |
| --- | --- |
| Configuration |  |
| Configure Voltage Level | IviACPwr_ConfigureVoltageLevel |
| Configure Current Limit | IviACPwr_ConfigureCurrentLimit |
| Configure Output Enabled | IviACPwr_ConfigureOutputEnabled |
| Configure Waveform | IviACPwr_ConfigureWaveform |
| Configure Voltage Range | IviACPwr_ConfigureVoltageRange |
| Query Voltage Range Capabilities | IviACPwr_QueryVoltageRangeCapabilities |
| Get Output Phase Name | IviACPwr_GetOutputPhaseName |
| Configure Frequency | IviACPwr_ConfigureFrequency |
| Configure Frequency Range | IviACPwr_ConfigureFrequencyRange |
| Query Frequency Range Capabilities | IviACPwr_QueryFrequencyRangeCapabilities |
| Phase Angle |  |
| Configure Phase Angle | IviACPwr_ConfigurePhaseAngle |
| Arb Waveform |  |
| Clear Arbitrary Waveform | IviACPwr_ClearArbWaveform |
| Query Arbitrary Waveform Catalog | IviACPwr_QueryArbWaveformCatalog |
| Write Arbitrary Waveform | IviACPwr_WriteArbWaveform |
| Impedance |  |
| Configure Output Impedance | IviACPwr_ConfigureOutputImpedance |
| Query Output Impedance Capabilities | IviACPwr_QueryOutputImpedanceCapabilities |
| Voltage Protection |  |
| Configure Voltage Protection | IviACPwr_ConfigureVoltageProtection |
| Query Voltage Protection Tripped | IviACPwr_QueryVoltageProtectionTripped |
| Reset Voltage Protection | IviACPwr_ResetVoltageProtection |
| Current Protection |  |
| Configure Current Protection | IviACPwr_ConfigureCurrentProtection |
| Query Current Protection Tripped | IviACPwr_QueryCurrentProtectionTripped |
| Reset Current Protection | IviACPwr_ResetCurrentProtection |
| DC Generation |  |
| Configure DC | IviACPwr_ConfigureDC |
| Configure DC Range | IviACPwr_ConfigureDCRange |
| Query DC Capabilities | IviACPwr_QueryDCCapabilities |
| External Sync |  |
| Configure External Sync | IviACPwr_ConfigureExternalSync |
| Query External Sync Locked | IviACPwr_QueryExternalSyncLocked |

| Action/Status |  |
| --- | --- |
| Voltage Ramp |  |
| Ramp Voltage | IviACPwr_RampVoltage |
| Abort Voltage Ramp | IviACPwr_AbortVoltageRamp |
| Query Voltage Ramp Busy | IviACPwr_QueryVoltageRampBusy |
| Current Ramp |  |
| Ramp Current | IviACPwr_RampCurrent |
| Abort Current Ramp | IviACPwr_AbortCurrentRamp |
| Query Current Ramp Busy | IviACPwr_QueryCurrentRampBusy |
| Frequency Ramp |  |
| Ramp Frequency | IviACPwr_RampFrequency |
| Abort Frequency Ramp | IviACPwr_AbortFrequencyRamp |
| Query Frequency Ramp Busy | IviACPwr_QueryFrequencyRampBusy |
| Measurement |  |
| Initiate Measurement | IviACPwr_InitiateMeasurement |
| Fetch Measurement | IviACPwr_FetchMeasurement |
| Fetch Measurement Array | IviACPwr_FetchMeasurementArray |

## IviACPwr C Attribute Hierarchy

The IviACPwr class attribute hierarchy is shown in the following table. The full IviACPwr C Attribute Hierarchy includes the Inherent Capabilities Hierarchy as defined in Section 4.2, C Inherent Capabilities of IVI-3.2: Inherent Capabilities Specification. To avoid redundancy, the Inherent Capabilities are omitted here.

|  | Table 20-7. IviACPwr C Attributes Hierarchy |  |
| --- | --- | --- |
|  | Category or Generic Attribute Name | C Defined Constant |
|  | Output |  |
|  | Current Limit | IVIACPWR_ATTR_CURRENT_LIMIT |
|  | Waveform | IVIACPWR_ATTR_WAVEFORM |
|  | Output Enabled | IVIACPWR_ATTR_OUTPUT_ENABLED |
|  | Voltage Range | IVIACPWR_ATTR_VOLTAGE_RANGE |
|  | Num Voltage Ranges | IVIACPWR_ATTR_NUM_VOLTAGE_RANGES |
|  | Voltage Level | IVIACPWR_ATTR_VOLTAGE_LEVEL |
|  | Phase Angle |  |
|  | Phase Angle | IVIACPWR_ATTR_PHASE_ANGLE |
|  | Outputs |  |
|  | Frequency | IVIACPWR_ATTR_FREQUENCY |
|  | Frequency Range | IVIACPWR_ATTR_FREQUENCY_RANGE |
|  | Num Frequency Ranges | IVIACPWR_ATTR_NUM_FREQUENCY_RANGES |
|  | Num Phases | IVIACPWR_ATTR_NUM_PHASES |
|  | Output Phase Count | IVIACPWR_ATTR_OUTPUT_PHASE_COUNT |
|  | Voltage Ramp |  |
|  | Voltage Ramp Busy | IVIACPWR_ATTR_VOLTAGE_RAMP_BUSY |
|  | Current Ramp |  |
|  | Current Ramp Busy | IVIACPWR_ATTR_CURRENT_RAMP_BUSY |
|  | Frequency Ramp |  |
|  | Frequency Ramp Busy | IVIACPWR_ATTR_FREQUENCY_RAMP_BUSY |
|  | ArbWaveform |  |
|  | Num Waveforms Max | IVIACPWR_ATTR_NUM_WAVEFORMS_MAX |
|  | Num Optimal Data Points | IVIACPWR_ATTR_NUM_OPTIMAL_DATA_POINTS |
|  | Fixed Waveform Count | IVIACPWR_ATTR_FIXED_WAVEFORM_COUNT |
|  | User Waveform Count | IVIACPWR_ATTR_USER_WAVEFORM_COUNT |
|  | Available Waveform Count | IVIACPWR_ATTR_AVAILABLE_WAVEFORM_COUNT |
|  | Impedance |  |
|  | Impedance Inductive | IVIACPWR_ATTR_OUTPUT_IMPEDANCE_INDUCTIVE |
|  | Impedance Resistive | IVIACPWR_ATTR_OUTPUT_IMPEDANCE_RESISTIVE |
|  | Impedance Enabled | IVIACPWR_ATTR_OUTPUT_IMPEDANCE_ENABLED |

| Voltage Protection |  |
| --- | --- |
| Over Voltage Protection Enabled | IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_ENABLED |
| Over Voltage Protection Limit | IVIACPWR_ATTR_OVER_VOLTAGE_PROTECTION_LIMIT |
| Under Voltage Protection Enabled | IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_ENABLED |
| Under Voltage Protection Limit | IVIACPWR_ATTR_UNDER_VOLTAGE_PROTECTION_LIMIT |
| Voltage Protection Tripped | IVIACPWR_ATTR_VOLTAGE_PROTECTION_TRIPPED |
| Current Protection |  |
| Current Protection Delay | IVIACPWR_ATTR_CURRENT_PROTECTION_DELAY |
| Current Protection Enabled | IVIACPWR_ATTR_CURRENT_PROTECTION_ENABLED |
| Current Protection Threshold | IVIACPWR_ATTR_CURRENT_PROTECTION_THRESHOLD |
| Current Protection Tripped | IVIACPWR_ATTR_CURRENT_PROTECTION_TRIPPED |
| DC Generation |  |
| Mode | IVIACPWR_ATTR_DC_MODE |
| DCVoltageLevel | IVIACPWR_ATTR_DC_VOLTAGE_LEVEL |
| Range Min | IVIACPWR_ATTR_DC_RANGE_MIN |
| Range Max | IVIACPWR_ATTR_DC_RANGE_MAX |
| Num Ranges | IVIACPWR_ATTR_DC_NUM_RANGES |
| External Sync |  |
| Enabled | IVIACPWR_ATTR_EXTERNAL_SYNC_EABLED |
| Phase Offset | IVIACPWR_ATTR_EXTERNAL_SYNC_PHASE_OFFSET |
| Locked | IVIACPWR_ATTR_EXTERNAL_SYNC_LOCKED |

- Specific Driver Development Guidelines

  - Introduction

This section describes situations driver developers should be aware of when developing a specific instrument driver that complies with the IviACPwr class.

  - Disabling Unused Extensions

Specific drivers are required to disable extension capability groups that an application program does not explicitly use. The specific driver can do so by setting the attributes of an extension capability group to the values that this section recommends. A specific driver can set these values for all extension capability groups when the Initialize or Reset functions execute. This assumes that the extension capability groups remain disabled until the application program explicitly uses them. For the large majority of instruments, this assumption is true.

Under certain conditions, a specific driver might have to implement a more complex approach. For some instruments, configuring a capability group might affect instrument settings that correspond to an unused extension capability group. If these instrument settings affect the behavior of the instrument, then this might result in an interchangeability problem. If this can occur, the specific driver must take appropriate action so that the instrument settings that correspond to the unused extension capability group do not affect the behavior of the instrument when the application program performs an operation that might be affected by those settings.

The remainder of this section recommends attribute values that effectively disable each extension capability group.

- Disabling the IviACPwrMeasurement Extension Group

The IviACPwrMeasurement extension group affects the instrument behavior only when the user calls the measurement functions. Therefore, this specification does not recommend attribute values that disable the IviACPwrMeasurement extension group.

- Disabling the IviACPwrPhase Extension Group

The Phase Angle attribute value in the IviACPwrPhase extension group for each output phase should be initialized to a known value based on the following equation:

Phase Angle = 360 – [ (Phase Number – 1) * 360 / Number of Phases]

Where Phase Number is the 1 based output phase index and Number of Phases is the attribute defined in IviACPwrBase. For example, the initialized phase angle values for a three phase power source are shown in the following table.

| Output Phase | Phase Angle Attribute Value |
| --- | --- |
| 2 (Phase B) | 240 |
| 3 (Phase C) | 120 |

- Disabling the IviACPwrExternalSync Extension Group

Attribute values that effectively disable the IviACPwrExternalSync extension group are shown in the following table.

| Attribute | Value |
| --- | --- |
| External Synchronization Enabled | False |

- Disabling the IviACPwrCurrentProtection Extension Group

Attribute values that effectively disable the IviACPwrCurrentProtection extension group are shown in the following table.

| Attribute | Value |
| --- | --- |
| Current Protection Enabled | False |

- The instrument driver must also make sure the AC power source is not in a current protection state

- Disabling the IviACPwrVoltageProtection Extension Group

Attribute values that effectively disable the IviACPwrVoltageProtection extension group are shown in the following table.

| Attribute | Value |
| --- | --- |
| Under Voltage Enabled | False |
| Over Voltage Enabled | False |

- The instrument driver must also make sure the AC power source is not in a voltage protection state

- Disabling the IviACPwrArbWaveform Extension Group

The IviACPwrArbWaveform extension group affects the instrument behavior only when the Waveform attribute in IviACPwrBase group is set to a value other than ‘Sine’. Therefore, this specification recommends the attribute value for all output phases be set as shown in the following table.

| Capability Group | Attribute | Value |
| --- | --- | --- |
| Base | Waveform | ‘Sine’ |

- Disabling the IviACPwrImpedance Extension Group

Attribute values that effectively disable the IviACPwrImpedance extension group are shown in the following table.

| Attribute | Value |
| --- | --- |
| Output Impedance Enabled | False |

- Disabling the IviACPwrDCGeneration Extension Group

Attribute values that effectively disable the IviACPwrDCGeneration extension group are shown in the following table.

| Attribute | Value |
| --- | --- |
| DC Mode | AC Only |

- Disabling the IviACPwrVoltageRamp Extension Group

The IviACPwrVoltageRamp extension group affects the instrument behavior only when the user calls the voltage ramp functions. Therefore, this specification does not recommend attribute values that disable the IviACPwrVoltageRamp extension group.

- Disabling the IviACPwrCurrentRamp Extension Group

The IviACPwrCurrentRamp extension group affects the instrument behavior only when the user calls the current ramp functions. Therefore, this specification does not recommend attribute values that disable the IviACPwrCurrentRamp extension group.

- Disabling the IviACPwrFrequencyRamp Extension Group

The IviACPwrFrequencyRamp extension group affects the instrument behavior only when the user calls the frequency ramp functions. Therefore, this specification does not recommend attribute values that disable the IviACPwrFrequencyRamp extension group.

  - Special Repeated Capability Identifier

All attributes that apply to an OutputPhase and all functions that have a phase name parameter in the following IviACPwr groups must support the “AllPhases” repeated capability name:

- IviACPwrBase

- IviACPwrCurrentProtection

- IviACPwrVoltageProtection

- IviACPwrImpedance

- IviACPwrDCGeneration

- IviACPwrVoltageRamp

- IviACPwrCurrentRamp

These groups may or may not support repeated capability names for a specific phase. If they don’t, they must return the IVIACPWR_ERROR_ALL_PHASES_REQUIRED completion code.

  - Current and Voltage Protection

When a current or voltage protection condition occurs (refer to the IviACPwrCurrentProtection and IviACPwrVoltageProtection extension groups), the Output Enabled attribute in IviACPwrBase is set to False. If you attempt to set Output Enabled to True prior to resetting the voltage or current protection state, the driver shall return IVIACPWR_ERROR CURRENT_PROTECTION or IVIACPWR_ERROR VOLTAGE_PROTECTION.

  - Operations that Require the Output be Disabled

Certain operations may require the power source output be reconfigured, requiring that the power source output first be disabled. This can adversely affect automated testing because it may require that the load be reinitialized. The table below shows examples of actions that may require that output power be disabled before they are performed.

| Capability Group | Action |
| --- | --- |
| IviACPwrBase | Changing the voltage or frequency range |
| IviACPwrImpedance | Enabling or disabling the output impedance or changing the resistance or inductance value. |
| IviACPwrArbWaveform | Writing a waveform |
| IviACPwrDCGeneration | Changing the DC mode, DC Voltage Range, or DC Voltage Level |

If the power source is presently supplying output power (Output Enabled attribute is True) and the user attempts to perform these or other actions that require output power be disabled, it is the responsibility of the driver to return the IVIACPWR_ERROR_OUTPUT_ENABLED completion code. The user is responsible for setting the Output Enabled attribute to False then reattempt the operation.

Operations requiring the output be disabled may also affect other attributes. For example, changing the  voltage or frequency range may also invalidate or reset other attributes as described below:

- Changing the voltage or frequency range may cause the previous Voltage Level , Frequency or Current Limit attribute values in IviACPwrBase to be outside the new range.

- Changing the voltage or frequency range may cause previous attribute values in IviACPwrImpedance to be outside the impedance capabilities of the new range.

- Changing the voltage or frequency range may cause previous attribute values in IviACPwrDCGeneration to be outside the capabilities of the new range.

It is the responsibility of a particular instrument driver to document which attributes must be checked following an operation requiring the output be disabled. It is the responsibility of the user to check and set these attributes to the desired values before setting the Output Enabled attribute to True.

  - Waveform RMS and Peak Voltage

The figures below show the power source output voltage with the Voltage Level attribute in IviACPwrBase set to 120. In order to achieve 120 volts RMS, the waveform on the left (a pure sine wave) requires a peak voltage of 170 volts and the waveform on the right requires a peak voltage of 320 volts. The peak voltage required to generate the user specified RMS voltage for a particular waveform may exceed the capabilities of the power source hardware. The driver must return the IVI_ERROR_VALUE_NOT_SUPPORTED completion code if the user attempts to set the Voltage Level attribute (or calls the Configure Voltage Level function) and the requested RMS voltage requires a peak voltage that exceeds the capabilities of the power source.

The Query Voltage Range Capabilities function in IviACPwrBase allows the user to determine the minimum and maximum RMS voltages supported by the power source for a particular voltage range and waveform. The driver must take into account the peak voltage limitations of the power source when returning the minimum and maximum RMS voltage for the Query Voltage Range Capabilities function.

- Interchangeability Checking Rules

  - Introduction

IVI drivers have a feature called interchangeability checking. Interchangeability checking returns a warning when it encounters a situation where the application program might not produce the same behavior when the user attempts to use a different instrument.

  - When to Perform Interchangeability Checking

Interchangeability checking occurs when all of the following conditions are met:

The Interchange Check attribute is set to True

The user calls one of the following functions.

    - Configure Output Enabled

    - Configure Waveform

    - Initiate Measurement

The user sets any of the attributes described in section B.3 Interchangeability Checking Rules.

  - Interchangeability Checking Rules

Interchangeability checking is performed on a capability group basis. When enabled, interchangeability checking is always performed on the base capability group. In addition, interchangeability checking is performed on extension capability groups for which the user has ever set any of the attributes of the group. If the user has never set any attributes of an extension capability group, interchangeability checking is not performed on that group.

In general interchangeability warnings are generated if the following conditions are encountered:

An attribute that affects the behavior of the instrument is not in a state that the user specifies.

The user sets a class driver defined attribute to an instrument-specific value.

The user configures the value of an attribute that the class defines as read-only. In a few cases the class drivers define read-only attributes that specific drivers might implement as read/write.

The remainder of this section defines additional rules and exceptions for each capability group.

IviACPwrBase Capability Group

The driver performs interchangeability checking on the IviACPwrBase group only when the Output Enabled attribute is set to True.

The Current Limit, Voltage Level, Voltage Range, Frequency and Frequency Range attributes must be in a user defined state prior to setting the Output Enabled attribute to True.

When setting the Waveform attribute to a value other than ‘Sine’, the waveform must first be in a user defined state by calling the Write Arbitrary Waveform function in the IviACPwrArbWaveform capability group. Using a fixed vendor defined waveform other than ‘Sine’ must always generate an interchangeability warning because vendor defined waveform names and data may vary between instruments and vendors.

IviACPwrMeasurement Capability Group

The driver performs interchangeability checking on the IviACPwrMeasurement group when the user calls the Initiate Measurement function.

The driver returns an interchangeability warning if the instrument is not capable of performing all measurements in the requested measurement groups on the same cycle of the output.

IviACPwrPhase Capability Group

No additional interchangeability rules or exceptions are defined for the IviACPwrPhase capability group.

IviACPwrExternalSync Group

The driver performs interchangeability checking on the IviACPwrExternalSync group only when the External Synchronization Enabled attribute is set to True.

The External Synchronization Phase Offset attribute must be in a user defined state prior to setting the External Synchronization Enabled attribute to True.

IviACPwrCurrentProtection Capability Group

The driver performs interchangeability checking on the IviACPwrCurrentProtection group only when the Current Protection Enabled attribute is set to True.

The Current Protection Threshold and Current Protection Delay attributes must be in a user defined state prior to setting the Current Protection Enabled attribute to True.

IviACPwrVoltageProtection Capability Group

The driver performs interchangeability checking on the IviACPwrVoltageProtection group only when the Under Voltage Enabled or Over Voltage Enabled attribute is set to True.

The Under Voltage Limit attribute must be in a user defined state prior to setting the Under Voltage Enabled attribute to True.

The Over Voltage Limit attribute must be in a user defined state prior to setting the Over Voltage Enabled attribute to True.

IviACPwrArbWaveform Capability Group

The driver performs interchangeability checking on the IviACPwrArbWaveform group only when the user sets the Waveform attribute in the IviACPwrBase group to a value other than ‘Sine’. Waveforms other than ‘Sine’ must have been previously set to a user defined state by using the Write Arbitrary Waveform function in the IviACPwrArbWaveform group.

IviACPwrImpedance Capability Group

The driver performs interchangeability checking on the IviACPwrImpedance group only when the Output Impedance Enabled attribute is set to True.

The Output Impedance Resistive and Output Impedance Inductive attribute must be in a user defined state prior to setting the Output Impedance Enabled attribute to True.

IviACPwrDCGeneration Capability Group

The driver performs interchangeability checking on the IviACPwrDCGeneration group only when the DC Mode attribute is set to DC Only or AC Plus DC.

The DC Voltage Level attribute must be in a user defined state prior to setting the DC Mode attribute to DC Only or AC Plus DC.

IviACPwrVoltageRamp Capability Group

No additional interchangeability rules or exceptions are defined for the IviACPwrVoltageRamp capability group.

IviACPwrCurrentRamp Capability Group

No additional interchangeability rules or exceptions are defined for the IviACPwrCurrentRamp capability group.

IviACPwrFrequencyRamp Capability Group

No additional interchangeability rules or exceptions are defined for the IviACPwrFrequencyRamp capability group.